DRAFT Greenhouse Gas Emissions Estimation and Reporting Guidelines for Agriculture, Fisheries and Forestry

Methodological Guidance

Chapter 2: Implementation Guidance and Data Checklists

© Commonwealth of Australia 2026

Ownership of intellectual property rights

Unless otherwise noted, copyright (and any other intellectual property rights) in this publication is owned by the Commonwealth of Australia (referred to as the Commonwealth).

Creative Commons licence

All material in this publication is licensed under a Creative Commons Attribution 4.0 International Licence except content supplied by third parties, logos and the Commonwealth Coat of Arms.

Inquiries about the licence and any use of this document should be submitted via our online contact form.

Cataloguing data

This publication (and any material sourced from it) should be attributed as: DCCEEW 2026, DRAFT Greenhouse Gas Emissions Estimation and Reporting Guidelines for Agriculture, Fisheries and Forestry - Methodological Guidance, Department of Climate Change, Energy, the Environment and Water, Canberra, March. CC BY 4.0.

This publication is available at dcceew.gov.au.

Department of Climate Change, Energy, the Environment and Water

GPO Box 3090 Canberra ACT 2601

Telephone 1800 920 528

Web dcceew.gov.au

Disclaimer

The Australian Government acting through the Department of Climate Change, Energy, the Environment and Water has exercised due care and skill in preparing and compiling the information and data in this publication. Notwithstanding, the Department of Climate Change, Energy, the Environment and Water, its employees and advisers disclaim all liability, including liability for negligence and for any loss, damage, injury, expense or cost incurred by any person as a result of accessing, using or relying on any of the information or data in this publication to the maximum extent permitted by law.

Acknowledgements

We thank Environmental Accounting Services for drafting this document, and the Voluntary Greenhouse Gas Estimation and Reporting Standards Reference Group for their valued contributions.

Acknowledgement of Country

We acknowledge the Traditional Owners of Country throughout Australia and recognise their continuing connection to land, waters and culture. We pay our respects to their Elders past and present.

Contents

2	Methodological implementation guides	5

2.1	Beef Feedlot	11

2.1.1	Module Map	11

2.1.2	Data Collection Checklist	12

2.2	Beef Pasture Rangeland and Paddock	18

2.2.1	Module Map	18

2.2.2	Data Collection Checklist	20

2.3	Cropping	27

2.3.1	Module Map	27

2.3.2	Data Collection Checklist	29

2.4	Dairy	36

2.4.1	Module Map	36

2.4.2	Data Collection Checklist	38

2.5	Sheep	47

2.5.1	Module Map	47

2.5.2	Data Collection Checklist	49

2.6	Poultry	57

2.6.1	Module Map	57

2.6.2	Input Data Checklist	58

2.7	Swine	64

2.7.1	Module Map	64

2.7.2	Input Data Checklist	65

2.8	Other Livestock	72

2.8.1	Module Map	72

2.8.2	Input Data Checklist	74

2.9	Horticulture	83

2.9.1	Module Map	83

2.9.2	Input Data Checklist	85

2.10	Viticulture	93

2.10.1	Module Map	93

2.10.2	Input Data Checklist	94

2.11	Aquaculture and Wildcatch Fisheries	102

2.11.1	Module Map	102

2.11.2	Input Data Checklist	104

2.12	Forestry	111

2.12.1	Module Map	111

2.12.2	Input Data Checklist	112

2.13	Post-production Module	117

2.13.1	Module Map	117

2.13.2	Input Data Checklist	118

References	122

Tables

Table 1: Dependant relationships between Modules	5

Table 2: Tranche 1 - Production system specific profile information	6

Table 3: Tranche 2 - Production system specific profile information	7

Table 4: Required data for beef feedlot system	12

Table 5: Optional data for beef feedlot system	15

Table 6: Required data for beef pasture range paddock system	20

Table 7: Optional data for beef pasture range paddock system	24

Table 8: Required data for cropping system	29

Table 9: Optional data for cropping system	33

Table 10: Required data for dairy system	38

Table 11: Optional data for dairy system	43

Table 12: Required data for sheep system	49

Table 13: Optional data for sheep system	53

Table 14: Required data for a poultry system	58

Table 15: Optional data for a poultry system	61

Table 16: Required data for a swine system	65

Table 17: Optional data for a swine system	69

Table 18: Required data for other livestock systems	74

Table 19: Optional data for other livestock systems	79

Table 20: Required data for horticulture system	85

Table 21: Optional data for horticulture system	90

Table 22: Required data for viticulture system	94

Table 23: Optional data for viticulture system	99

Table 24: Required data for aquaculture and/or wild fisheries system	104

Table 25: Optional data for aquaculture and/or wild fisheries system	109

Table 26: Required data for forestry system	112

Table 27: Optional data for forestry system	115

Table 18: Potentially required data for post-production	118

Table 19: Potential optional data for post-production	121

# Methodological implementation guides

This Guidance consists of a number of estimation Modules, each relevant to a particular emissions source from within a production system.

Development of an annual emissions inventory requires the collation of a number of Modules. Selection of the relevant Modules is based on the activities being undertaken within the defined reporting boundary (Case Study 1).

Sector specific implementation guides provided in this chapter are focused on production system emissions. Therefore, data collection will be to the end of the production system e.g. ‘farm-gate’, ‘point of harvest’ or ‘point of landing’. A generic post-production implementation Module has been provided in Section 2.13 for estimation of post-production emissions. For example, electricity used in post-production processing or packing should be excluded from electricity used to quantify production system emissions but can be used in the post-production Module to estimate post-production emissions.

Refer to Chapter 1, Section 1.3, for additional information on coverage and limitations of the guidance.

Some Modules can be implemented independently and others are dependent. Dependencies between modules are summarised at Table 1.

Table 1: Dependant relationships between Modules

| If you select… | Then you SHALL also select… |
| --- | --- |
| Enteric Methane (Scope 1) | Manure Management (Scope 1) |
| Manure Management (Scope 1) | Enteric Methane (Scope 1) [excluding poultry] |
| Fertiliser Use (Scope 1) | Purchased Fertiliser (Scope 3) |
| Transport and Stationary Fuel (Scope 1) | Upstream Emission from Fuel (Scope 3) |
| Purchased Electricity (Scope 2) | Purchased Electricity (Scope 3) |
| Upstream Emissions from Fuel (Scope 3) | Transport and Stationary Fuel (Scope 1) |
| Purchased Electricity (Scope 3) | Purchased Electricity (Scope 2) |
| Purchased Fertiliser (Scope 3) | Fertiliser Use (Scope 1) |

In this section, Module Maps (Tables) are provided for each production system. If a reporting boundary includes more than one production system (e.g. cropping and sheep) then each relevant Module Map should be considered, as sub-sections within Modules may differ (Case Study 1).

Data checklists are also provided to support implementation of the Modules and prioritisation of data collection efforts. These checklists are separated by Required and Optional data types. For Optional farm input data, Method 1 approaches may be used when data is limited or difficult to obtain (see Chapter 1 - General Guidance; Section 1.4.3 on prioritising data collection efforts). Higher level methods aim to result in more accurate production system specific emissions estimation compared to Method 1.

Table 2 and Table 3 list the characteristics required to enable linking to relevant Method 1 and Method 2 parameters that have an impact on the outcome of the GHG estimation. Some characteristics are required regardless of the estimation method selected, while others are dependent on the selected method.

Refer to CRF, Chapter 5 for additional information on boundary setting.

Chapter 1, Section 1.8.2 provides more detail on how to estimate climate zone, leaching and rainfall zones.

Table 2: Tranche 1 - Production system specific profile information

| Item | Beef pasture range and paddock | Beef Feedlot | Sheep | Dairy | Cropping |
| --- | --- | --- | --- | --- | --- |
| State/Territory | Required | Required | Required | Required | Required |
| Region | Method 1 only | NA | NA | NA | NA |
| Climate zone | Required | Required | Required | Required | Required |
| Temperature Zone | Method 2 | Method 2 | Method 2 | Method 2 | NA |
| Leaching zone | Required | NA | Required | Method 2 | Required |
| Rainfall zone and production type (irrigated vs non irrigated, or crop-specific) | Required | Required | Required | Required | Required |

Table 3: Tranche 2 - Production system specific profile information

| Item | Pig | Poultry | Other Livestock | Horticulture | Viticulture | Aquaculture | Forestry |
| --- | --- | --- | --- | --- | --- | --- | --- |
| State/Territory | Required | Required | Required | Required | Required | Required | Required |
| Region | NA | NA | NA | NA | NA | NA | NA |
| Climate zone | Required | Required | Required | Required | Required | NA | Required |
| Temperature zone | Method 2 | Method 2 | Method 2 | NA | NA | NA | NA |
| Leaching zone | Required | Required | Required | Required | Required | NA | Required |
| Rainfall zone and production type (irrigated vs non irrigated, or crop-specific) | Required | Required | Required | Required | Required | NA | Required |

Any requirements for additional data sub-stratification/classification will be dependent on the variable as detailed in the relevant Module Data Tables listed in Chapters 3 – 15.

Case Study 1a: Modules in a mixed farming scenario

Jarrod manages a beef feedlot in central NSW and grows grains on approximately 200 ha. To estimate his production system emissions, Jarrod would apply the following Modules and their relevant sections:

| Emission Activity/Source | Module | Relevant Section |
| --- | --- | --- |
| Direct GHG emissions (Scope 1) |  |  |
| Enteric Methane | 3 | 3.1 |
| Manure Management | 4 | 4.1 |
| Fertiliser Use | 5 | 5.1 – 5.5 |
| Agriculture Residue Management | 6 | 6.2 – 6.3 |
| Transport Fuel | 8 | 8.1 |
| Stationary Fuel | 8 | 8.2 |
| Solid Waste Treatment | 10 | 10.1 |
| Indirect GHG emissions from imported energy (Scope 2) |  |  |
| Purchased Electricity | 14 | 14.1 |
| Indirect GHG emissions (Scope 3) |  |  |
| Purchased Livestock | 15 | 15.1 |
| Purchased Feed | 15 | 15.2 |
| Purchased Mineral supplements | 15 | 15.3 |
| Purchased Fertiliser | 15 | 15.4 |
| Purchased Agrichemicals | 15 | 15.5 |
| Purchased Services/Contractors | 15 | 15.7 |
| Other Purchased Goods and Services | 15 | 15.10 |
| Upstream Emission from Fuel | 15 | 15.10 |
| Upstream Emissions of Purchased Electricity | 15 | 15.11 |
| Management of Waste | 15 | 15.12 |

Tranche 2 – Case Study 1: Modules in a mixed production and post-production system

Garrett manages a viticulture business with both a vineyard and winery located on site. To estimate his production system emissions, Garrett would apply the following Modules and their relevant sections to the vineyard production system:

| Emission Activity/Source | Module | Relevant Section |
| --- | --- | --- |
| Direct GHG emissions (Scope 1) |  |  |
| Fertiliser Use | 5 | 5.1 – 5.5 |
| Agriculture Residue Management | 6 | 6.1 and 6.3 |
| Transport Fuel | 8 | 8.1 |
| Stationary Fuel | 8 | 8.2 |
| Solid Waste Treatment | 10 | 10.1 |
| Indirect GHG emissions from imported energy (Scope 2) |  |  |
| Purchased Electricity | 14 | 14.1 |
| Indirect GHG emissions (Scope 3) |  |  |
| Purchased Agrichemicals | 15 | 15.5 |
| Purchased Services/Contractors | 15 | 15.7 |
| Upstream Emission from Fuel | 15 | 15.11 |
| Upstream Emissions of Purchased Electricity | 15 | 15.12 |
| Management of Waste | 15 | 15.13 |

Refer to Section 2.10 for additional guidance on viticulture emission estimation.

To estimate his post-production system emissions, Garrett would apply the following Modules and their relevant sections to the winery production system:

| Emission Activity/Source | Module | Relevant Section |
| --- | --- | --- |
| Direct GHG emissions (Scope 1) |  |  |
| Transport Fuel | 8 | 8.1 |
| Stationary Fuel | 8 | 8.2 |
| Refrigerant Use | 9 | 9.1 |
| Solid Waste Treatment | 10 | 10.1 |
| Wastewater Treatment | 11 | 11.1 |
| Purchased CO2 | 12 | 12.1 |
| Indirect GHG emissions from imported energy (Scope 2) |  |  |
| Purchased Electricity | 14 | 14.1 |
| Indirect GHG emissions (Scope 3) |  |  |
| Purchased Services/Contractors | 15 | 15.7 |
| Purchased Packaging | 15 | 15.8 |
| Other Purchased Goods and Services | 15 | 15.10 |
| Upstream Emission from Fuel | 15 | 15.11 |
| Upstream Emissions of Purchased Electricity | 15 | 15.12 |
| Management of Waste | 15 | 15.13 |
| Upstream Freight | 15 | 15.14 |

Refer to Section 2.13 for additional guidance on post-production emission estimation.

## Beef Feedlot

This section outlines all relevant Modules and data checklists to develop a GHG inventory for a beef feedlot system. Relevant emission sources and therefore relevant Modules for a specific entity will depend on an entity-specific boundary set as described in Chapter 5 of the CRF. For example, if a specific entity does not use lime, then this emission source and Module will not be relevant for this specific entity.

Module map and input data checklists are designed for estimating emissions from cradle-to-end of the farming system/farm-gate. If the boundary of the organisation extends to processing, packing and transport of beef products, the post-production Module can also be implemented to estimate emissions from these sources. Refer to Chapter 1, Section 1.3, for additional information on coverage and limitations of the guidance.

### Module Map

| Emission Activity/Source | Relevant Module |
| --- | --- |
| Direct GHG emissions (Scope 1) |  |
| Enteric Methane | 3.1 |
| Manure Management | 4.1 |
| Transport Fuel | 8.1 |
| Stationary Fuel | 8.2 |
| Solid Waste Treatment | 10.1 |
| Indirect GHG emissions from imported energy (Scope 2) |  |
| Purchased Electricity | 14.1 |
| Indirect GHG emissions (Scope 3) |  |
| Purchased Livestock | 15.1 |
| Purchased Feed | 15.2 |
| Purchased Mineral supplements | 15.3 |
| Purchased Services/Contractors | 15.7 |
| Other Purchased Goods and Services | 15.10 |
| Upstream Emissions from Fuel | 15.11 |
| Upstream Emissions of Purchased Electricity | 15.12 |
| Management of Waste | 15.13 |

### Data Collection Checklist

The checklist below summarises all the required input data, required for Method 1, that should be collected at farm-level for a beef feedlot production system to complete the equations detailed in subsequent chapters based on the beef feedlot module map. Data collected may be listed as ‘not applicable’ for emissions sources that are not relevant to the specific entity. Further information regarding relevant sources and sinks can be found in Chapter 5.2 of the CRF.

Additional information and guidance on specific parameters can be found within the beef feedlot guidance Chapter 3 – Section; 3.1 and Chapter 4 – Section; 4.1. With additional related parameters found within Chapter 8, Chapter 14, Chapter 15; Sections 15.1 – 15.3, 15.7 and 15.10 – 15.13.

Table 4: Required data for beef feedlot system

| Parameter | Data to be collected | Data collected Yes/No/N/A | Unit | Source of data | Module |
| --- | --- | --- | --- | --- | --- |
| Required – Scope 1 |  |  |  |  |  |
| Nj | Numbers of beef cattle in each group |  | head | Farm stock records e.g. head counts, diary entries, purchase, and sales records | Enteric Methane & Manure Management |
| Dj | Duration of stay for each cattle group |  | days | Farm stock records; system type records, purchase and sales records and National Vendor Declarations or NLID transfer records may be evaluated to determine average length of stay. | Enteric Methane & Manure Management |
| MMSmT=2 | Fraction of manure at treatment stage 2 for each manure management system |  | fraction | Farmer records as to what fraction of manure flows to each system based on the available MMS options | Manure Management |
| PF | Fraction of manure applied to soil within the farm boundary. |  | fraction | Farmer records of manure sales or transport weights compared to total manure treated on farm. | Manure Management |
| QTrans,q | Quantity of each fuel used for transportation (including off-road vehicles and equipment, aircraft, and/or vessels), up to the farm-gate. |  | Depends on fuel type | Invoices, fuel cards, receipts | Transport Fuel & Upstream emissions |
| QSC,q | Quantity of each fuel used for stationary combustion, up to the farm-gate. |  | Depends on fuel type | Invoices, fuel cards, receipts | Stationary Fuel & Upstream Emissions |
| Qwt | Quantity of waste per type being treated within the farm-gate. |  | tonnes | Farm records of weight of waste disposed | Solid Waste Treatment |
| Vwt | Volume of waste per type being treated within the farm-gate. |  | m3 | Only required if Qwt is not known. Estimated from trailer-loads, or other known volume of waste transported. | Solid Waste Treatment |
| Required – Scope 2 |  |  |  |  |  |
| Qelec | Quantity of electricity purchased from the grid, up to the farm-gate. |  | kWh | Invoices from electricity supplier | Purchased Electricity & Upstream Emissions from Purchased Electricity |
| Required – Scope 3 |  |  |  |  |  |
| Nco | Number of animals purchased per livestock class, including state of origin of purchase |  | head | Farm stock records, invoices | Purchased Livestock |
| Qj | Quantity of each feed type purchased |  | t | Invoices, receipts or other records of purchased quantities | Purchased Feed |
| Qm | Quantity of each mineral supplement purchased |  | t | Invoices, receipts or other records of purchased quantities | Purchased Mineral Supplements |
| Fcf | Fraction of inorganic fertiliser components in each fertiliser |  | wt% | Product information label | Purchased Fertiliser |
| Ap | Activity data for each service purchased, up to the farm-gate. |  | Depends on purchased service | Invoices, receipts or other records of quantity of service provided | Purchased Services and Contractors |
| Qwt | Weight of waste sent to waste treatment facility |  | tonnes | Invoices for skip disposal, weighbridge receipts, farm records | Management of Waste |
| Vwt | Volume of waste being sent waste treatment facility (only required if weight is unknown) |  | m3 | Invoices for skip disposal, or other farm records | Management of Waste |

The checklist below summarises all the optional data points, required for Method 2, that may be collected at farm-level for a beef feedlot system to complete the equations detailed in subsequent chapters based on the beef feedlot module map.

Table 5: Optional data for beef feedlot system

| Parameter | Data to be collected | Data collected Yes/No/N/A | Unit | Data source | Module |
| --- | --- | --- | --- | --- | --- |
| Optional – Scope 1 |  |  |  |  |  |
| NDFj | Average neutral detergent fibre as a percentage intake for each feedlot cattle group. |  | per-centage of dry matter intake | Farm records of neutral detergent fibre of TMR for each cattle group | Enteric Methane |
| EEj | Average ether extract as a percentage of intake for each feedlot cattle group |  | per-centage of dry matter intake | Farm records of ether extract of TMR for each cattle group | Enteric Methane |
| Ij | Average dry matter intake per head for each group of beef cattle within feedlot system per day |  | kgDM/head/day | Farm records relating to dry matter assessment of total mixed ration (TMR), and daily feed and feed waste weights for each group | Enteric Methane & Manure Management |
| DMDi | Dry matter digestibility of diet used within feedlot system |  | fraction | Farm records or feed receipt stating dry matter digestibility of TMR | Manure Management |
| CPj | Average crude protein content of feed given to each cattle group |  | fraction | Farm records or feed receipt stating crude protein of TMR | Manure Management |
| MCFim | Methane conversion factor for temperature zone and system |  | fraction | Select relevant MCF for MMS and temperature zone | Manure Management |
| NRj | Nitrogen retention, as a per cent of intake |  | per cent | Farm specific estimates (estimated by mass balance) | Manure Management |
| Optional – Scope 2 |  |  |  |  |  |
| RECsurrendered | Eligible Renewable Energy Certificates voluntarily surrendered in the reporting year |  | MWh | Records of transactions | Purchased Electricity & Upstream Emissions from Purchased Electricity (market-based approach only) |
| REConsite | Eligible Renewable Energy Certificates that have been or will be issued for electricity produced on-site during the reporting period and consumed by the entity |  | MWh | Records of transactions | Purchased Electricity & Upstream Emissions from Purchased Electricity (market-based approach only) |
| Optional – Scope 3 |  |  |  |  |  |
| Wco | Liveweight of purchased livestock at point of purchase |  | kg/head | Purchasing records/invoices and herd flow models | Purchased Livestock |
| EF | Supplier-provided life cycle emission factor for any of the following purchased goods: livestock, feed, mineral supplements |  | t CO2e/t purchased good | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 14067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard | Corresponding Scope 3 Purchased Good Module |

## Beef Pasture Rangeland and Paddock

This section outlines all relevant Modules and data checklists to develop a GHG inventory of for a beef pasture, range, and paddock system. Relevant emission sources and therefore relevant Modules for a specific entity will depend on an entity-specific boundary set as described in Chapter 5 of the CRF. For example, if a specific entity does not use lime, then this emission source and Module will not be relevant for this specific entity.

Module map and input data checklists are designed for estimation of emissions from cradle-to-end of the farming system/farm-gate. If the boundary of the organisation extends to processing, packing and transport of beef products, the post-production Module can also be implemented to estimate emissions from these sources. Refer to Chapter 1, Section 1.3, for additional information on coverage and limitations of the guidance.

### Module Map

| Emission Activity/Source | Relevant Module |
| --- | --- |
| Direct GHG emissions (Scope 1) |  |
| Enteric Methane | 3.2 |
| Manure Management | 4.2 |
| Fertiliser Use | 5.1 – 5.5 |
| Agriculture Residue Management | 6.2 – 6.3 |
| Transport Fuel | 8.1 |
| Stationary Fuel | 8.2 |
| Solid Waste Treatment | 10.1 |
| Indirect GHG emissions from imported energy (Scope 2) |  |
| Purchased Electricity | 14.1 |
| Indirect GHG emissions (Scope 3) |  |
| Purchased Livestock | 15.1 |
| Purchased Feed | 15.2 |
| Purchased Mineral Supplements | 15.3 |
| Purchased Fertiliser | 15.4 |
| Purchased Agrichemicals | 15.5 |
| Purchased Lime | 15.6 |
| Purchased Services/Contractors | 15.7 |
| Other Purchased Goods and Services | 15.10 |
| Upstream Emissions from Fuel | 15.11 |
| Upstream Emissions of Purchased Electricity | 15.12 |
| Management of Waste | 15.13 |

### Data Collection Checklist

The checklist below summarises all the required input data, required for Method 1, that should be collected at farm-level for a beef pasture range paddock system to complete the equations detailed in subsequent chapters based on the beef pasture range paddock module map. Data collected may be listed as ‘not applicable’ for emissions sources that are not relevant to the specific entity. Further information regarding relevant sources and sink can be found in Chapter 5.2 of the CRF.

Additional information and guidance on specific parameters can be found within the beef pasture range paddock guidance Chapter 3 – Section; 3.2 and Chapter 4 – Section; 4.2. With additional related parameters found within Chapter 5 – Sections; 5.1 - 5.5, Chapter 6 – Sections; 6.2 and 6.3 and Chapter 8, Chapter 14, Chapter 15; Sections 15.1 – 15.7 and 15.10 – 15.13.

Table 6: Required data for beef pasture range paddock system

| Parameter | Data to be collected | Data collected Yes/No/N/A | Unit | Source of data | Module |
| --- | --- | --- | --- | --- | --- |
| Required – Scope 1 |  |  |  |  |  |
| Njkl | Number of beef cattle per time-period, and input class |  | head | Stock records – see herd flow modelling in Chapter 1 Section 1.3.7 | Enteric Methane & Manure Management |
| LCjk=5 | Ratio of the number of births to the number of cows over 2 years old |  | fraction | Farm records: Proportion of cows > 2 in calf may be based on scanning numbers where available. If scanning results are not available weaning numbers may also be used to approximate the proportion of cows > 2 years lactating. For farms with multiple calving seasons, the proportion of cows > 2 years in calf LCjk=5 should be reported separately for each season. Noting that for all other seasons LCjk=5 = 0 | Enteric Methane & Manure Management |
| TMjf | Amount of each inorganic fertiliser applied to each production system |  | kg | Receipts, invoices, farm records | Fertiliser Use – Inorganic Fertiliser Use & Purchased Fertiliser |
| FNinorganic,f | Fraction of nitrogen in each inorganic fertiliser |  | kg N/kg | Default values for common inorganic fertilisers based on average fertiliser composition from [1], or calculated from product label information. | Fertiliser Use – Inorganic fertiliser Use |
| FUf | Fraction of urea in each inorganic fertiliser applied |  | kg urea/kg | Default values for common inorganic fertilisers based on average fertiliser composition from [1], and or calculated from product label information. | Fertiliser Use – Inorganic fertiliser Use |
| Morganic,jf | Amount of each organic fertiliser applied to soils in production system |  | kg | Invoices of purchases, farm records of application rates | Fertiliser Use – Organic Fertiliser Use |
| FNorganic,f | Nitrogen fraction of each organic fertiliser applied to soils |  | kg N/kg | Supplier-provided values or product label information | Fertiliser Use – Organic Fertiliser Use |
| Mlime | Total amount of lime applied to soils |  | kg | Invoices, receipts | Fertiliser Use – Lime Application |
| FracLime | Fraction of lime as limestone |  | dimensionless | Product information label | Fertiliser Use – Lime Application |
| FracDol | Fraction of lime as dolomite |  | dimensionless | Product information label | Fertiliser Use – Lime Application |
| Ap | Area of each pasture crop renewed or removed |  | ha | Geospatial files, farm records | Agriculture Residue Management - Pasture Residues |
| QTrans,q | Quantity of each fuel used for transportation (including off-road vehicles and equipment, aircraft, and/or vessels), up to the farm-gate. |  | Depends on fuel type | Invoices, fuel cards, receipts | Transport Fuel & Upstream Emissions |
| QSC,q | Quantity of each fuel used for stationary combustion, up to the farm-gate. |  | Depends on fuel type | Invoices, fuel cards, receipts | Stationary Fuel & Upstream Emissions |
| Qwt | Quantity of waste per type being treated within the farm-gate. |  | tonnes | Farm records of weight of waste disposed | Solid Waste Treatment |
| Vwt | Volume of waste per type being treated within the farm-gate. |  | m3 | Only required if Qwt is not known. Estimated from trailer-loads, or other known volume of waste transported. | Solid Waste Treatment |
| Required – Scope 2 |  |  |  |  |  |
| Qelec | Quantity of electricity purchased from the grid, up to the farm-gate. |  | kWh | Invoices from electricity supplier | Purchased Electricity & Upstream Emissions from Purchased Electricity |
| Required – Scope 3 |  |  |  |  |  |
| Nco | Number of animals purchased per livestock class, including origin of purchase |  | head | Farm stock records, invoices | Purchased Livestock |
| Qj | Quantity of each feed type purchased |  | t | Invoices, receipts or other records of purchased quantities | Purchased Feed |
| Qm | Quantity of each mineral supplement purchased |  | t | Invoices, receipts or other records of purchased quantities | Purchased Mineral Supplements |
| Fcf | Fraction of inorganic fertiliser components in each fertiliser |  | wt% | Product information label | Purchased Fertiliser |
| Qh | Quantity of each agrichemical used |  | kg | Invoices, receipts or other records of purchased quantities | Purchased Herbicides/Pesticides |
| Ap | Activity data for each service purchased, up to the farm-gate. |  | Depends on purchased service | Invoices, receipts or other records of quantity of service provided | Purchased services and contractors |
| Qwt | Weight of waste sent to waste treatment facility |  | tonnes | Invoices for skip disposal, weighbridge receipts, farm records | Management of Waste |
| Vwt | Volume of waste being sent waste treatment facility (only required if weight is unknown) |  | m3 | Invoices for skip disposal, or other farm records | Management of Waste |

The checklist below summarises all the optional data points, required for Method 2, that may be collected at farm-level for a beef pasture range paddock system to complete the equations detailed in subsequent chapters based on the beef pasture range paddock module map.

Table 7: Optional data for beef pasture range paddock system

| Parameter | Data to be collected | Data collected Yes/No/N/A | Unit | Data source | Module |
| --- | --- | --- | --- | --- | --- |
| Optional – Scope 1 |  |  |  |  |  |
| Dj | Number of days in each time period animals in each class are on the farm |  | Days | Farm stock records and herd flow model | Enteric Methane & Manure Management |
| Wjkl | Average liveweight of beef pasture cattle per time-period, and input class |  | kg | Farm stock records and herd flow model | Enteric Methane & Manure Management |
| LWGjkl | Average liveweight gain of beef pasture cattle per time-period, and input class |  | kg/head/day | Farm stock records and herd flow model | Enteric Fermentation & Manure Management |
| DMDj | Dry matter digestibility of feed |  | fraction | Farm source records for seasonal dry matter digestibility of feed. | Manure Management |
| CPj | Crude protein content of feed |  | fraction | Records for seasonal crude protein of feed. | Manure Management |
| MCFim | Methane conversion factor for temperature zone and system |  | fraction | Select relevant MCF for MMS and temperature zone | Manure Management |
| Nfj | N application rate of inorganic fertiliser used in production system |  | kg N/ha | Farm records of fertiliser application rates, or calculated from: Nfj=Total fertiliser applied (kgN)Total area (ha) Data on total fertiliser applied and total area should be sourced from farm invoices and spatial records respectively. | Fertiliser Use – Inorganic Fertiliser Use |
| EFj,N2O | Emission factor for direct nitrous oxide emissions from inorganic fertiliser |  | kg N2O-N/kg N | Calculated from relevant equations in Chapter 5, sections 5.1.1.2 | Fertiliser Use – Inorganic Fertiliser Use |
| Yp | Average yield of pasture crop |  | tDM/ha | Yield records | Agriculture Residue Management - Pasture Residues |
| Optional – Scope 2 |  |  |  |  |  |
| RECsurrendered | Eligible Renewable Energy Certificates voluntarily surrendered in the reporting year |  | MWh | Records of transactions | Purchased Electricity & Upstream Emissions from Purchased Electricity (market-based approach only) |
| REConsite | Eligible Renewable Energy Certificates that have been or will be issued for electricity produced on-site during the reporting period and consumed by the entity |  | MWh | Records of transactions | Purchased Electricity & Upstream Emissions from Purchased Electricity (market-based approach only) |
| Optional – Scope 3 |  |  |  |  |  |
| Wco | Liveweight of purchased livestock at point of purchase |  | kg/head | Purchasing records/invoices and herd flow models | Purchased Livestock |
| EF | Supplier-provided life cycle emission factor for any of the following purchased goods: livestock, feed, mineral supplements, fertiliser, agrichemicals, or lime |  | t CO2e/t purchased good | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 14067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard | Corresponding Scope 3 Purchased Good Module |

## Cropping

This section outlines all relevant Modules and data checklists to develop a GHG inventory for the following cropping systems: grains, legumes, cotton, sugarcane, rice and forage. Relevant emission sources and therefore relevant Modules for a specific entity will depend on an entity-specific boundary set as described in Chapter 5 of the CRF. For example, if a specific entity does not use lime, then this emission source and Module will not be relevant for this specific entity.

Module map and input data checklists are designed for estimation of emissions from cradle-to-end of the farming system/farm-gate. If the boundary of the organisation extends to processing, packing and transport of crop products or byproducts, the post-production Module can also be implemented to estimate emissions from these sources. Refer to Chapter 1, Section 1.3, for additional information on coverage and limitations of the guidance.

### Module Map

| Emission Activity/Source | Relevant Module |
| --- | --- |
| Direct GHG emissions (Scope 1) |  |
| Fertiliser Use | 5.1 – 5.5 |
| Agriculture Residue Management | 6.1 – 6.4 |
| Rice Cultivation (only applicable to rice-growers) | 7.1 |
| Transport Fuel | 8.1 |
| Stationary Fuel | 8.2 |
| Refrigerant Use | 9.1 |
| Solid Waste Treatment | 10.1 |
| Indirect GHG emissions from imported energy (Scope 2) |  |
| Purchased Electricity | 14.1 |
| Indirect GHG emissions (Scope 3) |  |
| Purchased Fertiliser | 15.4 |
| Purchased Agrichemicals | 15.5 |
| Purchased Lime | 15.6 |
| Purchased Services/Contractors | 15.7 |
| Other Purchased Goods and Services | 15.10 |
| Upstream Emissions from Fuel | 15.11 |
| Upstream emissions of Purchased Electricity | 15.12 |
| Management of Waste | 15.13 |

### Data Collection Checklist

The checklist below summarises all the required input data, required for Method 1, that should be collected at farm-level for a cropping system to complete the equations detailed in subsequent chapters based on the cropping module map. Data collected may be listed as ‘not applicable’ for emissions sources that are not relevant to the specific entity. Further information regarding relevant sources and sink can be found in Chapter 5.2 of the CRF.

Additional information and guidance on cropping specific parameters can be found within Chapter 5 – Section; 5.1 – 5.5, Chapter 6 – Sections; 6.1 – 6.4, and Chapter 7 – Sections; 7.1, Chapter 8 – Sections; 8.1 and 8.2, Chapter 9 – Section; 9.1, Chapter 14 – Section; 14.1 and Chapter 15 – Sections; 15.4 – 15.7 and 15.10 – 15.13.

Table 8: Required data for cropping system

| Parameter | Data to be collected | Data collected Yes/No/N/A | Unit | Data source | Module |
| --- | --- | --- | --- | --- | --- |
| Required – Scope 1 |  |  |  |  |  |
| TMjf | Amount of each inorganic fertiliser applied to each production system |  | kg | Receipts, invoices, farm records | Fertiliser Use – Inorganic Fertiliser Use & Purchased Fertiliser |
| FNinorganic,f | Fraction of nitrogen in each inorganic fertiliser |  | kg N/kg | Default values for common inorganic fertilisers based on average fertiliser composition from [1], or calculated from product label information. | Fertiliser Use – Inorganic fertiliser Use |
| FUf | Fraction of urea in each inorganic fertiliser applied |  | kg urea/kg | Default values for common inorganic fertilisers based on average fertiliser composition from [1], and or calculated from product label information. | Fertiliser Use – Inorganic fertiliser Use |
| Morganic,jf | Amount of each organic fertiliser applied to soils in production system |  | kg | Invoices of purchases, farm records of application rates | Fertiliser Use – Organic Fertiliser Use |
| FNorganic,f | Nitrogen fraction of each organic fertiliser applied to soils |  | kg N/kg | Supplier-provided values or product label information | Fertiliser Use – Organic Fertiliser Use |
| Mlime | Total amount of lime applied to soils |  | kg | Invoices, receipts | Fertiliser Use – Lime Application |
| FracLime | Fraction of lime as limestone |  | dimensionless | Product information label | Fertiliser Use – Lime Application |
| FracDol | Fraction of lime as dolomite |  | dimensionless | Product information label | Fertiliser Use – Lime Application |
| Pc | Annual production of each crop |  | kg | Farm yield records from the reporting year. Where crop failure has occurred, may be estimated based on the expected harvest of that year or historical yield data. | Agricultural Residue Management – Crop Residues & Field Burning |
| Fc | Fraction of crop residue that is burnt |  | dimensionless | Calculated from records of area burnt and area cultivated | Agricultural Residue Management – Crop Residues & Field Burning |
| Ap | Area of each pasture crop renewed or removed |  | ha | Farm records | Agriculture Residue Management - Pasture Residues |
| Arice,wpo | Annual area of rice cultivation under each water regime during the growing season, pre-season water regime, and organic amendment condition |  | ha | Spatial analysis, farm records | Rice Cultivation |
| trice,wpo | Number of days in rice growing under each water regime during the growing season, pre-season water regime, and organic amendment condition |  | days | Monitoring records | Rice Cultivation |
| ROAo | Rate of application of each organic amendment |  | Straw: t (dry weight)/ha Other: t (fresh weight)/ha | Farm records of purchased organic amendments | Rice Cultivation |
| QTrans,q | Quantity of each fuel used for transportation (including off-road vehicles and equipment, aircraft, and/or vessels), up to the farm-gate. |  | Depends on fuel type | Invoices, fuel cards, receipts | Transport Fuel & Upstream Emissions |
| QSC,q | Quantity of each fuel used for stationary combustion, up to the farm-gate. |  | Depends on fuel type | Invoices, fuel cards, receipts | Stationary Fuel & Upstream Emissions |
| chargeRa | Amount of each refrigerant gas contained in each appliance |  | kg | Manufacturer’s product specification | Refrigerant Use |
| Qwt | Quantity of waste per type being treated within the farm-gate. |  | tonnes | Farm records of weight of waste disposed | Solid Waste Treatment |
| Vwt | Volume of waste per type being treated within the farm-gate. |  | m3 | Only required if Qwt is not known. Estimated from trailer-loads, or other known volume of waste transported. | Solid Waste Treatment |
| Required – Scope 2 |  |  |  |  |  |
| Qelec | Quantity of electricity purchased from the grid, up to the farm-gate. |  | kWh | Invoices from electricity supplier | Purchased Electricity & Upstream Emissions from Purchased Electricity |
| Required – Scope 3 |  |  |  |  |  |
| Fcf | Fraction of inorganic fertiliser components in each fertiliser |  | wt% | Product information label | Purchased Fertiliser |
| Qh | Quantity of each agrichemical used |  | kg | Invoices, receipts or other records of purchased quantities | Purchased Herbicides/Pesticides |
| Ap | Activity data for service purchased, up to the farm-gate. |  | Depends on purchased service | Invoices, receipts or other records of quantity of service provided | Purchased services and Contractors |
| Qwt | Weight of waste sent to waste treatment facility |  | tonnes | Invoices for skip disposal, weighbridge receipts, farm records | Management of Waste |
| Vwt | Volume of waste being sent waste treatment facility (only required if weight is unknown) |  | m3 | Invoices for skip disposal, or other farm records | Management of Waste |

The checklist below summarises all the optional data points, required for Method 2, that may be collected at farm-level for a cropping system to complete the equations detailed in subsequent chapters based on the cropping module map.

Table 9: Optional data for cropping system

| Parameter | Data to be collected | Data collected Yes/No/N/A | Unit | Data source | Module |
| --- | --- | --- | --- | --- | --- |
| Optional – Scope 1 |  |  |  |  |  |
| Nfj | N application rate of inorganic fertiliser used in production system (only relevant for production systems where a Method 2 option exists) |  | kg N/ha | Farm records of fertiliser application rates, or calculated from: Nfj=Total fertiliser applied (kgN)Total area (ha) Data on total fertiliser applied and total area should be sourced from farm invoices and spatial records respectively. | Fertiliser Use – Inorganic Fertiliser Use |
| EFj,N2O | Emission factor for direct nitrous oxide emissions from inorganic fertiliser |  | kg N2O-N/kg N | Calculated from relevant equations in Chapter 5, sections 5.1.1.2 | Fertiliser Use – Inorganic Fertiliser Use |
| FFODc | Fraction of the crop residue that is removed per crop type |  | dimensionless | Calculation from equation in Chapter 6, equation 6.1.1.2 | Agricultural Residue Management – Crop Residues |
| B | Number of bales generated |  | dimensionless | Invoices for the number of bales sold off-farm, or other farm records documenting the number of bales made | Agricultural Residue Management – Crop Residues |
| MB | Average mass of each bale generated |  | kg | Invoices from bale sales | Agricultural Residue Management – Crop Residues |
| Yp | Average yield of pasture crop |  | t DM/ha | Yield records | Agriculture Residue Management - Pasture Residues |
| Optional – Scope 2 |  |  |  |  |  |
| RECsurrendered | Eligible Renewable Energy Certificates voluntarily surrendered in the reporting year |  | MWh | Records of transactions | Purchased Electricity & Upstream Emissions from Purchased Electricity (market-based approach only) |
| REConsite | Eligible Renewable Energy Certificates that have been or will be issued for electricity produced on-site during the reporting period and consumed by the entity |  | MWh | Records of transactions | Purchased Electricity & Upstream Emissions from Purchased Electricity (market-based approach only) |
| Optional – Scope 3 |  |  |  |  |  |
| EF | Supplier-provided life cycle emission factor for any of the following purchased goods: fertiliser, agrichemicals, or lime |  | t CO2e/t purchased good | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 14067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard | Corresponding Scope 3 Purchased Good Module |

## Dairy

This section outlines all relevant Modules and data checklists to develop a GHG inventory for dairy farming system. Relevant emission sources and therefore relevant Modules for a specific entity will depend on an entity-specific boundary set as described in Chapter 5 of the CRF. For example, if a specific entity does not use lime, then this emission source and Module will not be relevant for this specific entity.

Module map and input data checklists are designed for estimation of emissions from cradle-to-end of the farming system/farm-gate. If the boundary of the organisation extends to processing, packing and transport of dairy products, the post-production Module can also be implemented to estimate emissions from these sources. Refer to Chapter 1, Section 1.3, for additional information on coverage and limitations of the guidance.

### Module Map

| Emission Activity/Source | Relevant Module |
| --- | --- |
| Direct GHG emissions (Scope 1) |  |
| Enteric Methane | 3.3 |
| Manure Management | 4.3 |
| Fertiliser Use | 5.1 – 5.5 |
| Agricultural residue management | 6.2 – 6.3 |
| Transport Fuel | 8.1 |
| Stationary Fuel | 8.2 |
| Refrigerant Use | 9.1 |
| Solid Waste Treatment | 10.1 |
| Indirect GHG emissions from imported energy (Scope 2) |  |
| Purchased Electricity | 14.1 |
| Indirect GHG emissions (Scope 3) |  |
| Purchased Livestock | 15.1 |
| Purchased Feed | 15.2 |
| Purchased Mineral Supplements | 15.3 |
| Purchased Fertiliser | 15.4 |
| Purchased Agrichemicals | 15.5 |
| Purchased Lime | 15.6 |
| Purchased Services/Contractors | 15.7 |
| Other Purchased Goods and Services | 15.10 |
| Upstream Emissions from Fuel | 15.11 |
| Upstream Emissions of Purchased Electricity | 15.12 |
| Management of Waste | 15.13 |

### Data Collection Checklist

The checklist below summarises all the required input data, required for Method 1, that should be collected at farm-level for a dairy system to complete the equations detailed in subsequent chapters based on the dairy module map. Data collected may be listed as ‘not applicable’ for emissions sources that are not relevant to the specific entity. Further information regarding relevant sources and sink can be found in Chapter 5.2 of the CRF.

Additional information and guidance on specific parameters can be found within the dairy guidance in Chapter 3 – Section; 3.3 and Chapter 4 – Section; 4.3. With additional related parameters found within Chapter 5 – Sections; 5.1 - 5.5, Chapter 6 – Sections; 6.2 and 6.3 and Chapter 8 – Sections; 8.1 and 8.2, Chapter 9 – Section; 9.1, Chapter 14 – Section; 14.1 and Chapter 15 – Sections; 15.1 – 15.7 and 15.10 – 15.13.

Table 10: Required data for dairy system

| Parameter | Data to be collected | Data collected Yes/No/N/A | Unit | Data source | Module |
| --- | --- | --- | --- | --- | --- |
| Required – Scope 1 |  |  |  |  |  |
| Nj | Number of dairy cattle in each class |  | head | Farm stock records and herd flow model see Chapter 1 Section 1.4.6 | Enteric Methane & Manure Management |
| MPj | Daily milk production per milking cow |  | L/head/day | Average farm milk sales records and average number of milking cows overt the reporting period may be used to calculate the daily milk production. The average amount of milk sold per day (in litres) may be divided by the average number of milking cows on the farm to calculated the daily milk production per milking cow. | Enteric Methane & Manure Management |
| MSj | Daily milk solid production per milking cow (Only required where MPj is not known) |  | kgMS/head/ day | Farm milk sales records and number of milking cows. The average amount of milk sold per day (in kg of milk solids) may be divided by the average number of milking cows on the farm to calculate the daily milk production per milking cow. | Enteric Methane |
| PF | Fraction of manure from MMS applied to soil within the farm boundary |  | fraction | Farmer records of manure sales or transport weights | Manure Management |
| TMjf | Amount of each inorganic fertiliser applied to each production system |  | kg | Receipts, invoices, farm records | Fertiliser Use – Inorganic Fertiliser Use & Purchased Fertiliser |
| FNinorganic,f | Fraction of nitrogen in each inorganic fertiliser |  | kg N/kg | Default values for common inorganic fertilisers based on average fertiliser composition from [1], or calculated from product label information. | Fertiliser Use – Inorganic fertiliser Use |
| FUf | Fraction of urea in each inorganic fertiliser applied |  | kg urea/kg | Default values for common inorganic fertilisers based on average fertiliser composition from [1], and or calculated from product label information. | Fertiliser Use – Inorganic fertiliser Use |
| Morganic,jf | Amount of each organic fertiliser applied to soils in production system |  | kg | Invoices of purchases, farm records of application rates | Fertiliser Use – Organic Fertiliser Use |
| FNorganic,f | Nitrogen fraction of each organic fertiliser applied to soils |  | kg N/kg | Supplier-provided values or product label information | Fertiliser Use – Organic Fertiliser Use |
| Mlime | Total amount of lime applied to soils |  | kg | Invoices, receipts | Fertiliser Use – Lime Application |
| FracLime | Fraction of lime as limestone |  | dimensionless | Product information label | Fertiliser Use – Lime Application |
| FracDol | Fraction of lime as dolomite |  | dimensionless | Product information label | Fertiliser Use – Lime Application |
| Ap | Area of pasture crop renewed or removed |  | ha | Geospatial files, farm records | Agriculture Residue Management - Pasture Residues |
| QTrans,q | Quantity of each fuel used for transportation (including off-road vehicles and equipment, aircraft, and/or vessels), up to the farm-gate. |  | Depends on fuel type | Invoices, fuel cards, receipts | Transport Fuel & Upstream Emissions |
| QSC,q | Quantity of each fuel used for stationary combustion, up to the farm-gate. |  | Depends on fuel type | Invoices, fuel cards, receipts | Stationary Fuel & Upstream emissions |
| chargeRa | Amount of each refrigerant gas contained in each appliance |  | kg | Manufacturer’s product specification | Refrigerant Use |
| Qwt | Quantity of waste per type being treated within the farm-gate. |  | tonnes | Farm records of weight of waste disposed | Solid Waste Treatment |
| Vwt | Volume of waste per type being treated within the farm-gate. |  | m3 | Only required if Qwt is not known. Estimated from trailer-loads, or other known volume of waste transported. | Solid Waste Treatment |
| Required – Scope 2 |  |  |  |  |  |
| Qelec | Quantity of electricity purchased from the grid, up to the farm-gate. |  | kWh | Invoices from electricity supplier | Purchased Electricity & Upstream Emissions from Purchased Electricity |
| Required – Scope 3 |  |  |  |  |  |
| Nco | Number of animals purchased per livestock class, including origin of purchase |  | head | Farm stock records, invoices | Purchased Livestock |
| Qj | Quantity of each feed type purchased |  | tonnes | Invoices, receipts or other records of purchased quantities | Purchased Feed |
| Qm | Quantity of each mineral supplement purchased |  | tonnes | Invoices, receipts or other records of purchased quantities | Purchased Mineral Supplements |
| Fcf | Fraction of inorganic fertiliser components in each fertiliser |  | wt% | Product information label | Purchased Fertiliser |
| Qh | Quantity of each agrichemical used |  | kg | Invoices, receipts or other records of purchased quantities | Purchased Herbicides/Pesticides |
| Ap | Activity data for each service purchased, up to the farm-gate. |  | Depends on purchased service | Invoices, receipts or other records of quantity of service provided | Purchased Services and Contractors |
| Qwt | Weight of waste sent to waste treatment facility |  | tonnes | Invoices for skip disposal, weighbridge receipts, farm records | Management of Waste |
| Vwt | Volume of waste being sent waste treatment facility (only required if weight is unknown) |  | m3 | Invoices for skip disposal, or other farm records | Management of Waste |

The checklist below summarises all the optional data points, required for Method 2, that may be collected at farm-level for a dairy system to complete the equations detailed in subsequent chapters based on the dairy module map.

Table 11: Optional data for dairy system

| Parameter | Data to be collected | Data collected Yes/No/N/A | Unit | Data source | Module |
| --- | --- | --- | --- | --- | --- |
| Optional – Scope 1 |  |  |  |  |  |
| Dj | Duration of stay for each dairy cattle input class |  | days | Farm stock records; system type records or purchase and sales may be evaluated to determine average duration of each cattle input class | Enteric Methane & Manure Management |
| Wj | Liveweight of livestock |  | kg | Farm stock records and herd flow model | Enteric Methane & Manure Management |
| LWGj | Liveweight gain of livestock |  | kg/day | Farm stock records and herd flow model | Enteric Methane & Manure Management |
| DMDj | Dry matter digestibility of feed |  | fraction | If average DMD for the reporting period of different livestock classes is known based on farm records of feed sources and quality, a farm-specific DMD value may be used. The ADCC, 2025 [2] provides support on how to calculate DMD from feed sources. | Enteric Methane & Manure Management |
| FCj | Fat content in fat and protein corrected milk |  | per cent | Farm records specific to the milk production of the reporting period | Enteric Methane |
| PCj | Protein content in fat and protein corrected milk |  | per cent | Farm records specific to the milk production of the reporting period | Enteric Methane |
| MMSm | Fraction of waste voided in each manure management system |  | fraction | Annual fraction of time spent in each area may be estimated from farm records for time split between paddock and housed for each livestock class. Alternatively, the ADCC tool provides a questionnaire-based approach to these calculations, which may be utilised to calculate fm [2]. | Manure Management |
| SSj | Fraction of waste separated by pre-treatment of manure |  | fraction | Farm records on what fraction of waste is screened before flowing to primary MMS | Manure Management |
| CPj | Crude protein of feed |  | fraction | Farm records or feed receipt stating crude protein of TMR (total mixed ration) for different livestock classes. The ADCC, 2025 [2] provides support on how to calculate CP from feed sources. | Manure Management |
| MCFim | Methane conversion factor for temperature zone and system |  | fraction | Select relevant MCF for MMS and temperature zone | Manure Management |
| Nfj | N application rate of inorganic fertiliser used in production system |  | kg N/ha | Farm records of fertiliser application rates, or calculated from: Nfj=Total fertiliser applied (kgN)Total area (ha) Data on total fertiliser applied and total area should be sourced from farm invoices and spatial records respectively. | Fertiliser Use – Inorganic Fertiliser Use |
| EFj,N2O | Emission factor for direct nitrous oxide emissions from inorganic fertiliser |  | kg N2O-N/kg N | Calculated from relevant equations in Chapter 5, sections 5.1.1.2 | Fertiliser Use – Inorganic Fertiliser Use |
| Yp | Average yield of pasture |  | t DM/ha | Yield records | Agriculture Residue Management - Pasture Residues |
| Optional – Scope 2 |  |  |  |  |  |
| RECsurrendered | Eligible Renewable Energy Certificates voluntarily surrendered in the reporting year |  | MWh | Records of transactions | Purchased Electricity & Upstream Emissions from Purchased Electricity (market-based approach only) |
| REConsite | Eligible Renewable Energy Certificates that have been or will be issued for electricity produced on-site during the reporting period and consumed by the entity |  | MWh | Records of transactions | Purchased Electricity & Upstream Emissions from Purchased Electricity (market-based approach only) |
| Optional – Scope 3 |  |  |  |  |  |
| Wco | Liveweight of purchased livestock at point of purchase |  | kg/head | Purchasing records/invoices and herd flow models | Purchased Livestock |
| EF | Supplier-provided life cycle emission factor for any of the following purchased goods: livestock, feed, mineral supplements, fertiliser, agrichemicals, or lime |  | t CO2e/t purchased good | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 14067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard | Corresponding Scope 3 Purchased Good Module |

## Sheep

This section outlines all relevant Modules and data checklists to develop a GHG inventory for a sheep farming system. Relevant emission sources and therefore relevant Modules for a specific entity will depend on an entity-specific boundary set as described in Chapter 5 of the CRF. For example, if a specific entity does not use lime, then this emission source and Module will not be relevant for this specific entity.

Module map and input data checklists are designed for estimation of emissions from cradle-to-end of the farming system/farm-gate. If the boundary of the organisation extends to processing, packing and transport of sheep or wool products, the post-production Module can also be implemented to estimate emissions from these sources. Refer to Chapter 1, Section 1.3, for additional information on coverage and limitations of the guidance.

### Module Map

| Emission Activity/Source | Relevant Module |
| --- | --- |
| Direct GHG emissions (Scope 1) |  |
| Enteric Methane | 3.4 |
| Manure Management | 4.4 |
| Fertiliser Use | 5.1 – 5.5 |
| Agriculture residue management | 6.2 – 6.3 |
| Transport Fuel | 8.1 |
| Stationary Fuel | 8.2 |
| Solid Waste Treatment | 10.1 |
| Indirect GHG emissions from imported energy (Scope 2) |  |
| Purchased Electricity | 14.1 |
| Indirect GHG emissions (Scope 3) |  |
| Purchased Livestock | 15.1 |
| Purchased Feed | 15.2 |
| Purchased Mineral Supplements | 15.3 |
| Purchased Fertiliser | 15.4 |
| Purchased Agrichemicals | 15.5 |
| Purchased Lime | 15.6 |
| Purchased Services/Contractors | 15.7 |
| Other Purchased Goods and Services | 15.10 |
| Upstream Emissions from Fuel | 15.11 |
| Upstream Emissions of Purchased Electricity | 15.12 |
| Management of Waste | 15.13 |

### Data Collection Checklist

The checklist below summarises all the required input data, required for Method 1, that should be collected at farm-level for a sheep farming system to complete the equations detailed in subsequent chapters based on the sheep module map. Data collected may be listed as ‘not applicable’ for emissions sources that are not relevant to the specific entity. Further information regarding relevant sources and sink can be found in Chapter 5.2 of the CRF.

Additional information and guidance on specific parameters can be found within the sheep guidance in Chapter 3 – Section; 3.4 and Chapter 4 – Section; 4.4. With additional related parameters found within Chapter 5 – Sections; 5.1 - 5.5, Chapter 6 – Sections; 6.2 and 6.3 and Chapter 8 – Sections; 8.1 and 8.2, Chapter 14 – Section; 14.1 and Chapter 15 – Sections; 15.1 – 15.7 and 15.10 – 15.13.

Table 12: Required data for sheep system

| Parameter | Data to be collected | Data collected Yes/No/N/A | Unit | Data Source | Module |
| --- | --- | --- | --- | --- | --- |
| Required – Scope 1 |  |  |  |  |  |
| Njk | Number of sheep in each time-period and class |  | head | Farm records and herd flow model | Enteric Methane & Manure Management |
| LRjk=3,4 | Number of ewes and maidens lambing in the season as a proportion of the total number of ewes and maidens carried in the season |  | per cent | Lambing rate should be based on weaning numbers. The percentage of ewes carrying a lamb at weaning and percentage of maidens carrying lamb at weaning should be reported separately where available. | Enteric Methane |
| LMRjk=3,4 | Lamb marking rate for ewes and maidens in each lambing season or time period |  | per cent | Lamb marking rate should be calculated by number of lambs marked per ewe mated. | Enteric Methane |
| GWk | Average greasy wool fleece weight for each sheep class |  | kg/head | Should be based on average wool fleece weight for each class. Invoices from wool sales may be used to confirm the weight of wool sold in the reporting period. If not known this may be calculated from the total greasy weight sold divided by the number of head in each class shorn. | Manure Management |
| Yk | Average clean wool yield for each sheep class |  | fraction | Wool testing results at point of sale for the clean wool percentage for each class. If only total greasy and total clean weights are known the fraction of clean may be calculated as: Y =cleanweightgreasyweight | Manure Management |
| TMjf | Amount of each inorganic fertiliser applied to each production system |  | kg | Receipts, invoices, farm records | Fertiliser Use – Inorganic Fertiliser Use & Purchased Fertiliser |
| FNinorganic,f | Fraction of nitrogen in each inorganic fertiliser |  | kg N/kg | Default values for common inorganic fertilisers based on average fertiliser composition from [1], or calculated from product label information. | Fertiliser Use – Inorganic fertiliser Use |
| FUf | Fraction of urea in each inorganic fertiliser applied |  | kg urea/kg | Default values for common inorganic fertilisers based on average fertiliser composition from [1], and or calculated from product label information. | Fertiliser Use – Inorganic fertiliser Use |
| Morganic,jf | Amount of each organic fertiliser applied to soils in production system |  | kg | Invoices of purchases, farm records of application rates | Fertiliser Use – Organic Fertiliser Use |
| FNorganic,f | Nitrogen fraction of each organic fertiliser applied to soils |  | kg N/kg | Supplier-provided values or product label information | Fertiliser Use – Organic Fertiliser Use |
| Mlime | Total amount of lime applied to soils |  | kg | Invoices, receipts | Fertiliser Use – Lime Application |
| FracLime | Fraction of lime as limestone |  | dimensionless | Product information label | Fertiliser Use – Lime Application |
| FracDol | Fraction of lime as dolomite |  | dimensionless | Product information label | Fertiliser Use – Lime Application |
| Ap | Area of pasture crop renewed or removed |  | ha | Geospatial files, farm records | Agriculture Residue Management - Pasture Residues |
| QTrans,q | Quantity of each fuel used for transportation (including off-road vehicles and equipment, aircraft, and/or vessels), up to the farm-gate. |  | Depends on fuel type | Invoices, fuel cards, receipts | Transport Fuel & Upstream Emissions |
| QSC,q | Quantity of each fuel used for stationary combustion, up to the farm-gate. |  | Depends on fuel type | Invoices, fuel cards, receipts | Stationary Fuel & Upstream Emissions |
| Qwt | Quantity of waste per type being treated within the farm-gate. |  | tonnes | Farm records of weight of waste disposed | Solid Waste Treatment |
| Vwt | Volume of waste per type being treated within the farm-gate. |  | m3 | Only required if Qwt is not known. Estimated from trailer-loads, or other known volume of waste transported. | Solid Waste Treatment |
| Required – Scope 2 |  |  |  |  |  |
| Qelec | Quantity of electricity purchased from the grid, up to the farm-gate. |  | kWh | Invoices from electricity supplier | Purchased Electricity & Upstream Emissions from Purchased Electricity |
| Required – Scope 3 |  |  |  |  |  |
| Nco | Number of animals purchased per livestock class, including sheep type |  | head | Farm stock records, invoices | Purchased Livestock |
| Qj | Quantity of each feed type purchased |  | t | Invoices, receipts or other records of purchased quantities | Purchased Feed |
| Qm | Quantity of each mineral supplement purchased |  | t | Invoices, receipts or other records of purchased quantities | Purchased Mineral Supplements |
| Fcf | Fraction of inorganic fertiliser components in each fertiliser |  | wt% | Product information label | Purchased Fertiliser |
| Qh | Quantity of each agrichemical used |  | kg | Invoices, receipts or other records of purchased quantities | Purchased Herbicides/Pesticides |
| Ap | Activity data for each service purchased, up to the farm-gate. |  | Depends on purchased service | Invoices, receipts or other records of quantity of service provided | Purchased Services and Contractors |
| Qwt | Weight of waste sent to waste treatment facility |  | tonnes | Invoices for skip disposal, weighbridge receipts, farm records | Management of Waste |
| Vwt | Volume of waste being sent waste treatment facility (only required if weight is unknown) |  | m3 | Invoices for skip disposal, or other farm records | Management of Waste |

The checklist below summarises all the optional data points, required for Method 2, that may be collected at farm-level for a sheep system to complete the equations detailed in subsequent chapters based on the sheep module map.

Table 13: Optional data for sheep system

| Parameter | Data to be collected | Data collected Yes/No/N/A | Unit | Data source | Module |
| --- | --- | --- | --- | --- | --- |
| Optional – Scope 1 |  |  |  |  |  |
| Dj | Duration of stay for each sheep input class |  | days | Farm stock records and herd flow model | Enteric Methane & Manure Management |
| Wjk | Liveweight of livestock per sheep class and season |  | kg | Farm stock records and herd flow model | Enteric Methane & Manure Management |
| DMDjk | Dry matter digestibility of feed |  | fraction | Farm records | Enteric Methane & Manure Management |
| DMAik | Dry matter availability of feed per livestock group |  | per cent | Calculated based on available pasture growth and supplementary feed provided. During periods of containment feeding, if DMA can be calculated more readily due to control of intake, this may be applied to the sheep classes and time periods when this feeding is taking place and DMA is known. | Enteric Methane |
| CPjk | Crude protein of feed |  | fraction | Farm records for each season and class of stock. | Manure Management |
| LWGik | Liveweight gain of livestock |  | kg | Farm records and see herd flow model | Manure Management |
| MCFim | Methane conversion factor for temperature zone and system |  | fraction | Select relevant MCF for MMS and temperature zone | Manure Management |
| Nfj | N application rate of inorganic fertiliser used in production system |  | kg N/ha | Farm records of fertiliser application rates, or calculated from: Nfj=Total fertiliser applied (kgN)Total area (ha) Data on total fertiliser applied and total area should be sourced from farm invoices and spatial records respectively. | Fertiliser Use – Inorganic Fertiliser Use |
| EFj,N2O | Emission factor for direct nitrous oxide emissions from inorganic fertiliser |  | kg N2O-N/kg N | Calculated from relevant equations in Chapter 5, sections 5.1.1.2 | Fertiliser Use – Inorganic Fertiliser Use |
| Yp | Average yield of pasture |  | t DM/ha | Yield records | Agriculture Residue Management - Pasture Residues |
| Optional – Scope 2 |  |  |  |  |  |
| RECsurrendered | Eligible Renewable Energy Certificates voluntarily surrendered in the reporting year |  | MWh | Records of transactions | Purchased Electricity & Upstream Emissions from Purchased Electricity (market-based approach only) |
| REConsite | Eligible Renewable Energy Certificates that have been or will be issued for electricity produced on-site during the reporting period and consumed by the entity |  | MWh | Records of transactions | Purchased Electricity & Upstream Emissions from Purchased Electricity (market-based approach only) |
| Optional – Scope 3 |  |  |  |  |  |
| Wco | Liveweight of purchased livestock at point of purchase |  | kg/head | Purchasing records/invoices and herd flow models | Purchased Livestock |
| EF | Supplier-provided life cycle emission factor for any of the following purchased goods: livestock, feed, mineral supplements, fertiliser, agrichemicals, or lime |  | t CO2e/t purchased good | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 14067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard | Corresponding Scope 3 Purchased Good Module |

## Poultry

This section outlines all relevant Modules and data checklists to develop a GHG inventory for a poultry farming system. Relevant emission sources and therefore relevant Modules for a specific entity will depend on an entity-specific boundary set as described in Chapter 5 of the CRF. For example, if a specific entity does not use lime, then this emission source and Module will not be relevant for this specific entity.

Module map and input data checklists are designed for estimation of emissions from cradle-to-end of the farming system/farm-gate. If the boundary of the organisation extends to processing, packing and transport of poultry products or eggs, the post-production Module can also be implemented to estimate emissions from these sources. Refer to Chapter 1, Section 1.3, for additional information on coverage and limitations of the guidance.

### Module Map

| Emission Activity/Source | Relevant Module |
| --- | --- |
| Direct GHG emissions (Scope 1) |  |
| Manure Management | 4.6 |
| Transport Fuel | 8.1 |
| Stationary Fuel | 8.2 |
| Solid Waste Treatment | 10.1 |
| Indirect GHG emissions from imported energy (Scope 2) |  |
| Purchased Electricity | 14.1 |
| Indirect GHG emissions (Scope 3) |  |
| Purchased Livestock | 15.1 |
| Purchased Feed | 15.2 |
| Purchased Services/Contractors | 15.7 |
| Other Purchased Goods and Services | 15.10 |
| Upstream Emission from Fuel | 15.11 |
| Upstream Emissions of Purchased Electricity | 15.12 |
| Management of Waste | 15.13 |

### Input Data Checklist

The checklist below summarises all the required input data, required for Method 1, that should be collected at farm-level for a poultry system to complete the equations detailed in subsequent chapters based on the poultry module map. Data collected may be listed as ‘not applicable’ for emissions sources that are not relevant to the specific entity. Further information regarding relevant sources and sink can be found in Chapter 5.2 of the CRF.

Additional information and guidance on specific parameters can be found within Chapter 4 – Section; 4.6 for a poultry system. With additional related parameters found within, Chapter 6 – Sections; 6.1 and 6.2, Chapter 10 – Section; 10.1, Chapter 14 – Section; 14.1, and Chapter 15 – Sections; 15.1, 15.2, 15.7, 15.10 – 15.12.

Table 14: Required data for a poultry system

| Parameter | Data to be collected | Data collected Yes/No/N/A | Unit | Data Source | Module |
| --- | --- | --- | --- | --- | --- |
| Required – Scope 1 |  |  |  |  |  |
| Njk | Numbers of poultry in each class and subclass |  | birds | Farm flock records. | Manure Management |
| Djk | Duration of stay for each class and subclass of poultry |  | days | Farm flock records; housing/system records and placement and dispatch documentation may be used to determine average duration of stay For broiler chickens records of depletions, or “flock thinning” may be used to ascertain length of stay within each class and subclass of poultry. | Manure Management |
| MMSjmT=1 | Fraction of manure in each primary MMS |  | fraction | Farmer records as to what fraction of manure flows to each system based on the available MMS options (m). If treatment is different for different poultry groups this may also be recorded per poultry group. | Manure Management |
| MMSjmT=2 | Fraction of manure at treatment stage 2 for each MMS. i.e. what fraction of manure goes to what MMS once it has been removed from the house. If there is only one available MMS then this should be equal to 1. This may be different for different poultry groups. |  | fraction | Farmer records as to what fraction of manure flows to each system based on the available MMS options (m). If treatment is different for different poultry groups this may also be recorded per poultry group. | Manure Management |
| PF | Fraction of manure from MMS applied to soil within the farm boundary |  | fraction | Farm source data of manure sales or transport weights. When all manure treated in MMS is applied within the farm boundary and none is sold off-site to be used in a different enterprise, then PF = 1. | Manure Management |
| QTrans,q | Quantity and type of fuel used for transportation, (including off-road vehicles and equipment, aircraft, and/or vessels) up to the farm-gate. |  | Depends on fuel type | Invoices, fuel cards, receipts. | Transport Fuel & Upstream Emissions |
| QSC,q | Quantity and type of fuel used for stationary equipment, up to the farm-gate. |  | Depends on fuel type | Invoices, fuel cards, receipts. | Stationary Fuel & Upstream Emissions |
| Qwt | Quantity of waste per type being treated within the farm-gate. |  | tonnes | Farm records (e.g., number and weight of deceased livestock). | Solid Waste Treatment |
| Vwt | Volume of waste per type being treated within the farm-gate. |  | m3 | Only required if Qwt is not known. Estimated from trailer-loads, or other known volume of waste transported. | Solid Waste Treatment |
| Required – Scope 2 |  |  |  |  |  |
| Qelec | Quantity of electricity purchased from the grid, up to the farm-gate. |  | kWh | Invoices. | Purchased Electricity & Emissions from Purchased Electricity |
| Required – Scope 3 |  |  |  |  |  |
| Nco | Number of animals purchased per livestock type and origin |  | head | Farm stock records, invoices. | Purchased Livestock |
| Qj | Quantity of feed type purchased |  | t | Invoices, receipts or other records of purchased quantities. | Purchased Feed |
| Ap | Activity data for service purchased, up to the farm-gate |  | Depends on purchased service | Invoices, receipts, farm records. | Purchased services and contractors |
| Qwt | Quantity of waste per type being disposed beyond the farm-gate boundary. |  | tonnes | Weighbridge records, skip disposal invoices. | Management of Waste |
| Vwt | Volume of waste per type being disposed beyond the farm-gate boundary. |  | m3 | Only required if Qwt is not known. Estimated from known volumes of waste transported (e.g., number of skips filled). | Management of Waste |

The checklist below summarises all the optional data points, required for Method 2, that may be collected at farm-level for a poultry system to complete the equations detailed in subsequent chapters based on the poultry module map.

Table 15: Optional data for a poultry system

| Parameter | Data to be collected | Data collected Yes/No/N/A | Unit | Data Source | Module |
| --- | --- | --- | --- | --- | --- |
| Optional – Scope 1 |  |  |  |  |  |
| Ijk | Dry matter intake for each class and subclass |  | kg DM/head/day | Farm records relating to dry matter assessment of total mixed ration (TMR), and daily feed weights for each class. For layers the Australian Egg Tool [3] provides a calculation of intake based on total feed delivered for each subclass of layer (cage, barn, free range) and feed composition, if known. | Manure Management |
| DMDjk | Dry matter digestibility |  | fraction | If average DMD for the reporting period of different poultry classes is known based on farm records of feed sources and quality, a farm specific DMD value may be used. For any stock classes where farm records of feed sources and quality are not available, the National Inventory Report default shall be applied. | Manure Management |
| CPj | Crude protein content of feed intake |  | fraction | Farm records or feed receipt stating crude protein of TMR (total mixed ration) for different poultry classes. For any poultry classes where farm records of feed sources and quality are not available, the National Inventory Report default shall be applied. | Manure Management |
| Optional – Scope 2 |  |  |  |  |  |
| RECsurrendered | Eligible Renewable Energy Certificates voluntarily surrendered in the reporting year. |  | MWh | Retailer invoice or Clean Energy Regulator REC registry. | Purchased Electricity & Emissions from Purchased Electricity (market-based approach only) |
| REConsite | Eligible Renewable Energy Certificates that have been or will be issued for electricity produced on-site during the reporting period and consumed by the entity. |  | MWh | Clean Energy Regulator REC registry and Meter readings. | Purchased Electricity & Emissions from Purchased Electricity (market-based approach only) |
| Optional – Scope 3 |  |  |  |  |  |
| Wco | Liveweight of purchased livestock at point of purchase |  | kg/head | Purchasing records/invoices and herd flow models. | Purchased Livestock |
| EFco | Emission factor for purchased livestock per type of livestock |  | kg CO2e/kg | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 15067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. | Purchased Livestock |
| EFj | Emission factor of purchased feed type |  | kg CO2e/kg | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 15067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. | Purchased Feed |

## Swine

This section outlines all relevant Modules and data checklists to develop a GHG inventory for a swine system. Relevant emission sources and therefore relevant Modules for a specific entity will depend on an entity-specific boundary set as described in Chapter 5 of the CRF. For example, if a specific entity does not use lime, then this emission source and Module will not be relevant for this specific entity.

Module map and input data checklists are designed for estimation of emissions from cradle-to-end of the farming system/farm-gate. If the boundary of the organisation extends to processing, packing and transport of pork, the post-production Module can also be implemented to estimate emissions from these sources. Refer to Chapter 1, Section 1.3, for additional information on coverage and limitations of the guidance.

### Module Map

| Emission Activity/Source | Relevant Module |
| --- | --- |
| Direct GHG emissions (Scope 1) |  |
| Enteric Methane | 3.5 |
| Manure Management | 4.5 |
| Transport Fuel | 8.1 |
| Stationary Fuel | 8.2 |
| Solid Waste Treatment | 10.1 |
| Indirect GHG emissions from imported energy (Scope 2) |  |
| Purchased Electricity | 14.1 |
| Indirect GHG emissions (Scope 3) |  |
| Purchased Livestock | 15.1 |
| Purchased Feed | 15.2 |
| Purchased Services/Contractors | 15.7 |
| Other Purchased Goods and Services | 15.10 |
| Upstream Emission from Fuel | 15.11 |
| Upstream Emissions of Purchased Electricity | 15.12 |
| Management of Waste | 15.13 |

### Input Data Checklist

The checklist below summarises all the required input data, required for Method 1, that should be collected at farm-level for a swine system to complete the equations detailed in subsequent chapters based on the swine module map. Data collected may be listed as ‘not applicable’ for emissions sources that are not relevant to the specific entity. Further information regarding relevant sources and sink can be found in Chapter 5.2 of the CRF.

Additional information and guidance on specific parameters can be found within Chapter 3 – Section; 3.5 and Chapter 4 – Section; 4.5 for a swine system. With additional related parameters found within Chapter 6 – Sections; 6.1 and 6.2, Chapter 10 – Section; 10.1, Chapter 14 – Section; 14.1, and Chapter 15 – Sections; 15.1, 15.2, 15.7, 15.10 – 15.12.

Table 16: Required data for a swine system

| Parameter | Data to be collected | Data collected Yes/No/N/A | Unit | Data Source | Module |
| --- | --- | --- | --- | --- | --- |
| Required – Scope 1 |  |  |  |  |  |
| Nj | Number of swine of each class |  | head | Farm stock records; system type records, purchase and sales records and National Vendor Declarations or NLID transfer records may be evaluated to determine average length of stay of each class. | Enteric Fermentation & Manure Management |
| Dj | Average number of days on farm for each swine class |  | days | Farm stock records; system type records or purchase and sales records and National Vendor Declarations or NLID transfer records may be evaluated to determine average length of stay of each class. Breeding stock that are kept year-round shall be assumed to be on the farm for 365 days. | Enteric Fermentation & Manure Management |
| MMSjmT=1 | Fraction of manure in each primary MMS |  | fraction | Farmer records as to what fraction of manure flows to each system based on the available MMS options (m). This fraction should be based on either weight or volume of manure. If treatment is different for different swine groups this may also be recorded per swine group. See Chapter 4, Section 4.5.2.1 for primary MMS options. | Manure Management |
| MMSjmT=2 | Fraction of manure in each secondary MMS |  | fraction | Farmer records as to what fraction of manure flows to each system based on the available MMS options (m). This fraction should be based on either weight or volume of manure. If treatment is different for different swine groups this may also be recorded per swine group. See Chapter 4, Section 4.5.2.1 for secondary MMS options. | Manure Management |
| PF | Fraction of manure from MMS applied to soil within the farm boundary |  | fraction | Farmer to specify fraction of manure applied to soil within the farm boundary. | Manure Management |
| QTrans,q | Quantity and type of fuel used for transportation, (including off-road vehicles and equipment, aircraft, and/or vessels) up to the farm-gate. |  | Depends on fuel type | Invoices, fuel cards, receipts. | Transport Fuel & Upstream Emissions |
| QSC,q | Quantity and type of fuel used for stationary equipment, up to the farm-gate. |  | Depends on fuel type | Invoices, fuel cards, receipts. | Stationary Fuel & Upstream Emissions |
| Qwt | Quantity of waste per type being treated within the farm-gate. |  | tonnes | Farm records (e.g., number and weight of deceased livestock). | Solid Waste Treatment |
| Vwt | Volume of waste per type being treated within the farm-gate. |  | m3 | Only required if Qwt is not known. Estimated from trailer-loads, or other known volume of waste transported. | Solid Waste Treatment |
| Required – Scope 2 |  |  |  |  |  |
| Qelec | Quantity of electricity purchased from the grid, up to the farm-gate. |  | kWh | Invoices. | Purchased Electricity & Emissions from Purchased Electricity |
| Required – Scope 3 |  |  |  |  |  |
| Nco | Number of animals purchased per livestock type and origin |  | head | Farm stock records, invoices. | Purchased Livestock |
| Qj | Quantity of feed type purchased |  | t | Invoices, receipts or other records of purchased quantities. | Purchased Feed |
| Ap | Activity data for service purchased, up to the farm-gate |  | Depends on purchased service | Invoices, receipts, farm records. | Purchased services and contractors |
| Qwt | Quantity of waste per type being disposed beyond the farm-gate boundary. |  | tonnes | Weighbridge records, skip disposal invoices. | Management of Waste |
| Vwt | Volume of waste per type being disposed beyond the farm-gate boundary. |  | m3 | Only required if Qwt is not known. Estimated from known volumes of waste transported (e.g., number of skips filled). | Management of Waste |

The checklist below summarises all the optional data points, required for Method 2, that may be collected at farm-level for a swine system to complete the equations detailed in subsequent chapters based on the swine module map.

Table 17: Optional data for a swine system

| Parameter | Data to be collected | Data collected Yes/No/N/A | Unit | Data Source | Module |
| --- | --- | --- | --- | --- | --- |
| Optional – Scope 1 |  |  |  |  |  |
| Ij | Average weight of fed ingested per head class of swine within farming system per day. |  | kg/head/day | Farm records relating to daily feed and feed waste weights for each class. PigBal [4] may be utilized to support the calculation annual feed fed per head to different classes of swine. It should be noted that feeding weights may change over time therefore an average should be calculated based on the amount of feed throughout time period reported for the swine class. | Enteric Methane |
| SSj | Fraction of volatile solids separated by pre-treatment of manure |  | fraction | User specified input as to what fraction of volatile solids is screened before flowing into primary MMS. PigBal [4] may be used to support the estimation of this fraction based on the options effluent pre-treatment. | Manure Management |
| VSi | Volatile solids production from swine in each class |  | kg/head/day | PigBal [4] may be used to calculate the excretion of volatile solids per swine class per head per day. This calculation is based on the total solids excreted less the fixed solids excreted. | Manure Management |
| MCFim | Methane conversion factor for climate zone and system |  | fraction | Selection relevant MCF for MMS and temperature zone. | Manure Management |
| SNj | Fraction of nitrogen separated by pre-treatment of manure |  | fraction | User specified input as to what fraction of nitrogen is screened before flowing into primary MMS. PigBal [4] may be used to support the estimation of this fraction based on the options effluent pre-treatment. | Manure Management |
| NWj | Nitrogen waste production per pig and swine class per day including both animal waste and feed waste |  | kgN/head/day | PigBal [4] may be used to calculate the nitrogen waste produced per swine class per head per day. This is calculated by mass balance, as the difference between the amount in the ingested feed and the amount retained by the animal as live weight gain and the nitrogen in the waste feed. | Manure Management |
| Optional – Scope 2 |  |  |  |  |  |
| RECsurrendered | Eligible Renewable Energy Certificates voluntarily surrendered in the reporting year. |  | MWh | Retailer invoice or Clean Energy Regulator REC registry. | Purchased Electricity & Emissions from Purchased Electricity (market-based approach only) |
| REConsite | Eligible Renewable Energy Certificates that have been or will be issued for electricity produced on-site during the reporting period and consumed by the entity. |  | MWh | Clean Energy Regulator REC registry and Meter readings. | Purchased Electricity & Emissions from Purchased Electricity (market-based approach only) |
| Optional – Scope 3 |  |  |  |  |  |
| Wco | Liveweight of purchased livestock of type c from origin or at point of purchase |  | kg/head | Purchasing records/invoices and herd flow models. | Purchased Livestock |
| EFco | Emission factor for purchased livestock per type of livestock c and origin o |  | kg CO2e/kg | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 15067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. | Purchased Livestock |
| EFj | Emission factor of purchased feed type j |  | kg CO2e/kg | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 15067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. | Purchased Feed |

## Other Livestock

This section outlines all relevant Modules and data checklists to develop a GHG inventory of for other livestock systems. Relevant emission sources and therefore relevant Modules for a specific entity will depend on an entity-specific boundary set as described in Chapter 5 of the CRF. For example, if a specific entity does not use lime, then this emission source and Module will not be relevant for this specific entity.

Other livestock systems included in this guidance are

- Buffalo,

- Goats,

- Deer,

- Camels,

- Alpacas,

- Horses,

- Mules/asses, and

- Emus/ostriches.

Module map and input data checklists are designed for estimation of emissions from cradle-to-end of the farming system/farm-gate. If the boundary of the organisation extends to processing, packing and transport of animal products, the post-production Module can also be implemented to estimate emissions from these sources. Refer to Chapter 1, Section 1.3, for additional information on coverage and limitations of the guidance.

### Module Map

| Emission Activity/Source | Relevant Module |
| --- | --- |
| Direct GHG emissions (Scope 1) |  |
| Enteric Methane | 3.6 |
| Manure Management | 4.7 |
| Fertiliser Use | 5.1 – 5.5 |
| Agriculture Residue Management | 6.2 – 6.4 |
| Transport Fuel | 8.1 |
| Stationary Fuel | 8.2 |
| Solid Waste Treatment | 10.1 |
| Indirect GHG emissions from imported energy (Scope 2) |  |
| Purchased Electricity | 14.1 |
| Indirect GHG emissions (Scope 3) |  |
| Purchased Livestock | 15.1 |
| Purchased Feed | 15.2 |
| Purchased Mineral Supplements | 15.3 |
| Purchased Fertiliser | 15.4 |
| Purchased Agrichemicals | 15.5 |
| Purchased Lime | 15.6 |
| Purchased Services/Contractors | 15.7 |
| Other Purchased Goods and Services | 15.10 |
| Upstream Emission from Fuel | 15.11 |
| Upstream Emissions of Purchased Electricity | 15.12 |
| Management of Waste | 15.13 |

### Input Data Checklist

The checklist below summarises all the required input data, required for Method 1, that should be collected at farm-level for other livestock systems to complete the equations detailed in subsequent chapters based on the other livestock module map. Data collected may be listed as ‘not applicable’ for emissions sources that are not relevant to the specific entity. Further information regarding relevant sources and sink can be found in Chapter 5.2 of the CRF.

Additional information and guidance on specific parameters can be found within Chapter 3 – Section; 3.6 and Chapter 4 – Section; 4.7 for other livestock system. With additional related parameters found within Chapter 5 – Sections; 5.1 – 5.5, Chapter 6 – Sections; 6.1 and 6.2, Chapter 10 – Section; 10.1, Chapter 14 – Section; 14.1, and Chapter 15 Sections; 15.1 – 15.7, 15.8 15.10 – 15.12.

Table 18: Required data for other livestock systems

| Parameter | Data to be collected | Data collected Yes/No/N/A | Unit | Data Source | Module |
| --- | --- | --- | --- | --- | --- |
| Required – Scope 1 |  |  |  |  |  |
| Nj | Average annual numbers of each other livestock type |  | head | Farm stock records noting that the enteric fermentation factor applies assumes animals are on farm all year. If there are changes in livestock numbers throughout the year, calculate the average numbers of livestock on farm for the reporting year. | Enteric Methane & Manure Management |
| TMjf | The total mass of inorganic fertiliser type applied to production system |  | kg | Receipts, invoices, farm records. | Fertiliser Use – Inorganic Fertiliser Use & Purchased Fertiliser |
| FNinorganic,f | Fraction of nitrogen in inorganic fertiliser |  | kg N/kg | Default values for common inorganic fertilisers based on average fertiliser composition from [1], or calculated from product label information. See additional guidance information in Chapter 5, Section 5.1.2.1. | Fertiliser Use – Inorganic Fertiliser Use |
| FUf | Fraction of urea in inorganic fertiliser |  | kg urea/kg | Default values for common inorganic fertilisers based on average fertiliser composition from [1] and [5] or calculated from product label information. See additional guidance information in Chapter 5, Section 5.1.2.1. | Fertiliser Use – Inorganic Fertiliser Use |
| Morganic,jf | Mass of organic fertiliser type (manure) applied to soils in production system |  | kg | Invoices of purchases, farm records of application rates Where the manure was generated on-farm, Mmanure,jf is not required and instead the value of MNSoil shall be calculated using outputs from the MMS emission estimation equations (Chapter 4). | Fertiliser Use – Organic Fertiliser Use |
| FNorganic,f | Nitrogen fraction of the organic fertiliser type (manure) applied to soils |  | kg N/kg | Supplier-provided values or product label information should be used. Default values from IPCC Volume 4 Chapter 5, Table 5.5c [6] (provided in Table A.1.2.2.6 of the Appendix) may be used for manure-based organic fertiliser that was purchased off-farm, where the livestock that generated the organic fertiliser is known. See additional guidance information in Chapter 5, Section 5.2.2.1. | Fertiliser Use – Organic Fertiliser Use |
| Mlime | The mass of lime applied to soils |  | kg | Invoices, receipts. | Fertiliser Use – Lime Application & Purchased Lime |
| FracLime | The fraction of lime as limestone |  | dimensionless | The limestone (calcium carbonate - CaCO3) content of lime may be determined based on the content by weight per cent (wt%) stated on the product information label. Product information label values expressed as wt% shall be converted into a fraction (e.g., for a 60 wt% content of limestone, FracLime = 0.6). | Fertiliser Use – Lime Application |
| FracDol | The fraction of lime as dolomite |  | dimensionless | The dolomite (magnesium carbonate - CaMg(CO3)2) content of lime may be determined based on the content by weight per cent (wt%) on the product information label Product information label values expressed as wt% shall be converted into a fraction (e.g., for a 35 wt% content of dolomite, FracDol = 0.35). | Fertiliser Use – Lime Application |
| Ap | Area of pasture crop renewed or removed |  | ha | Geospatial files, farm records. | Agriculture Residue Management - Pasture Residues |
| QTrans,q | Quantity and type of fuel used for transportation, (including off-road vehicles and equipment, aircraft, and/or vessels) up to the farm-gate. |  | Depends on fuel type | Invoices, fuel cards, receipts. | Transport Fuel & Upstream Emissions |
| QSC,q | Quantity and type of fuel used for stationary equipment, up to the farm-gate. |  | Depends on fuel type | Invoices, fuel cards, receipts. | Stationary Fuel & Upstream Emissions |
| Qwt | Quantity of waste per type being treated within the farm-gate. |  | tonnes | Farm records (e.g., number and weight of deceased livestock). | Solid Waste Treatment |
| Vwt | Volume of waste per type being treated within the farm-gate. |  | m3 | Only required if Qwt is not known. Estimated from trailer-loads, or other known volume of waste transported. | Solid Waste Treatment |
| Required – Scope 2 |  |  |  |  |  |
| Qelec | Quantity of electricity purchased from the grid, up to the farm-gate. |  | kWh | Invoices. | Purchased Electricity & Emissions from Purchased Electricity |
| Required – Scope 3 |  |  |  |  |  |
| Nco | Number of animals purchased per livestock type and origin |  | head | Farm stock records, invoices. | Purchased Livestock |
| Qj | Quantity of feed type purchased |  | t | Invoices, receipts or other records of purchased quantities. | Purchased Feed |
| Qm | Quantity of mineral supplement per type purchased |  | t | Invoices, receipts or other records of purchased quantities. | Purchased Mineral Supplements |
| Fcf | Fraction of component (N, P, K and S) contained within the inorganic fertiliser, by weight |  | wt% | Product information label. This data is only required where inorganic fertilisers are used for which default values of EFf are not provided in the Constant Parameters section below. | Purchased Fertiliser |
| Qh | Total quantity of agrichemical per type purchased |  | kg | Receipts, invoices, or records of quantities purchased. Farm records for quantities used or application rates specific to the reporting period. | Purchased Agrichemicals |
| Ap | Activity data for service purchased, up to the farm-gate |  | Depends on purchased service | Invoices, receipts, farm records. | Purchased services and contractors |
| Qwt | Quantity of waste per type being disposed beyond the farm-gate boundary. |  | tonnes | Weighbridge records, skip disposal invoices. | Management of Waste |
| Vwt | Volume of waste per type being disposed beyond the farm-gate boundary. |  | m3 | Only required if Qwt is not known. Estimated from known volumes of waste transported (e.g., number of skips filled). | Management of Waste |

The checklist below summarises all the optional data points, required for Method 2, that may be collected at farm-level for another livestock system to complete the equations detailed in subsequent chapters based on the other livestock module map.

Table 19: Optional data for other livestock systems

| Parameter | Data to be collected | Data collected Yes/No/N/A | Unit | Data Source | Module |
| --- | --- | --- | --- | --- | --- |
| Optional – Scope 1 |  |  |  |  |  |
| Dj | Average duration of stay on the farm for each other livestock class in the year reported |  | days | Farm stock records; system type records or purchase and sales may be evaluated to determine average duration of each other livestock input class | Enteric Fermentation & Manure Management |
| Nfj | N application rate of inorganic fertiliser per type in production system |  | kg N/ha | Farm records of fertiliser application rates, or calculated. Data on total fertiliser applied and total area should be sourced from farm invoices and spatial records respectively. See Chapter 5, Section 5.1.2.2 for calculation method. | Fertiliser Use – Inorganic Fertiliser Use |
| EFj,N2O | Emission factor for direct nitrous oxide emissions from inorganic fertiliser applied to production system |  | kg N2O-N/kg N | An appropriate value of EFj,N2O should be selected from Table A.1.2.2.3 in the Appendix depending on the production system. See Chapter 5, Section 5.1.2.2 for guidance. | Fertiliser Use – Inorganic Fertiliser Use |
| Yp | Average yield of pasture crop |  | t DM/ha | Where yield, in dry matter, of pasture crops is known, this may be used to replace the default (Method 1) values. Yield records shall be used to justify the use of the Method 2 approach, rather than estimates. | Agriculture Residue Management - Pasture Residues |
| Optional – Scope 2 |  |  |  |  |  |
| RECsurrendered | Eligible Renewable Energy Certificates voluntarily surrendered in the reporting year. |  | MWh | Retailer invoice or Clean Energy Regulator REC registry. | Purchased Electricity & Emissions from Purchased Electricity (market-based approach only) |
| REConsite | Eligible Renewable Energy Certificates that have been or will be issued for electricity produced on-site during the reporting period and consumed by the entity. |  | MWh | Clean Energy Regulator REC registry and Meter readings. | Purchased Electricity & Emissions from Purchased Electricity (market-based approach only) |
| Optional – Scope 3 |  |  |  |  |  |
| Wco | Liveweight of purchased livestock at point of purchase |  | kg/head | Purchasing records/invoices and herd flow models. | Purchased Livestock |
| EFco | Emission factor for purchased livestock per type of livestock |  | kg CO2e/kg | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 15067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. | Purchased Livestock |
| EFj | Emission factor of purchased feed type |  | kg CO2e/kg | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 15067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. | Purchased Feed |
| EFm | Emission factor of purchased mineral supplement type |  | kg CO2e/kg | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 15067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. | Purchased Mineral Supplements |
| EFf | Emission factor of purchased inorganic fertiliser type |  | kg CO2e/kg | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 15067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. | Purchased Fertiliser |
| EFh | Emission factor of purchased agrichemical type |  | kg CO2e/kg | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 15067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. | Purchased Agrichemicals |
| EFl | Emission factor for purchased lime |  | kg CO2e/kg | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 15067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. | Purchased Lime |

## Horticulture

This section outlines all relevant Modules and data checklists to develop a GHG inventory of for a horticulture system. Relevant emission sources and therefore relevant Modules for a specific entity will depend on an entity-specific boundary set as described in Chapter 5 of the CRF. For example, if a specific entity does not use lime, then this emission source and Module will not be relevant for this specific entity.

Module map and input data checklists are designed for estimation of emissions from cradle-to-point of harvesting. If the boundary of the horticulture organisation extends to processing, packing and transport of product the post-production Module can also be implemented to estimate these post-production emissions. Refer to Chapter 1, Section 1.3, for additional information on coverage and limitations of the guidance.

Additional information and guidance on specific parameters can be found within Chapter 5 – Sections; 5.1 – 5.5, Chapter 6 – Sections; 6.1, 6.3 and 6.4, Chapter 8 – Sections; 8.1 and 8.2, Chapter 9 – Sections 9.1, Chapter 10 – Section; 10.1, Chapter 14 – Section; 14.1, and Chapter 15 – Sections; 15.4 – 15.12

### Module Map

| Emission Activity/Source | Relevant Module |
| --- | --- |
| Direct GHG emissions (Scope 1) |  |
| Fertiliser Use | 5.1 – 5.5 |
| Agriculture Residue Management | 6.1, 6.3, 6.4 |
| Transport Fuel | 8.1 |
| Stationary Fuel | 8.2 |
| Solid Waste Treatment | 10.1 |
| Indirect GHG emissions from imported energy (Scope 2) |  |
| Purchased Electricity | 14.1 |
| Indirect GHG emissions (Scope 3) |  |
| Purchased Fertiliser | 15.4 |
| Purchased Agrichemicals | 15.5 |
| Purchased Lime | 15.6 |
| Purchased Services/Contractors | 15.7 |
| Purchased Packaging (including pots) | 15.8 |
| Purchased Grow Media | 15.9 |
| Other Purchased Goods and Services | 15.10 |
| Upstream Emissions from Fuel | 15.11 |
| Upstream Emissions of Purchased Electricity | 15.12 |
| Management of Waste | 15.13 |

### Input Data Checklist

The checklist below summarises all the required input data, required for Method 1, that should be collected at farm-level for a horticulture system to complete the equations detailed in subsequent chapters based on the horticulture module map. Data collected may be listed as ‘not applicable’ for emissions sources that are not relevant to the specific entity. Further information regarding relevant sources and sink can be found in Chapter 5.2 of the CRF.

Additional information and guidance on specific parameters can be found within Chapter 5 – Sections; 5.1 – 5.5, Chapter 6 – Sections; 6.1, 6.3 and 6.4, Chapter 8 – Sections; 8.1 and 8.2, Chapter 10 – Section; 10.1, Chapter 14 – Section; 14.1, and Chapter 15 – Sections; 15.4 – 15.13.

Table 20: Required data for horticulture system

| Parameter | Data to be collected | Data collected Yes/No/N/A | Unit | Data Source | Module |
| --- | --- | --- | --- | --- | --- |
| Required – Scope 1 |  |  |  |  |  |
| TMjf | The total mass of inorganic fertiliser type applied to production system |  | kg | Receipts, invoices, farm records. | Fertiliser Use – Inorganic Fertiliser Use & Purchased Fertiliser |
| FNinorganic,f | Fraction of nitrogen in inorganic fertiliser |  | kg N/kg | Default values for common inorganic fertilisers based on average fertiliser composition from [1], or calculated from product label information. See additional guidance information in Chapter 5, Section 5.1.2.1. | Fertiliser Use – Inorganic Fertiliser Use |
| FUf | Fraction of urea in inorganic fertiliser |  | kg urea/kg | Default values for common inorganic fertilisers based on average fertiliser composition from [1] and [5] or calculated from product label information. See additional guidance information in Chapter 5, Section 5.1.2.1. | Fertiliser Use – Inorganic Fertiliser Use |
| Morganic,jf | Mass of organic fertiliser type (manure) applied to soils in production system |  | kg | Invoices of purchases, farm records of application rates Where the manure was generated on-farm, Mmanure,jf is not required and instead the value of MNSoil shall be calculated using outputs from the MMS emission estimation equations (Chapter 4). | Fertiliser Use – Organic Fertiliser Use |
| FNorganic,f | Nitrogen fraction of the organic fertiliser type (manure) applied to soils |  | kg N/kg | Supplier-provided values or product label information should be used. Default values from IPCC Volume 4 Chapter 5, Table 5.5c [6] (provided in Table A.1.2.2.6 of the Appendix) may be used for manure-based organic fertiliser that was purchased off-farm, where the livestock that generated the organic fertiliser is known. See additional guidance information in Chapter 5, Section 5.2.2.1. | Fertiliser Use – Organic Fertiliser Use |
| Mlime | The mass of lime applied to soils |  | kg | Invoices, receipts. | Fertiliser Use – Lime Application & Purchased Lime |
| FracLime | The fraction of lime as limestone |  | dimensionless | The limestone (calcium carbonate - CaCO3) content of lime may be determined based on the content by weight per cent (wt%) stated on the product information label. Product information label values expressed as wt% shall be converted into a fraction (e.g., for a 60 wt% content of limestone, FracLime = 0.6). | Fertiliser Use – Lime Application |
| FracDol | The fraction of lime as dolomite |  | dimensionless | The dolomite (magnesium carbonate - CaMg(CO3)2) content of lime may be determined based on the content by weight per cent (wt%) on the product information label Product information label values expressed as wt% shall be converted into a fraction (e.g., for a 35 wt% content of dolomite, FracDol = 0.35). | Fertiliser Use – Lime Application |
| Pc | Annual production of crop |  | kg | Farm records. | Agriculture Residue Management – Crop Residues to Soil |
| Fc | Fraction of crop residue that is burnt for crop c |  | dimensionless | Farm records of area burnt and area cultivated. See Chapter 6, Section 6.1.2.1 for guidance on calculation method and sources. | Agriculture Residue Management – Crop Residues to Soil |
| QTrans,q | Quantity and type of fuel used for transportation, (including off-road vehicles and equipment, aircraft, and/or vessels) up to and including harvesting |  | Depends on fuel type | Invoices, fuel cards, receipts. | Transport Fuel & Upstream Emissions |
| QSC,q | Quantity and type of fuel used for stationary equipment, up to and including harvesting |  | Depends on fuel type | Invoices, fuel cards, receipts. | Stationary Fuel & Upstream Emissions |
| Qwt | Quantity of waste per type being treated within the farm-gate. |  | tonnes | Farm records of weight of waste disposed | Solid Waste Treatment |
| Vwt | Volume of waste per type being treated within the farm-gate. |  | m3 | Only required if Qwt is not known. Estimated from trailer-loads, or other known volume of waste transported. | Solid Waste Treatment |
| Required – Scope 2 |  |  |  |  |  |
| Qelec | Quantity of electricity purchased from the grid, up to and including harvesting |  | kWh | Invoices. | Purchased Electricity & Emissions from Purchased Electricity |
| Required – Scope 3 |  |  |  |  |  |
| Fcf | Fraction of component (N, P, K and S) contained within the inorganic fertiliser, by weight |  | wt% | Product information label. This data is only required where inorganic fertilisers are used for which default values of EFf are not provided in the Constant Parameters section below. | Purchased Fertiliser |
| Qh | Total quantity of agrichemical per type purchased |  | kg | Receipts, invoices, or records of quantities purchased. Farm records for quantities used or application rates specific to the reporting period. | Purchased Agrichemicals |
| Ap | Activity data for service purchased, up to and including harvesting |  | Depends on purchased service | Invoices, receipts, farm records. | Purchased services and contractors |
| Qp | Units of purchased packaging |  | units | Invoices, or other records | Purchased Packaging |
| Qg | Mass of purchased grow media |  | kg | Invoices, or other records | Purchased Grow Media |
| Qwt | Quantity of waste per type being disposed beyond the farm-gate boundary. |  | tonnes | Weighbridge records, skip disposal invoices. | Management of Waste |
| Vwt | Volume of waste per type being disposed beyond the farm-gate boundary. |  | m3 | Only required if Qwt is not known. Estimated from known volumes of waste transported (e.g., number of skips filled). | Management of Waste |

The checklist below summarises all the optional data points, required for Method 2, that may be collected at farm-level for a horticulture system to complete the equations detailed in subsequent chapters based on the horticulture module map.

Table 21: Optional data for horticulture system

| Parameter | Data to be collected | Data collected Yes/No/N/A | Unit | Data Source | Module |
| --- | --- | --- | --- | --- | --- |
| Optional – Scope 1 |  |  |  |  |  |
| Nfj | N application rate of inorganic fertiliser per type in production system |  | kg N/ha | Farm records of fertiliser application rates, or calculated. Data on total fertiliser applied and total area should be sourced from farm invoices and spatial records respectively. See Chapter 5, Section 5.1.2.2 for calculation method. | Fertiliser Use – Inorganic Fertiliser Use |
| EFj,N2O | Emission factor for direct nitrous oxide emissions from inorganic fertiliser applied to production system |  | kg N2O-N/kg N | An appropriate value of EFj,N2O should be selected from Table A.1.2.2.3 in the Appendix depending on the production system. See Chapter 5, Section 5.1.2.2 for guidance. | Fertiliser Use – Inorganic Fertiliser Use |
| FFODc | Fraction of the crop residue that is removed per crop type |  | dimensionless | See Chapter 6 equation 6.1.1.2 (1) for more guidance. Where residues are stubble-grazed, the default value for FFODc should be used. | Agriculture Residue Management – Crop Residues to Soil |
| B | Number of bales generated |  | dimensionless | Invoices for the number of bales sold off-farm, or other farm records documenting the number of bales made. | Agriculture Residue Management – Crop Residues to Soil |
| MB | Average mass of each bale generated |  | kg | Invoices from bale sales. | Agriculture Residue Management – Crop Residues to Soil |
| Optional – Scope 2 |  |  |  |  |  |
| RECsurrendered | Eligible Renewable Energy Certificates voluntarily surrendered in the reporting year. |  | MWh | Retailer invoice or Clean Energy Regulator REC registry. | Purchased Electricity & Emissions from Purchased Electricity (market-based approach only) |
| REConsite | Eligible Renewable Energy Certificates that have been or will be issued for electricity produced on-site during the reporting period and consumed by the entity. |  | MWh | Clean Energy Regulator REC registry and Meter readings. | Purchased Electricity & Emissions from Purchased Electricity (market-based approach only) |
| Optional – Scope 3 |  |  |  |  |  |
| EFf | Emission factor of purchased inorganic fertiliser type |  | kg CO2e/kg | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 15067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. | Purchased Fertiliser |
| EFh | Emission factor of purchased agrichemical type |  | kg CO2e/kg | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 15067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. | Purchased Agrichemicals |
| EFl | Emission factor for purchased lime |  | kg CO2e/kg | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 15067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. | Purchased Lime |
| EFp | Emission factor for purchased packaging |  | kg CO2e/unit | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 15067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. | Purchased Packaging |
| EFg | Emission factor for purchased grow media |  | kg CO2e/kg | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 15067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. | Purchased Grow Media |

## Viticulture

This section outlines all relevant Modules and data checklists to develop a GHG inventory of for a viticulture system. Relevant emission sources and therefore relevant Modules for a specific entity will depend on an entity-specific boundary set as described in Chapter 5 of the CRF. For example, if a specific entity does not use lime, then this emission source and Module will not be relevant for this specific entity.

Module map and input data checklists are designed for estimation of emissions from cradle-to-point of harvesting. If the boundary of the organisation extends to processing, packing and transport of grapes, the post-production Module can also be implemented to estimate emissions from these sources. Refer to Chapter 1, Section 1.3, for additional information on coverage and limitations of the guidance.

### Module Map

| Emission Activity/Source | Relevant Module |
| --- | --- |
| Direct GHG emissions (Scope 1) |  |
| Fertiliser Use | 5.1 – 5.5 |
| Agriculture Residue Management | 6.1, 6.3, 6.4 |
| Transport Fuel | 8.1 |
| Stationary Fuel | 8.2 |
| Solid Waste Treatment | 10.1 |
| Indirect GHG emissions from imported energy (Scope 2) |  |
| Purchased Electricity | 14.1 |
| Indirect GHG emissions (Scope 3) |  |
| Purchased Fertiliser | 15.4 |
| Purchased Agrichemicals | 15.5 |
| Purchased Lime | 15.6 |
| Purchased Services/Contractors | 15.7 |
| Other Purchased Goods and Services | 15.10 |
| Upstream Emissions from Fuel | 15.11 |
| Upstream Emissions of Purchased Electricity | 15.12 |
| Management of Waste | 15.13 |

### Input Data Checklist

The checklist below summarises all the required input data, required for Method 1, that should be collected at farm-level for a viticulture system to complete the equations detailed in subsequent chapters based on the viticulture module map. Data collected may be listed as ‘not applicable’ for emissions sources that are not relevant to the specific entity. Further information regarding relevant sources and sink can be found in Chapter 5.2 of the CRF.

Additional information and guidance on specific parameters can be found within Chapter 5 – Sections; 5.1 – 5.5, Chapter 6 – Sections; 6.1, 6.3 and 6.4, Chapter 8 – Sections; 8.1 and 8.2, Chapter 9 – Sections 9.1, Chapter 10 – Section; 10.1, Chapter 14 – Section; 14.1, and Chapter 15 – Sections; 15.4 – 15.7 and 15.10 – 15.12.

Table 22: Required data for viticulture system

| Parameter | Data to be collected | Data collected Yes/No/N/A | Unit | Data Source | Module |
| --- | --- | --- | --- | --- | --- |
| Required – Scope 1 |  |  |  |  |  |
| TMjf | The total mass of inorganic fertiliser type applied to production system |  | kg | Receipts, invoices, farm records. | Fertiliser Use – Inorganic Fertiliser Use & Purchased Fertiliser |
| FNinorganic,f | Fraction of nitrogen in inorganic fertiliser |  | kg N/kg | Default values for common inorganic fertilisers based on average fertiliser composition from [1], or calculated from product label information. See additional guidance information in Chapter 5, Section 5.1.2.1. | Fertiliser Use – Inorganic Fertiliser Use |
| FUf | Fraction of urea in inorganic fertiliser |  | kg urea/kg | Default values for common inorganic fertilisers based on average fertiliser composition from [1] and [5] or calculated from product label information. See additional guidance information in Chapter 5, Section 5.1.2.1. | Fertiliser Use – Inorganic Fertiliser Use |
| Morganic,jf | Mass of organic fertiliser type (manure) applied to soils in production system |  | kg | Invoices of purchases, farm records of application rates Where the manure was generated on-farm, Mmanure,jf is not required and instead the value of MNSoil shall be calculated using outputs from the MMS emission estimation equations (Chapter 4). | Fertiliser Use – Organic Fertiliser Use |
| FNorganic,f | Nitrogen fraction of the organic fertiliser type (manure) applied to soils |  | kg N/kg | Supplier-provided values or product label information should be used. Default values from IPCC Volume 4 Chapter 5, Table 5.5c [6] (provided in Table A.1.2.2.6 of the Appendix) may be used for manure-based organic fertiliser that was purchased off-farm, where the livestock that generated the organic fertiliser is known. See additional guidance information in Chapter 5, Section 5.2.2.1. | Fertiliser Use – Organic Fertiliser Use |
| Mlime | The mass of lime applied to soils |  | kg | Invoices, receipts. | Fertiliser Use – Lime Application & Purchased Lime |
| FracLime | The fraction of lime as limestone |  | dimensionless | The limestone (calcium carbonate - CaCO3) content of lime may be determined based on the content by weight per cent (wt%) stated on the product information label. Product information label values expressed as wt% shall be converted into a fraction (e.g., for a 60 wt% content of limestone, FracLime = 0.6). | Fertiliser Use – Lime Application |
| FracDol | The fraction of lime as dolomite |  | dimensionless | The dolomite (magnesium carbonate - CaMg(CO3)2) content of lime may be determined based on the content by weight per cent (wt%) on the product information label Product information label values expressed as wt% shall be converted into a fraction (e.g., for a 35 wt% content of dolomite, FracDol = 0.35). | Fertiliser Use – Lime Application |
| Pc | Annual production of crop |  | kg | Farm records. | Agriculture Residue Management – Crop Residues to Soil |
| Fc | Fraction of crop residue that is burnt for crop c |  | dimensionless | Farm records of area burnt and area cultivated. See Chapter 6, Section 6.1.2.1 for guidance on calculation method and sources. | Agriculture Residue Management – Crop Residues to Soil |
| QTrans,q | Quantity and type of fuel used for transportation, (including off-road vehicles and equipment, aircraft, and/or vessels) up to and including harvesting |  | Depends on fuel type | Invoices, fuel cards, receipts. | Transport Fuel & Upstream Emissions |
| QSC,q | Quantity and type of fuel used for stationary equipment, up to and including harvesting |  | Depends on fuel type | Invoices, fuel cards, receipts. | Stationary Fuel & Upstream Emissions |
| Qwt | Quantity of waste per type being treated within the farm-gate. |  | tonnes | Farm records of weight of waste disposed | Solid Waste Treatment |
| Vwt | Volume of waste per type being treated within the farm-gate. |  | m3 | Only required if Qwt is not known. Estimated from trailer-loads, or other known volume of waste transported. | Solid Waste Treatment |
| Required – Scope 2 |  |  |  |  |  |
| Qelec | Quantity of electricity purchased from the grid, up to and including harvesting |  | kWh | Invoices. | Purchased Electricity & Emissions from Purchased Electricity |
| Required – Scope 3 |  |  |  |  |  |
| Fcf | Fraction of component (N, P, K and S) contained within the inorganic fertiliser, by weight |  | wt% | Product information label. This data is only required where inorganic fertilisers are used for which default values of EFf are not provided in the Constant Parameters section below. | Purchased Fertiliser |
| Qh | Total quantity of agrichemical per type purchased |  | kg | Receipts, invoices, or records of quantities purchased. Farm records for quantities used or application rates specific to the reporting period. | Purchased Agrichemicals |
| Ap | Activity data for service purchased, up to and including harvesting |  | Depends on purchased service | Invoices, receipts, farm records. | Purchased services and contractors |
| Qwt | Quantity of waste per type being disposed beyond the farm-gate boundary. |  | tonnes | Weighbridge records, skip disposal invoices. | Management of Waste |
| Vwt | Volume of waste per type being disposed beyond the farm-gate boundary. |  | m3 | Only required if Qwt is not known. Estimated from known volumes of waste transported (e.g., number of skips filled). | Management of Waste |

The checklist below summarises all the optional data points, required for Method 2, that may be collected at farm-level for a viticulture system to complete the equations detailed in subsequent chapters based on the viticulture module map.

Table 23: Optional data for viticulture system

| Parameter | Data to be collected | Data collected Yes/No/N/A | Unit | Data Source | Module |
| --- | --- | --- | --- | --- | --- |
| Optional – Scope 1 |  |  |  |  |  |
| Nfj | N application rate of inorganic fertiliser per type in production system |  | kg N/ha | Farm records of fertiliser application rates, or calculated. Data on total fertiliser applied and total area should be sourced from farm invoices and spatial records respectively. See Chapter 5, Section 5.1.2.2 for calculation method. | Fertiliser Use – Inorganic Fertiliser Use |
| EFj,N2O | Emission factor for direct nitrous oxide emissions from inorganic fertiliser applied to production system |  | kg N2O-N/kg N | An appropriate value of EFj,N2O should be selected from Table A.1.2.2.3 in the Appendix depending on the production system. See Chapter 5, Section 5.1.2.2 for guidance. | Fertiliser Use – Inorganic Fertiliser Use |
| FFODc | Fraction of the crop residue that is removed per crop type |  | dimensionless | See Chapter 6 equation 6.1.1.2 (1). Where residues are stubble-grazed, the default value for FFODc should be used. | Agriculture Residue Management – Crop Residues to Soil |
| B | Number of bales generated |  | dimensionless | Invoices for the number of bales sold off-farm, or other farm records documenting the number of bales made. | Agriculture Residue Management – Crop Residues to Soil |
| MB | Average mass of each bale generated |  | kg | Invoices from bale sales. | Agriculture Residue Management – Crop Residues to Soil |
| Optional – Scope 2 |  |  |  |  |  |
| RECsurrendered | Eligible Renewable Energy Certificates voluntarily surrendered in the reporting year. |  | MWh | Retailer invoice or Clean Energy Regulator REC registry. | Purchased Electricity & Emissions from Purchased Electricity (market-based approach only) |
| REConsite | Eligible Renewable Energy Certificates that have been or will be issued for electricity produced on-site during the reporting period and consumed by the entity. |  | MWh | Clean Energy Regulator REC registry and Meter readings. | Purchased Electricity & Emissions from Purchased Electricity (market-based approach only) |
| Optional – Scope 3 |  |  |  |  |  |
| EFf | Emission factor of purchased inorganic fertiliser type |  | kg CO2e/kg | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 15067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. | Purchased Fertiliser |
| EFh | Emission factor of purchased agrichemical type |  | kg CO2e/kg | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 15067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. | Purchased Agrichemicals |
| EFl | Emission factor for purchased lime |  | kg CO2e/kg | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 15067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. | Purchased Lime |

## Aquaculture and Wildcatch Fisheries

This section outlines all relevant Modules and data checklists to develop a GHG inventory for aquaculture and wild fisheries. Relevant emission sources and therefore relevant Modules for a specific entity will depend on an entity-specific boundary set as described in Chapter 5 of the CRF. For example, if a specific entity does not use lime, then this emission source and Module will not be relevant for this specific entity.

Question Reference 2.1.

The current module map reflects that globally some aquaculture systems may use fertiliser or lime to increase available nutrients or control pH. Is this also the case in Australian aquaculture systems? Would inorganic or organic fertilisers be used or are these sources not applicable to Australian aquaculture?

Module map and input data checklists are designed for estimation of emissions from cradle-to-point of landing (from ponds/pens/sea as applicable). If the boundary of the organisation extends to processing, packing and transport of seafood post landing, the post-production Module can also be implemented to estimate emissions from these sources. Refer to Chapter 1, Section 1.3, for additional information on coverage and limitations of the guidance.

### Module Map

| Emission Activity/Source | Relevant Module |
| --- | --- |
| Direct GHG emissions (Scope 1) |  |
| Fertiliser Use | 5.1 – 5.5 |
| Transport Fuel | 8.1 |
| Stationary Fuel | 8.2 |
| Refrigerant Use | 9.1 |
| Solid Waste Treatment | 10.1 |
| Wastewater Treatment | 11.1 |
| N2O Emissions from Aquaculture | 13.1 |
| Indirect GHG emissions from imported energy (Scope 2) |  |
| Purchased Electricity | 14.1 |
| Indirect GHG emissions (Scope 3) |  |
| Purchased Feed | 15.2 |
| Purchased Fertiliser | 15.4 |
| Purchased Lime | 15.6 |
| Other Purchased Goods and Services | 15.10 |
| Upstream Emissions from Fuel | 15.11 |
| Upstream emissions of Purchased Electricity | 15.12 |
| Management of Waste | 15.13 |

### Input Data Checklist

The checklist below summarises all the required input data, required for Method 1, that should be collected at entity-level for an aquaculture and/or wild fisheries system to complete the equations detailed in subsequent chapters based on the aquaculture and wild fisheries module map. Data collected may be listed as ‘not applicable’ for emissions sources that are not relevant to the specific entity. Further information regarding relevant sources and sink can be found in Chapter 5.2 of the CRF.

Additional information and guidance on aquaculture and wild fisheries parameters can be found within Chapter 5 – Sections; 5.1 – 5.5, Chapter 6 – Sections; 6.1 – 6.3, Chapter 10 – Section; 10.1, Chapter 11 – Section; 11.1, Chapter 13 – Section; 13.1, Chapter 14 – Section; 14.1, and Chapter 15 – Sections; 15.2, 15.4, 15.6 15.10 – 15.12

Table 24: Required data for aquaculture and/or wild fisheries system

| Parameter | Data to be collected | Data collected Yes/No/N/A | Unit | Data Source | Module |
| --- | --- | --- | --- | --- | --- |
| Required – Scope 1 |  |  |  |  |  |
| TMjf | The total mass of inorganic fertiliser type applied to production system |  | kg | Receipts, invoices, entity records. | Fertiliser Use – Inorganic Fertiliser Use & Purchased Fertiliser |
| FNinorganic,f | Fraction of nitrogen in inorganic fertiliser |  | kg N/kg | Default values for common inorganic fertilisers based on average fertiliser composition from [1], or calculated from product label information. See additional guidance information in Chapter 5, Section 5.1.2.1. | Fertiliser Use – Inorganic Fertiliser Use |
| FUf | Fraction of urea in inorganic fertiliser |  | kg urea/kg | Default values for common inorganic fertilisers based on average fertiliser composition from [1] and [5] or calculated from product label information. See additional guidance information in Chapter 5, Section 5.1.2.1. | Fertiliser Use – Inorganic Fertiliser Use |
| Morganic,jf | Mass of organic fertiliser type (manure) applied to soils in production system |  | kg | Invoices of purchases, entity records of application rates Where the manure was generated by the entity, Mmanure,jf is not required and instead the value of MNSoil shall be calculated using outputs from the MMS emission estimation equations (Chapter 4). | Fertiliser Use – Organic Fertiliser Use |
| FNorganic,f | Nitrogen fraction of the organic fertiliser type (manure) applied to soils |  | kg N/kg | Supplier-provided values or product label information should be used. Default values from IPCC Volume 4 Chapter 5, Table 5.5c [6] (provided in Table A.1.2.2.6 of the Appendix) may be used for manure-based organic fertiliser that was purchased outside the entity, where the livestock that generated the organic fertiliser is known. See additional guidance information in Chapter 5, Section 5.2.2.1. | Fertiliser Use – Organic Fertiliser Use |
| Mlime | The mass of lime applied to soils |  | kg | Invoices, receipts. | Fertiliser Use – Lime Application & Purchased Lime |
| FracLime | The fraction of lime as limestone |  | dimensionless | The limestone (calcium carbonate - CaCO3) content of lime may be determined based on the content by weight per cent (wt%) stated on the product information label. Product information label values expressed as wt% shall be converted into a fraction (e.g., for a 60 wt% content of limestone, FracLime = 0.6). | Fertiliser Use – Lime Application |
| FracDol | The fraction of lime as dolomite |  | dimensionless | The dolomite (magnesium carbonate - CaMg(CO3)2) content of lime may be determined based on the content by weight per cent (wt%) on the product information label Product information label values expressed as wt% shall be converted into a fraction (e.g., for a 35 wt% content of dolomite, FracDol = 0.35). | Fertiliser Use – Lime Application |
| QTrans,q | Quantity and type of fuel used for transportation, (including off-road vehicles and equipment, aircraft, and/or vessels) up to and including point of landing |  | Depends on fuel type | Invoices, fuel cards, receipts. | Transport Fuel & Upstream Emissions |
| QSC,q | Quantity and type of fuel used for stationary equipment, up to and including point of landing |  | Depends on fuel type | Invoices, fuel cards, receipts. | Stationary Fuel & Upstream Emissions |
| chargeRa | The amount of refrigerant gas R contained in appliance, up to and including point of landing |  | kg | Manufacturer’s product specification should contain information of the charge of the refrigerant contained in each appliance. | Refrigerant Use |
| Qwt | Quantity of waste per type being treated within the farm-gate. |  | tonnes | Entity records of weight of waste disposed | Solid Waste Treatment |
| Vwt | Volume of waste per type being treated within the farm-gate. |  | m3 | Only required if Qwt is not known. Estimated from trailer-loads, or other known volume of waste transported. | Solid Waste Treatment |
| Qt | Quantity of wastewater treated at each facility type, up to and including point of landing |  | m3 | Direct measurement of daily or hourly flows scaled to give an annual average | Wastewater Treatment |
| CODin | Average inlet COD concentration of wastewater |  | kg COD/m3 | Direct measurement | Wastewater Treatment |
| CODout | average outlet COD concentration of wastewater |  | kg COD/m3 | Direct measurement | Wastewater Treatment |
| Fsludge | Fraction of COD removed from the wastewater treatment facility as sludge |  | dimensionless | Measurement records, averaged over the reporting period | Wastewater Treatment |
| Fremoved | Fraction of COD removed from the wastewater treatment facility and transferred to landfill, biochar production, or other site |  | dimensionless | Measurement records, averaged over the reporting period | Wastewater Treatment |
| Qcap | Quantity of methane in sludge biogas captured for combustion for the operation of the treatment facility |  | m3 CH4 | Direct measurement of methane content of sludge biogas, multiplied by total volume of biogas combusted by the facility | Wastewater Treatment |
| Qflared | Quantity of methane in sludge biogas flared by the treatment facility |  | m3 CH4 | Direct measurement of methane content of sludge biogas, multiplied by total volume of biogas combusted by the facility | Wastewater Treatment |
| Qout | Quantity of methane in sludge biogas transferred out of the treatment facility |  | m3 CH4 | Direct measurement of methane content of sludge biogas, multiplied by total volume of biogas combusted by the facility | Wastewater Treatment |
| Q | Quantity of fish produced in aquaculture ponds in in the reporting period |  | tonnes | Production records, sales records | N2O Emissions from Aquaculture in Wetlands |
| Required – Scope 2 |  |  |  |  |  |
| Qelec | Quantity of electricity purchased from the grid, up to and including point of landing |  | kWh | Invoices. | Purchased Electricity & Emissions from Purchased Electricity |
| Required – Scope 3 |  |  |  |  |  |
| Qj | Quantity of feed type purchased |  | t | Invoices, receipts or other records of purchased quantities. | Purchased Feed |
| Fcf | Fraction of component (N, P, K and S) contained within the inorganic fertiliser, by weight |  | wt% | Product information label. This data is only required where inorganic fertilisers are used for which default values of EFf are not provided in the Constant Parameters section below. | Purchased Fertiliser |
| Qwt | Quantity of waste per type being disposed beyond the farm-gate boundary. |  | tonnes | Weighbridge records, skip disposal invoices. | Management of Waste |
| Vwt | Volume of waste per type being disposed beyond the farm-gate boundary. |  | m3 | Only required if Qwt is not known. Estimated from known volumes of waste transported (e.g., number of skips filled). | Management of Waste |

The checklist below summarises all the optional data points, required for Method 2, that may be collected at entity-level for an aquaculture and/or wild-fisheries system to complete the equations detailed in subsequent chapters based on the aquaculture and wild-fisheries module map.

Table 25: Optional data for aquaculture and/or wild fisheries system

| Parameter | Data to be collected | Data collected Yes/No/N/A | Unit | Data Source | Module |
| --- | --- | --- | --- | --- | --- |
| Optional – Scope 1 |  |  |  |  |  |
| EFj,N2O | Emission factor for direct nitrous oxide emissions from inorganic fertiliser applied to production system |  | kg N2O-N/kg N | An appropriate value of EFj,N2O should be selected from Table A.1.2.2.3 in the Appendix depending on the production system. See Chapter 5, Section 5.1.2.2 for guidance. | Fertiliser Use – Inorganic Fertiliser Use |
| Optional – Scope 2 |  |  |  |  |  |
| RECsurrendered | Eligible Renewable Energy Certificates voluntarily surrendered in the reporting year. |  | MWh | Retailer invoice or Clean Energy Regulator REC registry. | Purchased Electricity & Emissions from Purchased Electricity (market-based approach only) |
| REConsite | Eligible Renewable Energy Certificates that have been or will be issued for electricity produced on-site during the reporting period and consumed by the entity. |  | MWh | Clean Energy Regulator REC registry and Meter readings. | Purchased Electricity & Emissions from Purchased Electricity (market-based approach only) |
| Optional – Scope 3 |  |  |  |  |  |
| EFj | Emission factor of purchased feed type |  | kg CO2e/kg | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 15067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. | Purchased Feed |
| EFf | Emission factor of purchased inorganic fertiliser type |  | kg CO2e/kg | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 15067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. | Purchased Fertiliser |
| EFl | Emission factor for purchased lime |  | kg CO2e/kg | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 15067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. | Purchased Lime |

## Forestry

This section outlines all relevant Modules and data checklists to develop a GHG inventory for forestry. Relevant emission sources and therefore relevant Modules for a specific entity will depend on an entity-specific boundary set as described in Chapter 5 of the CRF. For example, if a specific entity does not use lime, then this emission source and Module will not be relevant for this specific entity.

Module map and input data checklists are designed for estimation of emissions from cradle-to-point of delivery at the mill. If the boundary of the forestry organisation extends to processing of product the post-production Module can also be implemented to estimate these post-production emissions. Refer to Chapter 1, Section 1.3, for additional information on coverage and limitations of the guidance.

### Module Map

| Emission Activity/Source | Relevant Module |
| --- | --- |
| Direct GHG emissions (Scope 1) |  |
| Fertiliser Use | 5.1 – 5.2 |
| Transport Fuel | 8.1 |
| Stationary Fuel | 8.2 |
| Solid Waste Treatment | 10.1 |
| Indirect GHG emissions from imported energy (Scope 2) |  |
| Purchased Electricity | 14.1 |
| Indirect GHG emissions (Scope 3) |  |
| Purchased Fertiliser | 15.4 |
| Purchased Agrichemicals | 15.5 |
| Purchased Services/Contractors | 15.7 |
| Other Purchased Goods and Services | 15.10 |
| Upstream Emissions from Fuel | 15.11 |
| Upstream Emissions of Purchased Electricity | 15.12 |
| Management of Waste | 15.13 |

### Input Data Checklist

The checklist below summarises all the required input data, required for Method 1, that should be collected at entity-level for a forestry system to complete the equations detailed in subsequent chapters based on the forestry module map. Data collected may be listed as ‘not applicable’ for emissions sources that are not relevant to the specific entity. Further information regarding relevant sources and sink can be found in Chapter 5.2 of the CRF.

Additional information and guidance on specific parameters can be found within Chapter 5 – Sections; 5.1 – 5.2, Chapter 8 – Sections; 8.1 and 8.2, Chapter 10 – Section; 10.1, Chapter 14 – Section; 14.1, and Chapter 15 – Sections; 15.4, 15.5, 15.7, 15.10 – 15.12

Table 26: Required data for forestry system

| Parameter | Data to be collected | Data collected Yes/No/N/A | Unit | Data Source | Module |
| --- | --- | --- | --- | --- | --- |
| Required – Scope 1 |  |  |  |  |  |
| TMjf | The total mass of inorganic fertiliser type applied to production system |  | kg | Receipts, invoices, entity records. | Fertiliser Use – Inorganic Fertiliser Use & Purchased Fertiliser |
| FNinorganic,f | Fraction of nitrogen in inorganic fertiliser |  | kg N/kg | Default values for common inorganic fertilisers based on average fertiliser composition from [1], or calculated from product label information. See additional guidance information in Chapter 5, Section 5.1.2.1. | Fertiliser Use – Inorganic Fertiliser Use |
| FUf | Fraction of urea in inorganic fertiliser |  | kg urea/kg | Default values for common inorganic fertilisers based on average fertiliser composition from [1] and [5] or calculated from product label information. See additional guidance information in Chapter 5, Section 5.1.2.1. | Fertiliser Use – Inorganic Fertiliser Use |
| Morganic,jf | Mass of organic fertiliser type (manure) applied to soils in production system |  | kg | Invoices of purchases, entity records of application rates Where the manure was generated by the entity, Mmanure,jf is not required and instead the value of MNSoil shall be calculated using outputs from the MMS emission estimation equations (Chapter 4). | Fertiliser Use – Organic Fertiliser Use |
| FNorganic,f | Nitrogen fraction of the organic fertiliser type (manure) applied to soils |  | kg N/kg | Supplier-provided values or product label information should be used. Default values from IPCC Volume 4 Chapter 5, Table 5.5c [6] (provided in Table A.1.2.2.6 of the Appendix) may be used for manure-based organic fertiliser that was purchased outside the entity, where the livestock that generated the organic fertiliser is known. See additional guidance information in Chapter 5, Section 5.2.2.1. | Fertiliser Use – Organic Fertiliser Use |
| QTrans,q | Quantity and type of fuel used for transportation, (including off-road vehicles and equipment, aircraft, and/or vessels) up to and including harvesting |  | Depends on fuel type | Invoices, fuel cards, receipts. | Transport Fuel & Upstream Emissions |
| QSC,q | Quantity and type of fuel used for stationary equipment, up to and including harvesting |  | Depends on fuel type | Invoices, fuel cards, receipts. | Stationary Fuel & Upstream Emissions |
| Qwt | Quantity of waste per type being treated within the farm-gate. |  | tonnes | Entity records of weight of waste disposed | Solid Waste Treatment |
| Vwt | Volume of waste per type being treated within the farm-gate. |  | m3 | Only required if Qwt is not known. Estimated from trailer-loads, or other known volume of waste transported. | Solid Waste Treatment |
| Required – Scope 2 |  |  |  |  |  |
| Qelec | Quantity of electricity purchased from the grid, up to and including harvesting |  | kWh | Invoices. | Purchased Electricity & Emissions from Purchased Electricity |
| Required – Scope 3 |  |  |  |  |  |
| Qh | Total quantity of agrichemical per type purchased |  | kg | Receipts, invoices, or records of quantities purchased. Entity records for quantities used or application rates specific to the reporting period. | Purchased Agrichemicals |
| Ap | Activity data for service purchased, up to and including harvesting |  | Depends on purchased service | Invoices, receipts, entity records. | Purchased services and contractors |
| Qwt | Quantity of waste per type being disposed beyond the farm-gate boundary. |  | tonnes | Weighbridge records, skip disposal invoices. | Management of Waste |
| Vwt | Volume of waste per type being disposed beyond the farm-gate boundary. |  | m3 | Only required if Qwt is not known. Estimated from known volumes of waste transported (e.g., number of skips filled). | Management of Waste |

The checklist below summarises all the optional data points, required for Method 2, that may be collected at entity-level for a forestry system to complete the equations detailed in subsequent chapters based on the forestry module map.

Table 27: Optional data for forestry system

| Parameter | Data to be collected | Data collected Yes/No/N/A | Unit | Data Source | Module |
| --- | --- | --- | --- | --- | --- |
| Optional – Scope 1 |  |  |  |  |  |
| Nfj | N application rate of inorganic fertiliser per type in production system |  | kg N/ha | Entity records of fertiliser application rates or calculated. Data on total fertiliser applied and total area should be sourced from entity invoices and spatial records respectively. See Chapter 5, Section 5.1.2.2 for calculation method. | Fertiliser Use – Inorganic Fertiliser Use |
| EFj,N2O | Emission factor for direct nitrous oxide emissions from inorganic fertiliser applied to production system |  | kg N2O-N/kg N | An appropriate value of EFj,N2O should be selected from Table A.1.2.2.3 in the Appendix depending on the production system. See Chapter 5, Section 5.1.2.2 for guidance. | Fertiliser Use – Inorganic Fertiliser Use |
| Optional – Scope 2 |  |  |  |  |  |
| RECsurrendered | Eligible Renewable Energy Certificates voluntarily surrendered in the reporting year. |  | MWh | Retailer invoice or Clean Energy Regulator REC registry. | Purchased Electricity & Emissions from Purchased Electricity (market-based approach only) |
| REConsite | Eligible Renewable Energy Certificates that have been or will be issued for electricity produced on-site during the reporting period and consumed by the entity. |  | MWh | Clean Energy Regulator REC registry and Meter readings. | Purchased Electricity & Emissions from Purchased Electricity (market-based approach only) |
| Optional – Scope 3 |  |  |  |  |  |
| EFf | Emission factor of purchased inorganic fertiliser type |  | kg CO2e/kg | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 15067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. | Purchased Fertiliser |
| EFh | Emission factor of purchased agrichemical type |  | kg CO2e/kg | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 15067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. | Purchased Agrichemicals |

## Post-production Module

This section outlines Modules and data checklists relevant to the development of a GHG inventory for post-processing. The Module map and data checklists are focused on estimation of Scope 1 and Scope 2 emissions from post-production activities and the associated upstream Scope 3 emissions. Individual users need to determine which, if any, post-processing activities fall within their Scope 1 or 2 or upstream Scope 3 emissions. Inclusion and scope categorisation of post-production activities will depend on control, ownership, boundary and materiality of emission sources. Refer to CRF, Chapter 5 for additional information boundary setting and materiality.

While some of the upstream Scope 3 modules, such as freight (5.14) and packaging (5.8), may also be relevant for estimating downstream Scope 3 emission sources this guidance does not cover to the full scope of downstream post-production emissions. Users will be required to set their own boundary for Scope 3 downstream emissions, collect the appropriate activity data, and identify associated emissions factors for the full boundary of their downstream Scope 3 emissions.

### Module Map

| Emission Activity/Source | Relevant Module |
| --- | --- |
| Direct GHG emissions (Scope 1) |  |
| Transport Fuel | 8.1 |
| Stationary Fuel | 8.2 |
| Refrigerant | 9.1 |
| Solid Waste Treatment | 10.1 |
| Wastewater Treatment | 11.1 |
| Supplementary CO2 | 12.1 |
| Indirect GHG emissions from imported energy (Scope 2) |  |
| Purchased Electricity | 14.1 |
| Indirect GHG emissions (Scope 3) |  |
| Purchased Packaging | 15.8 |
| Other Purchased Goods and Services | 15.10 |
| Upstream Emissions from Fuel | 15.11 |
| Upstream Emissions of Purchased Electricity | 15.12 |
| Management of Waste | 15.13 |
| Upstream Freight | 15.14 |

### Input Data Checklist

The checklist below summarises all the required input data, required for Method 1, that should be collected at entity-level for a post-production system to complete the equations detailed in subsequent chapters based on the post-production module map. Data collected may be listed as ‘not applicable’ for emissions sources that are not relevant to the specific entity. Further information regarding relevant sources and sink can be found in Chapter 5.2 of the CRF.

Additional information and guidance on specific parameters can be found within Chapter 8 – Sections; 8.1 and 8.2, Chapter 9 – Sections 9.1, Chapter 10 – Section; 10.1, Chapter 11 – Section; 11.1, Chapter 12 – Section; 12.1, Chapter 14 – Section; 14.1, and Chapter 15 – Sections; 15.8, 15.10 – 15.14

Table 28: Potentially required data for post-production

| Parameter | Data to be collected | Data collected Yes/No/N/A | Unit | Data Source | Module |
| --- | --- | --- | --- | --- | --- |
| Required – Scope 1 |  |  |  |  |  |
| QTrans,q | Quantity and type of fuel used for transportation, occurring post-production |  | Depends on fuel type | Invoices, fuel cards, receipts. | Transport Fuel & Upstream Emissions |
| QSC,q | Quantity and type of fuel used for stationary equipment, occurring post-production |  | Depends on fuel type | Invoices, fuel cards, receipts. | Stationary Fuel & Upstream Emissions |
| chargeRa | The amount of refrigerant gas R contained in appliance, occurring post-production |  | kg | Manufacturer’s product specification should contain information of the charge of the refrigerant contained in each appliance. | Refrigerant Use |
| Qwt | Quantity of waste per type being treated within the post-production boundary. |  | tonnes | Entity records of weight of waste disposed | Solid Waste Treatment |
| Vwt | Volume of waste per type being treated within the post-production boundary. |  | m3 | Only required if Qwt is not known. Estimated from trailer-loads, or other known volume of waste transported. | Solid Waste Treatment |
| Qt | Quantity of wastewater treated at each facility type, occurring post-production |  | m3 | Direct measurement of daily or hourly flows scaled to give an annual average | Wastewater Treatment |
| CODin | Average inlet COD concentration of wastewater |  | kg COD/m3 | Direct measurement | Wastewater Treatment |
| CODout | Average outlet COD concentration of wastewater |  | kg COD/m3 | Direct measurement | Wastewater Treatment |
| Fsludge | Fraction of COD removed from the wastewater treatment facility as sludge |  | dimensionless | Measurement records, averaged over the reporting period | Wastewater Treatment |
| Fremoved | Fraction of COD removed from the wastewater treatment facility and transferred to landfill, biochar production, or other site |  | dimensionless | Measurement records, averaged over the reporting period | Wastewater Treatment |
| Qcap | Quantity of methane in sludge biogas captured for combustion for the operation of the treatment facility |  | m3 CH4 | Direct measurement of methane content of sludge biogas, multiplied by total volume of biogas combusted by the facility | Wastewater Treatment |
| Qflared | Quantity of methane in sludge biogas flared by the treatment facility |  | m3 CH4 | Direct measurement of methane content of sludge biogas, multiplied by total volume of biogas combusted by the facility | Wastewater Treatment |
| Qout | Quantity of methane in sludge biogas transferred out of the treatment facility |  | m3 CH4 | Direct measurement of methane content of sludge biogas, multiplied by total volume of biogas combusted by the facility | Wastewater Treatment |
| Q | Quantity of supplementary CO2 used in post-production |  | kg CO2 | Purchase records. | Supplementary CO2 |
| Required – Scope 2 |  |  |  |  |  |
| Qelec | Quantity of electricity purchased from the grid for post-production activities |  | kWh | Invoices. | Purchased Electricity & Emissions from Purchased Electricity |
| Required – Scope 3 |  |  |  |  |  |
| Wf | Mass of goods transported per freight type |  | tonnes | Invoices for freight, weighbridge receipts or other entity records | Upstream Freight |
| Df | Total distance transported per freight type |  | km | Data from supplier, or other entity records | Upstream Freight |
| Qp | Units of purchased packaging per packaging type |  | units | Invoices for packaging purchased, or other entity records | Purchased Packaging |
| Qwt | Quantity of waste per type being disposed beyond the post-production boundary. |  | tonnes | Weighbridge records, skip disposal invoices. | Management of Waste |
| Vwt | Volume of waste per type being disposed beyond the post-production boundary. |  | m3 | Only required if Qwt is not known. Estimated from known volumes of waste transported (e.g., number of skips filled). | Management of Waste |

The checklist below summarises all the optional data points, required for Method 2, that may be collected at entity-level for a post-production system to complete the equations detailed in subsequent chapters based on the post-production module map.

Table 29: Potential optional data for post-production

| Parameter | Data to be collected | Data collected Yes/No/N/A | Unit | Data Source | Module |
| --- | --- | --- | --- | --- | --- |
| Optional – Scope 2 |  |  |  |  |  |
| RECsurrendered | Eligible Renewable Energy Certificates voluntarily surrendered in the reporting year. |  | MWh | Retailer invoice or Clean Energy Regulator REC registry. | Purchased Electricity & Emissions from Purchased Electricity (market-based approach only) |
| REConsite | Eligible Renewable Energy Certificates that have been or will be issued for electricity produced on-site during the reporting period and consumed by the entity. |  | MWh | Clean Energy Regulator REC registry and Meter readings. | Purchased Electricity & Emissions from Purchased Electricity (market-based approach only) |
| Optional – Scope 3 |  |  |  |  |  |
| EFp | Emission factor for purchased packaging |  | kg CO2e/unit | Supplier-provided life-cycle emission factors that have been third-party verified as conforming either to ISO 15067:2018 (carbon footprints of products) or the GHG Protocol Product Life Cycle Accounting and Reporting Standard. | Purchased Packaging |

# References

[1]	S. Wood and A. Cowie, A Review of Greenhouse Gas Emission Factors for Fertiliser Production, vol. 38. 2004.

[2]	KM Christie-Whitehead and Dairy Australia, “Australian Dairy Carbon Calculator,” Tasmanian Institute of Agriculture; Dairy Australia, Launceston, Tasmania; Melbourne, Victoria, Feb. 2025. [Online]. Available: https://www.dairyaustralia.com.au/en/climate-and-environment/greenhouse-gas-emissions/australian-dairy-carbon-calculator

[3]	Integrity Ag, Australia Egg Industry Emissions Calculator. (Feb. 21, 2025). Australian Eggs. [Online]. Available: https://www.australianeggs.org.au/for-farmers/tools-and-training/ghg-calculator

[4]	Eugene McGahan, Mary-Frances Copley, Sara Willis, and Alan Skerman, “PigBal 5 Technical Manual,” The Queensland Government, State of Queensland, 2025. [Online]. Available: https://australianpork.com.au/manure-and-effluent-calculators

[5]	“Urea Ammonium Nitrate - UAN Determination,” Mettler Toledo. Accessed: Aug. 11, 2025. [Online]. Available: https://www.mt.com/au/en/home/library/applications/lab-analytical-instruments/Urea-ammonium-nitrate-determination.html

[6]	E. Calvo Buendia et al., “Chapter 5: Cropland,” in 2019 Refinement to the 2006 IPCC Guidelines for National Greenhouse Gas Inventories. Volume 4: Agriculture, Forestry and Other Land Use (AFOLU), Intergovernmental Panel on Climate Change (IPCC), Ed., IPCC, 2019.