# DRAFT Greenhouse Gas Emissions Estimation and Reporting Guidelines for Agriculture, Fisheries and Forestry

Methodological Guidance

Chapters 5-7: Fertiliser, Agriculture Residue Management, and Rice

© Commonwealth of Australia 2026

Ownership of intellectual property rights

Unless otherwise noted, copyright (and any other intellectual property rights) in this publication is owned by the Commonwealth of Australia (referred to as the Commonwealth).

Creative Commons licence

All material in this publication is licensed under a Creative Commons Attribution 4.0 International Licence except content supplied by third parties, logos and the Commonwealth Coat of Arms.

Inquiries about the licence and any use of this document should be submitted via our online contact form.

Cataloguing data

This publication (and any material sourced from it) should be attributed as: DCCEEW 2026, DRAFT Greenhouse Gas Emissions Estimation and Reporting Guidelines for Agriculture, Fisheries and Forestry - Methodological Guidance, Department of Climate Change, Energy, the Environment and Water, Canberra, March. CC BY 4.0.

This publication is available at dcceew.gov.au.

Department of Climate Change, Energy, the Environment and Water

GPO Box 3090 Canberra ACT 2601

Telephone 1800 920 528

Web dcceew.gov.au

Disclaimer

The Australian Government acting through the Department of Climate Change, Energy, the Environment and Water has exercised due care and skill in preparing and compiling the information and data in this publication. Notwithstanding, the Department of Climate Change, Energy, the Environment and Water, its employees and advisers disclaim all liability, including liability for negligence and for any loss, damage, injury, expense or cost incurred by any person as a result of accessing, using or relying on any of the information or data in this publication to the maximum extent permitted by law.

Acknowledgements

We thank Environmental Accounting Services for drafting this document, and the Voluntary Greenhouse Gas Estimation and Reporting Standards Reference Group for their valued contributions.

Acknowledgement of Country

We acknowledge the Traditional Owners of Country throughout Australia and recognise their continuing connection to land, waters and culture. We pay our respects to their Elders past and present.

Contents

5	Fertiliser Use	6

5.1	Inorganic and urea fertiliser application	7

5.1.1	Estimation methodology	7

5.1.1.1	Method 1 – Inorganic Fertiliser Application N2O Emissions	7

5.1.1.2	Method 2 – Inorganic Fertiliser Application N2O Emissions	8

5.1.1.3	Method 1 – Inorganic Fertiliser Application CO2 Emissions	9

5.1.1.4	Method 2 – Inorganic Fertiliser Application CO2 Emissions	9

5.1.2	Data/Parameters	10

5.1.2.1	Input Data (Required)	10

Data (Method 1 and 2 Options)	12

5.1.2.2	Constant Parameters	13

5.2	Organic fertiliser application	14

5.2.1	Estimation methodology	14

5.2.1.1	Method 1 – Organic Fertiliser Application N2O Emissions	14

5.2.1.2	Method 2 – Organic Fertiliser Application N2O Emissions	15

5.2.2	Data/Parameters	15

5.2.2.1	Input Data (Required)	15

5.2.2.2	Data (Method 1 and 2 Options)	16

5.2.2.3	Constant Parameters	16

5.3	Lime application	17

5.3.1	Estimation methodology	17

5.3.1.1	Lime Application CO2 Emissions – Method 1	17

5.3.1.2	Lime Application CO2 Emissions – Method 2	17

5.3.2	Data/Parameters	17

5.3.2.1	Input Data (Required)	17

5.3.2.2	Data (Method 1 and 2 Options)	18

5.3.2.3	Constant Parameters	18

5.4	Atmospheric deposition resulting from fertiliser application	20

5.4.1	Estimation methodology	20

5.4.1.1	Method 1 – Inorganic Fertiliser Atmospheric Deposition N2O Emissions	20

5.4.1.2	Method 2 – Inorganic Fertiliser Atmospheric Deposition N2O Emissions	20

5.4.1.3	Method 1 – Organic Fertiliser Atmospheric Deposition N2O Emissions	20

5.4.1.4	Method 2 – Organic Fertiliser Atmospheric Deposition N2O Emissions	21

5.4.2	Data/Parameters	21

5.4.2.1	Input Data (Required)	21

5.4.2.2	Data (Method 1 and 2 Options)	21

5.4.2.3	Constant Parameters	21

5.5	Leaching and run-off resulting from fertiliser application	23

5.5.1	Estimation methodology	23

5.5.1.1	Method 1 — Fertiliser Leaching and Runoff N2O	23

5.5.1.2	Method 2 — Fertiliser Leaching and Runoff N2O	24

5.5.2	Data/Parameters	24

5.5.2.1	Input Data (Required)	24

5.5.2.2	Data (Method 1 and 2 Options)	24

5.5.2.3	Constant Parameters	24

6	Agricultural Residue Management	26

6.1	Return of crop residues to soils	27

6.1.1	Estimation methodology	27

6.1.1.1	Method 1 — Crop Residue N2O Emissions	27

6.1.1.2	Method 2 — Crop Residue N2O Emissions	28

6.1.2	Data/Parameters	28

6.1.2.1	Input Data (Required)	28

6.1.2.2	Data (Method 1 and 2 Options)	29

6.1.2.3	Constant Parameters	30

6.2	Return of pasture residues to soils	33

6.2.1	Estimation methodology	33

6.2.1.1	Method 1 — Pasture Residue N2O Emissions	33

6.2.1.2	Method 2 — Pasture Residue N2O Emissions	33

6.2.2	Data/Parameters	34

6.2.2.1	Input Data (Required)	34

6.2.2.2	Input Data (Method 1 and 2 Options)	34

6.2.2.3	Constant Parameters	35

6.3	Leaching and run-off from crop and pasture residues	37

6.3.1	Estimation methodology	37

6.3.1.1	Method 1 — Residue Leaching and Runoff N2O	37

6.3.1.2	Method 2 — Residue Leaching and Runoff N2O	37

6.3.2	Data/Parameters	37

6.3.2.1	Input Data (Required)	37

6.3.2.2	Data (Method 1 and 2 Options)	37

6.3.2.3	Constant Parameters	37

6.4	Field burning of crop residues	39

6.4.1	Estimation methodology	39

6.4.1.1	Method 1 — Burning of Crop Residues CH4 and N2O	39

6.4.1.2	Method 2 — Burning of Crop Residues CH4 and N2O	40

6.4.2	Data/Parameters	40

6.4.2.1	Input Data (Required)	40

6.4.2.2	Data (Method 1 and 2 Options)	40

6.4.2.3	Constants	40

7	Rice Cultivation	42

7.1	Estimation methodology	44

7.1.1	Method 1 — Rice Cultivation CH4 Emissions	44

7.1.2	Method 2 — Rice Cultivation CH4 Emissions	45

7.2	Data/Parameters	45

7.2.1	Input Data (Required)	45

7.2.2	Data (Method 1 and 2 Options)	46

7.2.3	Constant Parameters	46

References	48

# Fertiliser Use

This module covers the estimation of nitrous oxide and carbon dioxide emissions that result from the use of fertilisers, both organic and inorganic, and lime. Figure 5.1 summarises the emission sources associated with the application of fertilisers and lime to soils and aquatic systems, and references the relevant chapters for the estimation methodology. Note that embedded emissions of purchased goods are discussed in Chapter 11.

Figure 5.1 Emissions that result from fertiliser and lime application to soils and aquatic systems

The following subscripts are used in this module:

| Subscript | Meaning |
| --- | --- |
| i | Climate zone |
| j | Production system |
| f | Fertiliser type |

Emission methodologies provide a sum total of emissions from the application of amendments at the entity-level (i.e., across all production systems and fertilisers). When undertaking product emissions intensity estimation apply the rule for distributing these emissions across a production cycle as per Section 10.3 of the Common Requirements Framework (CRF).

## Inorganic and urea fertiliser application

### Estimation methodology

Emissions from the application of inorganic and urea-based fertilisers to soils and aquatic systems are estimated based on the fertiliser type (f), as different formulations contain varying levels of nitrogen. The production system (j) will affect the selection of the emissions factor.

For fertiliser mixtures containing both synthetic and organic nitrogen sources, it is preferable to estimate the emissions from each component separately. If this separation is not possible, emissions should be calculated using the method for inorganic fertiliser application, based on the total nitrogen (N) content of the mixture [1, p. 11].

Inorganic fertilisers can release both nitrous oxides and carbon dioxide, depending on their composition. The methodology calculates the emissions for each GHG separately.

#### Method 1 – Inorganic Fertiliser Application N2O Emissions

(1)	Nitrous oxide emissions from the application of all inorganic fertiliser types f to all production systems j, EN2O (t N2O), are calculated as:

EN2O=jf(MNjf×EFj,N2O ×CN2O)×10-3

Where	MNjf = mass of nitrogen in inorganic fertiliser type f applied to production system j (kg N)

EFj,N2O = emission factor for nitrous oxide for inorganic fertilisers (kg N2O-N/kg N applied)

CN2O = factor to convert elemental mass of nitrous oxide to molecular mass

(2)	In equation (1) mass of nitrogen in inorganic fertiliser applied to production system, MNjf (kg N), is calculated as:

MNjf=TMjf×FNinorganic,f

Where	TMjf = total mass of inorganic fertiliser type f applied to production system j (kg)

FNinorganic,f = fraction of nitrogen in inorganic fertiliser type f (kg N/kg)

#### Method 2 – Inorganic Fertiliser Application N2O Emissions

Method 2 is the same as Method 1 except that under equation 5.1.1.1 (1) an emission factor EFj,N2O is applied based on entity-specific application rates of fertiliser to different production systems. A Method 2 option is available for the following production systems:

- irrigated crop – specific only to maize crops (EFj=irrigatedmaize,N2O),

- non-irrigated crop – specific only to grains in the high rainfall zone (EFj=nonirrigatedgrains,N2O), and

- cotton (EFj=cotton,N2O).

Method 2 approaches only exist for the aforementioned 3 production systems, as these were the only 3 systems where there were sufficient data and satisfactory correlations between EFs and fertiliser application rate to give high confidence in the use of regressions to improve emission estimation accuracy. As more data for other production systems become available the scope of Method 2 options for other production systems could be expanded.

(1)	In equation 5.1.1.1 (1) for irrigated maize, the emission factor EFj=irrigatedmaize,N2O may be estimated based on the linear regression developed by [2] as:

EFfj=irrigatedmaize,N2O=-0.1474+0.0061×Nfj=irrigatedmaize ×10-2

Where	Nfj=irrigatedmaize = application rate of inorganic fertiliser type f in the irrigated maize crop (kg N/ha)

(2)	In equation 5.1.1.1 (1) for non-irrigated grains in high rainfall zone, a value for EFj=nonirrigatedgrains,N2O may be estimated based on the linear regression developed by [2] as:

EFfj=nonirrigatedgrains,N2O=0.0781+0.0075×Nfj=nonirrigatedgrains× 10-2

Where	Nfj=nonirrigatedgrains = application rate of inorganic fertiliser type f in the grains crop (kg N/ha)

(3)	In equation 5.1.1.1 (1) for cotton, an entity-specific value for EFj=cotton,N2O may be estimated based on the two-component model developed by [3] as:

EFfj=cotton,N2O=0.01×0.29+0.007e0.037×Nfj=cotton-1Nfj=cotton×10-2

Where 	Nfj=cotton = application rate of inorganic fertiliser type f in the cotton crop (kg N/ha). This is capped at a maximum of 0.0183 kg CO2e/kg N, equivalent to N application rates of 300 kg N/ha [3]

Note: the application rate Nfj shall be in kg of nitrogen, not in kg of fertiliser. Equation  5.1.1.1 (2) may be used to quantify the kg of N, based on the N content and application rate of the fertiliser.

#### Method 1 – Inorganic Fertiliser Application CO2 Emissions

- Carbon dioxide emissions resulting from the use of all inorganic, urea-based fertilisers, ECO2 (t CO2), are calculated as:

ECO2=jf(MUjf×EFurea,CO2 ×CCO2)×10-3

Where	MUjf = mass of urea of inorganic fertiliser type f applied to production system j (kg urea)

EFurea,CO2 = emission factor for carbon emissions from urea-based fertilisers (kg C/kg urea)

CCO2= factor to convert elemental mass of carbon dioxide to molecular mass

(2)	In equation (1) mass of urea applied to production system MUjf is calculated as:

MUjf=TMjf×FUf

Where	TMjf = the total mass of inorganic fertiliser type f applied to production system j (kg)

FUf = the fraction of urea in fertiliser type f (kg urea/kg)

#### Method 2 – Inorganic Fertiliser Application CO2 Emissions

There is no Method 2 quantification option for this emission source.

### Data/Parameters

#### Input Data (Required)

| Data / Parameter | TMjf |
| --- | --- |
| Data unit | kg |
| Description | The total mass of inorganic fertiliser type f applied to production system j |
| Data source | Receipts, invoices, entity records |
| Quality assurance / quality control considerations | TMjf should include all fertiliser that was applied to production system in the reporting year. There should be no temporal allocation of the mass of fertiliser or emissions across multiple years if the fertiliser type is applied less frequently than annually (e.g., every two years) or the crop the fertiliser was applied to is not harvested in the year of reporting. Where it is reasonable to assume that all fertiliser purchased within the reporting year is applied in that year, TMj,f should be based on concrete records or invoices of fertiliser purchased within the reporting year. This approach is preferential to estimating TMj,f from approximate fertiliser application rates (kg/ha). Where an enterprise containing multiple production systems only has records for total fertiliser applied, it may use fertiliser application rates to apportion total fertiliser use to different production systems. For example, Farm A has purchase records of 3500 kg of monoammonium phosphate (MAP) in the reporting period, which is assumed to all be applied in that reporting period across its 100 ha of land. Farm A has two production systems: 20 ha of irrigated crop and 80 ha of non-irrigated pasture. MAP application rates in each production system are approximately 50 kg/ha and 30 kg/ha respectively. TMjf=MAP for each production system is estimated as: TMj=irrigatedcrop,f=MAP = 20 ha x 50 kg/ha = 1000 kg TMj=nonirrigatedpasture,f=MAP = 80 ha x 30 kg/ha = 2400 kg The estimated application rates do not account for the total MAP purchase records (100 kg is unaccounted for). Based on the approximate application rates, there is a 30:70 ratio of MAP application to irrigated crop relative to non-irrigated pasture. Applying this ratio to the total MAP consumption of 3500 kg: TMj=irrigatedcrop,f=MAP = 3500 kg/ha x 30 % = 1030 kg TMj=nonirrigatedpasture,f=MAP = 3500 kg/ha x 70 % = 2470 kg |

Question Reference 5.1.

The Guidance provides the fraction of nitrogen in inorganic fertilisers (FN) for the following common fertilisers: AN, CAN, DAP, MAP, UAN, urea, and SOA.

Are any other key inorganic fertilisers missing, for which it would be valuable to provide default (Method 1) values?

| Data / Parameter | FNinorganic,f |
| --- | --- |
| Data unit | kg N/kg |
| Description | Fraction of nitrogen in inorganic fertiliser type f |
| Data source | Default values for common inorganic fertilisers based on average fertiliser composition from [4], or calculated from product label information. Default fraction of nitrogen for common fertiliser types are provided Appendix Table A.2.2.3. If using product information label, nitrogen content by weight per cent (wt%) fertiliser composition is typically stated in the form of N:P:K (the ratio of nitrogen, phosphorus and potassium nutrients. A ratio of 10:5:5 indicates the fertiliser contains 10% nitrogen, 5% phosphorus, and 5% potassium by weight. 10% nitrogen is equivalent to FNinorganic,f = 0.1. |
| Quality assurance / quality control considerations | N/A |

| Data / Parameter | FUf |
| --- | --- |
| Data unit | kg urea/kg |
| Description | Fraction of urea in inorganic fertiliser type f |
| Data source | Default values for common inorganic fertilisers based on average fertiliser composition from [4] and [5] or calculated from product label information. Default fraction of urea in common fertiliser types are provided in Appendix Table A.2.2.4. If using product information label, urea content may be determined based on the urea content by weight per cent (wt%) stated on the product information label. |
| Quality assurance / quality control considerations | N/A |

#### Data (Method 1 and 2 Options)

| Data / Parameter | Nfj |
| --- | --- |
| Data unit | kg N/ha |
| Description | N application rate of inorganic fertiliser type f in production system j |
| Method 1 data source | N/A – this parameter is not used in Method 1 |
| Method 1 value | N/A – this parameter is not used in Method 1 |
| Method 2 data source | Farm records of fertiliser application rates, or calculated from: Nfj=Total fertiliser applied (kgN)Total area (ha) Data on total fertiliser applied and total area should be sourced from farm invoices and spatial records respectively. |
| Quality assurance / quality control considerations | Value can be cross checked with industry benchmarks or management practices for fertiliser types or production systems. |

| Data / Parameter | EFj,N2O |
| --- | --- |
| Data unit | kg N2O-N/kg N |
| Description | Emission factor for direct nitrous oxide emissions from inorganic fertiliser f applied to production system j |
| Method 1 data source | See Appendix Table A.2.2.1 |
| Method 1 value | Select an appropriate value based on the production system. For non-irrigated cropping systems, the emission factor varies depending on whether the agricultural system is in a high rainfall or low rainfall zone (refer to Chapter 1 Section 1.8.2 for guidance on determining the rainfall zone). |
| Method 2 data source | Calculated from relevant equations in 5.1.1.2. |
| Quality assurance / quality control considerations | If Method 1 is used ensure the most recently available published data is used in alignment with the Australian National Inventory Report. If Method 2 is used, entity title data may be checked by verifiers to ensure locations used to identify the rainfall zone are correct. |

Question Reference 5.2.

Currently it has been assumed that non-irrigated pasture is the most suitable production system for forestry plantations for calculation of direct emissions from inorganic fertiliser application. Are there any other industry specific research papers on direct emissions from fertiliser application within forestry plantations that could be utilised here to support estimation?

Question Reference 5.3.

The current guidance reflects that globally some aquaculture systems may use fertilisers to increase available nutrients. Is this also the case in Australian aquaculture systems?

To estimate these direct emissions from fertiliser application to aquaculture systems an emission factor has been sourced from the IPCC 2019 Refinement based on peer review literature studies on rivers, reservoirs, and estuaries. Are there any examples of these emissions from fertiliser application to aquaculture systems being calculated by different estimation methods or using other emissions factors?

#### Constant Parameters

| Data / Parameter | CN2O |
| --- | --- |
| Data unit | fraction |
| Description | Factor to convert elemental mass of nitrous oxide to molecular mass |
| Data source | IPCC Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories. Chapter 5: Agriculture [6] |
| Value | 1.57 |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

| Data / Parameter | CCO2 |
| --- | --- |
| Data unit | fraction |
| Description | Factor to convert elemental mass of carbon dioxide to molecular mass |
| Data source | IPCC Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories. Chapter 5: Agriculture [6] |
| Value | 3.67 |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

| Data / Parameter | EFurea,CO2 |
| --- | --- |
| Data unit | kg C/kg urea |
| Description | Emission factor for carbon emissions from urea application |
| Data source | National Inventory Report Volume 1 [7] |
| Value | 0.2 |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

## Organic fertiliser application

### Estimation methodology

Emissions from the application of organic fertilisers applied to soils and aquatic systems are estimated based on the fertiliser type (f), which includes both purchased organic fertilisers as well as animal manure obtained from livestock manure management systems and applied to pastures or crops. The production system (j) and climate zone (i) will affect the selection of the emissions factor. Emissions associated deposition of urine and dung directly onto pasture by grazing animals is quantified in Chapter 4.

The application of organic fertilisers to soils and aquatic systems generates only nitrous oxide emissions.

#### Method 1 – Organic Fertiliser Application N2O Emissions

(1)	Nitrous oxide emissions for all organic fertiliser types f and all production systems j, EN2O (t N2O), are calculated as:

EN2O=jf(MNSoiljf×EFN2O,i ×CN2O)×10-3

Where	MNSoiljf = mass of nitrogen in organic fertiliser type f applied to production system j (kg N)

EFN2O,i  = emission factor for direct nitrous oxide emissions from organic fertiliser applied to soils and aquatic systems in climate zone i (kg N2O-N/kg N).

CN2O = factor to convert elemental mass of nitrous oxide to molecular mass

Where the organic fertiliser is manure and the manure is generated from a manure management system within the boundary of the entity, the value of MNSoil should be calculated using the methods provided in the Manure Management module (Chapter 4). If the manure is purchased, and is not amended with other products or additionally processed, and can be traced back to the source livestock and MMS, MNSoil could be estimated using methods provided in the Manure Management module (Chapter 4). Where the organic fertiliser is purchased and brought into the entity boundary from elsewhere and cannot be traced to its origin or is amended with other products or has undergone additional processing steps, use equation 5.2.1.1 (2) to calculate MNSoil.

(2) 	In equation (1) mass of nitrogen in purchased organic fertilisers applied to production system, MNSoiljf (kg N) is calculated as:

MNSoiljf= Morganic,jf×FNorganic,f

Where 	Morganic,jf = mass of organic fertiliser type f applied to production system j (kg)

FNorganic,f = nitrogen fraction of organic fertiliser type f (kg N/kg)

#### Method 2 – Organic Fertiliser Application N2O Emissions

There is no Method 2 quantification option for this emission source.

### Data/Parameters

#### Input Data (Required)

| Data / Parameter | Morganic,jf |
| --- | --- |
| Data unit | kg |
| Description | Mass of organic fertiliser type f applied to production system j |
| Data source | Where the manure can be traced back to the source livestock and MMS and is not amended with other products or additionally processed, Mmanure,jf is not required and instead the value of MNSoil shall be calculated using outputs from the MMS emission estimation equations (Chapter 4). |
| Quality assurance / quality control considerations | Morganic,jf should reflect all organic fertiliser that was applied to production system in the reporting year. There should be no temporal allocation of the mass of fertiliser or emissions across multiple years if the fertiliser type is applied less frequently than annually (e.g., every two years). Where it is reasonable to assume that all fertiliser purchased within the reporting year is applied in that year, Morganic,jf should be based on concrete records or invoices of fertiliser purchased within the reporting year. This approach is preferential to estimating Morganic,jf from approximate fertiliser application rates (kg/ha). Where an enterprise containing multiple production systems only has records for total fertiliser applied, it may use fertiliser application rates to apportion total fertiliser use to different production systems. For example, Farm A has purchase records of 3500 kg of organic fertiliser “Green Gold” in the reporting period, which is assumed to all be applied in that reporting period across its 100 ha of land. Farm A has two production systems: 20 ha of irrigated crop and 80 ha of non-irrigated pasture. Green Gold application rates in each production system are approximately 50 kg/ha and 30 kg/ha respectively. Morganic,jf=GreenGold for each production system is estimated as: Morganic,j=irrigatedcrop,f=GG = 20 ha x 50 kg/ha = 1000 kg Morganic,j=nonirrigatedpasture,f=GG = 80 ha x 30 kg/ha = 2400 kg The estimated application rates do not account for the total Green Gold purchase records (100 kg is unaccounted for). Based on the approximate application rates, there is a 30:70 ratio of Green Gold application to irrigated crop relative to non-irrigated pasture. Applying this ratio to the total Green Gold consumption of 3500 kg: Morganic,j=irrigatedcrop,f=GG = 3500 kg/ha x 30 % = 1030 kg Morganic,j=nonirrigatedpasture,f=GG = 3500 kg/ha x 70 % = 2470 kg |

| Data / Parameter | FNorganic,f |
| --- | --- |
| Data unit | kg N/kg |
| Description | Nitrogen fraction of the organic fertiliser type f applied to production system |
| Data source | Supplier-provided values or product label information should be used. Product information label values expressed as wt% shall be converted into a fraction for use in FNorganic,f. (e.g., for organic fertiliser that has a 20 wt% content of nitrogen, FNorganic,f = 0.2). or calculated from product label information. Default values from IPCC Volume 4 Chapter 5, Table 5.5c [8] (See Appendix Table A.2.2.5) may be used for manure-based organic fertiliser that was purchased off-farm, where the livestock that generated the organic fertiliser is known. |
| Quality assurance / quality control considerations | FNorganic,f is only required where the organic fertiliser has been sourced from outside of the entity boundary. Where manure generated from an MMS is being applied to soils or aquatic system, the value of MNSoil shall be calculated directly following the methodologies of the Manure Management module (Chapter 4). |

#### Data (Method 1 and 2 Options)

Not applicable.

#### Constant Parameters

Refer to Section 5.1 of this module for the following parameter tables: CN2O.

| Data / Parameter | EFN2O,i |
| --- | --- |
| Data unit | kg N2O-N/kg N |
| Description | The direct nitrous oxide EF from the organic fertiliser when applied to soils or aquatic systems in climate zone i |
| Data source | See Appendix Table A.2.1.4 |
| Value | Select appropriate value based on climate zone. Refer to Chapter 1 Section 1.8.2 on guidance to determine which climate zones the enterprise is located in. |
| Quality assurance / quality control considerations | If Method 1 is used ensure the most recently available published data is used in alignment with the Australian National Inventory Report. If Method 2 is used entity title data may be checked to ensure location and climate data used to climate zone are correct. |

## Lime application

### Estimation methodology

#### Lime Application CO2 Emissions – Method 1

- The emissions from the application of lime and dolomite to production systems Elime (t CO2) is calculated as:

Elime=(Mlime×FracLime ×Plime×EFlime)+(Mlime×FracDol ×Pdol×EFdol)×CCO2×10-3

Where	Mlime = the total mass of lime applied to production system (kg)

FracLime = the fraction of lime as limestone (CaCO3)

Plime = fractional purity of limestone

EFlime = emission factor for limestone fraction of lime (kg C/kg)

FracDol = the fraction of lime as dolomite

Pdol = fractional purity of dolomite

EFdol = emission factor for dolomite fraction of lime (kg C/kg)

CCO2= factor to convert elemental mass of carbon dioxide to molecular mass

#### Lime Application CO2 Emissions – Method 2

There is no Method 2 quantification option for this emission source.

### Data/Parameters

#### Input Data (Required)

| Data / Parameter | Mlime |
| --- | --- |
| Data unit | kg |
| Description | The mass of lime applied to production system |
| Data source | Invoices, receipts |
| Quality assurance / quality control considerations | Mlime should reflect all lime that was applied to production system in the reporting year. There should be no temporal allocation of the mass of lime or emissions across multiple years, if lime is applied less frequently than annually (e.g., lime only applied every five years). Where it is reasonable to assume that all lime purchased within the reporting year is applied in that year, Mlime should be based on concrete records or invoices of lime purchased within the reporting year. This approach is preferential to estimating Mlime from approximate application rates (kg/ha). |

| Data / Parameter | FracDol |
| --- | --- |
| Data unit | fraction |
| Description | The fraction of lime as dolomite |
| Data source | The dolomite (magnesium carbonate – CaMg (CO3)2) content of lime may be determined based on the content by weight per cent (wt%) on the product information label Product information label values expressed as wt% shall be converted into a fraction (e.g., for a 40 wt% content of dolomite, FracDol = 0.40). Additionally, if limestone content is known then FracDol = 1 – FracLime If only limestone is applied FracDol will be equal to 0 |
| Quality assurance / quality control considerations | FracLime and FracDol summed shall equal 1. Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

#### Data (Method 1 and 2 Options)

Not applicable

#### Constant Parameters

Refer to Section 5.1 of this module for the following parameter tables: CCO2.

| Data / Parameter | Plime |
| --- | --- |
| Data unit | fraction |
| Description | Fractional purity of limestone |
| Data source | National Inventory Report Volume 1 [7] |
| Value | 0.9 |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

| Data / Parameter | Pdol |
| --- | --- |
| Data unit | fraction |
| Description | Fractional purity of dolomite |
| Data source | National Inventory Report Volume 1 [7] |
| Value | 0.95 |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

| Data / Parameter | EFlime |
| --- | --- |
| Data unit | kg CO2e / kg |
| Description | Emission factor for limestone fraction of lime |
| Data source | National Inventory Report Volume 1 [9, p. 1] |
| Value | 0.12 |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

| Data / Parameter | EFdol |
| --- | --- |
| Data unit | kg CO2e / kg |
| Description | Emission factor for dolomite fraction of lime |
| Data source | National Inventory Report Volume 1 [7] |
| Value | 0.13 |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

## Atmospheric deposition resulting from fertiliser application

### Estimation methodology

Indirect emissions by atmospheric deposition can arise from the application of inorganic and organic fertilisers to soil and aquatic systems. Methods for estimating emissions from urine and dung excreted by grazing livestock directly on to pasture are covered in Chapter 4.

#### Method 1 – Inorganic Fertiliser Atmospheric Deposition N2O Emissions

- Nitrous oxide emissions from atmospheric deposition from all inorganic fertiliser types f and all production systems j, Ead,inorganic (t N2O), is calculated as:

Ead,inorganic=jf(Mvol,inorganic,jf×EFN2O,j)×CN2O×10-3

Where	Mvol,inorganic,jf = mass of nitrogen volatised from inorganic fertiliser applied to production system (kg N)

EFN2O,j = emission factor for atmospheric deposition (kg N2O-N/kg N)

CN2O = factor to convert elemental mass of nitrous oxide to molecular mass

(2)	In equation (1) mass of nitrogen volatilised from inorganic fertiliser, Mvol,inorganic,jf, is calculated as:

Mvol,inorganic,jf=MNjf×FracGASFf

Where	MNjf= mass of nitrogen in inorganic fertiliser applied (kg N) (calculated in Section 5.1)

FracGASFf= fraction of inorganic fertiliser that volatilises ((kg NH3–N + NOx–N)/kg N)

#### Method 2 – Inorganic Fertiliser Atmospheric Deposition N2O Emissions

There is no Method 2 quantification option for this emission source.

#### Method 1 – Organic Fertiliser Atmospheric Deposition N2O Emissions

(1)	Nitrous oxide emissions from atmospheric deposition of all organic fertiliser types f and all production systems j, Ead,organic (t N2O), is calculated as:

Ead,organic=jf(Mvol,organic,jf×EFN2O,j)×CN2O×10-3

Where	Mvol,organic,jf = mass of nitrogen volatilised from organic fertiliser applied to production system (kg N)

EFN2O,j= emission factor for atmospheric deposition in production system j (kg N2O–N/(kg N)

(2)	In equation (1) mass of nitrogen volatilised from organic fertiliser applied to production systems Mvol,organic,jf (kg N), is calculated as:

Mvol,organic,jf=MNSoiljf×FracGASMsoil

Where	MNSoiljf = mass of nitrogen in organic fertiliser type f applied to production system j (kg N) (calculated in Section 5.1.2)

FracGASMsoil = fraction of organic fertiliser N that volatilises ((kg NH3–N + NOx–N)/kg N)

#### Method 2 – Organic Fertiliser Atmospheric Deposition N2O Emissions

There is no Method 2 quantification option for this emission source.

### Data/Parameters

#### Input Data (Required)

There is no additional input data required for this module.

#### Data (Method 1 and 2 Options)

Not applicable

#### Constant Parameters

Refer to Section 5.1 of this module for the following parameter tables: CN2O.

| Data / Parameter | FracGASFf |
| --- | --- |
| Data unit | (kg NH3–N + NOx–N)/kg N |
| Description | Fraction of inorganic fertiliser f that volatilises as NH3 and NOx |
| Data source | See Appendix Table A.2.2.6 |
| Value | Select appropriate value based on fertiliser type. |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

| Data / Parameter | FracGASMsoil |
| --- | --- |
| Data unit | (kg NH3–N + NOx–N)/kg N |
| Description | Fraction of organic fertiliser that volatilises as NH3 and NOx |
| Data source | National Inventory Report Volume 1 [7] |
| Value | 0.21 |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

| Data / Parameter | EFN2O,j |
| --- | --- |
| Data unit | kg N2O-N/(kg NH3–N + NOx–N) |
| Description | Emission factor for atmospheric deposition from fertiliser application for production system j |
| Data source | National Inventory Report Volume 1 [7] |
| Value | The same emission factor as that for direct nitrous oxide emissions from inorganic fertiliser (EFN2O,j) is applied, based on the production system where the fertiliser is applied (or that best reflects the system surrounding the enterprise). These values are provided in Appendix Table A.2.2.1. Refer to Chapter 1 Section 1.8.2 for guidance on determining the rainfall zone, for non-irrigated crop systems. For aquaculture, it has been assumed that there are no atmospheric deposition emissions and therefore EFN2O,j is set to 0. |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

## Leaching and run-off resulting from fertiliser application

### Estimation methodology

Indirect emissions from leaching and runoff can arise from the application of inorganic and organic fertilisers to soil. Methods for estimating emissions from urine and dung excreted by grazing livestock directly on to pasture are covered in Chapter 4. The climate zone and production system will affect the fraction of nitrogen available for leach. Leaching and runoff only occurs in areas that receive enough rainfall, or are irrigated (except for drip irrigation), in order for water to drain through the soil profile.

#### Method 1 — Fertiliser Leaching and Runoff N2O

(1)	Nitrous oxide emissions from nitrogen leaching and runoff from the application of all fertilisers (both organic and inorganic) across all production systems j, Efert,leach (t N2O), is calculated as:

Efert,leach=j(Mleach,inorganic,j+Mleach,organic,j)×EFleach×CN2O×10-3

Where	Mleach,inorganic,j = mass of nitrogen lost through leaching and runoff from inorganic fertiliser application in production system j (kg N)

Mleach,organic,j = mass of nitrogen lost through leaching and runoff from organic fertiliser application (kg N)

EFleach = emission factor for nitrogen leaching and runoff (kg N2O-N / kg N)

CN2O = factor to convert elemental mass of nitrous oxide to molecular mass

(2)	In equation (1) mass of nitrogen lost through leaching and runoff from inorganic fertiliser application, Mleach,inorganic,j (kg N), is calculated as:

Mleach,inorganic,j=fMNjf×FracWET,j×FracLEACH

Where	MNjf= mass of nitrogen in inorganic fertiliser (kg N) (calculated in Section 5.1)

FracWET,j = fraction of N available for leaching and runoff

FracLEACH = fraction of N that is lost through leaching and runoff

(3)	In equation (2) mass of nitrogen lost through leaching and runoff from organic fertiliser application, Mleach,organic,j (kgN), is calculated as:

Mleach,organic,j=fMNSoiljf×FracWET,j×FracLEACH

Where	MNSoiljf = mass of nitrogen in organic fertiliser (kg N) (calculated in Section 5.2.1)

#### Method 2 — Fertiliser Leaching and Runoff N2O

There is no Method 2 option for this emission source.

### Data/Parameters

#### Input Data (Required)

There is no additional input data required for this module.

#### Data (Method 1 and 2 Options)

Not applicable.

#### Constant Parameters

Refer to Section 5.1 of this module for the following parameter tables: CN2O.

| Data / Parameter | FracWETj |
| --- | --- |
| Data unit | fraction |
| Description | Fraction of N that is available for leaching and runoff |
| Data source | National Inventory Report Volume 1 [10] |
| Value | Select appropriate value based on production system and state. To determine whether the entity is located in an area where leaching occurs refer to Chapter 1 Section 1.8.2 on how to determine whether the enterprise is located in a leaching zone. Where it is determined that leaching does occur, FracWET = 1 Where it is determined that leaching does not occur, FracWET = 0. Noting that for aquaculture, it has been assumed that the emissions associated with leaching and runoff from fertiliser application is has been captured in direct application emission factors and FracWET is equal to 0. |
| Quality assurance / quality control considerations | Entity title data may be checked to ensure spatial boundaries used to define leaching zone are correct. Other inputs such as electricity or fuel for irrigation purposes may be used to highlight if pastures are irrigated and therefore leaching will be occurring. |

| Data / Parameter | FracLEACH |
| --- | --- |
| Data unit | fraction |
| Description | Fraction of nitrogen that is lost through leaching and runoff |
| Data source | National Inventory Report Volume 1 [7] |
| Value | 0.24 |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

| Data / Parameter | EFleach |
| --- | --- |
| Data unit | kg N2O-N / kg N |
| Description | Emission factor for nitrogen leaching and runoff |
| Data source | National Inventory Report Volume 1 [9, p. 1] |
| Value | 0.011 |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

# Agricultural Residue Management

This module covers the estimation of methane and nitrous oxide emissions that result from agricultural residues, both crop and pasture residues, being applied to soils or burnt. Figure 6.1 summarises the emission sources from the application of crop and pasture residues to soils, as well as the burning of crop residues.

Figure 6.1: Emissions that result from agricultural residue management

The following subscripts are used in this module:

| Subscript | Meaning |
| --- | --- |
| i | Climate Zone |
| c | Crop type |
| p | Pasture crop type |

Emission methodologies result in  a total sum to quantify annual emissions at the entity-level (i.e., across all production systems and fertilisers).  When undertaking product emissions intensity estimation apply the rule for distributing emissions across a production cycle as per Section 10.3 of   the Common Requirements Framework.

## Return of crop residues to soils

### Estimation methodology

#### Method 1 — Crop Residue N2O Emissions

Emissions from the return of crop residues to soils are based on the crop type (c), as different crops will contain varying levels of nitrogen. The climate zone will affect the selection of the emission factor.

Where no crop residues remain in the soil after harvesting, there will be no N2O emissions from crop residues.

(1)	Nitrous oxide emissions from the return of crop residues across all crops c to the soil, E (t N2O), are calculated as:

E=c(Mc×EFNi)×CN2O×10-3

Where		Mc = mass of nitrogen in crop c residues returned to the soil (kg N)

EFNi = emission factor for crop residues returned to the soil in climate zone i (kg N2O-N/ kg N)

CN2O  = factor to convert elemental mass of nitrous oxide to molecular mass

(2)	In equation (1) mass of nitrogen in crop residues returned to the soil, Mc (kg N), is calculated as:

Mc=Pc×RAGc×1-Fc-FFODc×DMc×NCAGc+Pc×RAGc×RBGc×DMc×NCBGc

Where		Pc = annual production of crop c (kg)

RAGc = residue to crop ratio for crop c

RBGc = below ground-residue to above ground residue ratio for crop c

Fc = fraction of crop residue that is burnt for crop c

FFODc = fraction of the crop residue that is removed for crop c

DMc = dry matter content of crop c (kg dry weight/kg crop residue)

NCAGc = nitrogen content of above-ground crop residue of crop c (kg N/kg DM)

NCBGc = nitrogen content of below-ground crop residue of crop c (kg N/kg DM)

Question Reference 6.1.

The crop attribute data for perennial and annual horticultural crops is currently based on limited data – from hops and other annual crops from IPCC respectively. Please advise if there are sources of crop-specific attribute data for these or other crops not listed in Appendix table A.2.1.1.

#### Method 2 — Crop Residue N2O Emissions

Method 2 is the same as Method 1 except that under equation 6.1.1.1 (2) an entity-specific FFODc value is required.

- In equation (2) FFODc may be calculated where residues are baled as:

FFODc=B×MBPc×RAG,c

Where	B = number of bales generated

MB = mass of each bale (kg)

Where residues are stubble-grazed, the default value for FFODc should be used.

Question Reference 6.2.

Are the current default values for crop residue removals (FFOD) appropriate for each crop type? Are there alternative data sources that could be used to improve the default values? Or should Method 1 require the use of site-specific FFOD data?

A Method 2 approach has been provided for estimating removals from baling. Is there a recommended approach for estimating crop stubble removal from gazing that could be provided?

### Data/Parameters

#### Input Data (Required)

| Data / Parameter | Pc |
| --- | --- |
| Data unit | kg |
| Description | Annual production of crop c |
| Data source | Farm records |
| Quality assurance / quality control considerations | Records should be based on the total production of crop, in dry weight, within the reporting period. Where crop failure has occurred that resulted in no harvest occurring, Pc should be estimated based on the expected harvest of that year. Historical yield data or yield data from surrounding paddocks of the same crop type may be used to estimate an accurate value for Pc. This value of Pc is only used in the quantification of emissions of crop residues applied to soils and should not be used to attain a value of emission intensity of the crop product. |

| Data / Parameter | Fc |
| --- | --- |
| Data unit | fraction |
| Description | Fraction of crop residue that is burnt for crop c |
| Data source | Farm records of area burnt and area cultivated. Entity-specific estimate of Fc may be calculated as: total area burnt (ha)total area cultivated (ha) |
| Quality assurance / quality control considerations | The following sources may be used to estimate and assure Fc : - shapefiles of areas burnt, in conjunction with evidence of burning, such as - monitoring reports, - field notes, - photography, or - aerial data to demonstrate burning occurring |

#### Data (Method 1 and 2 Options)

| Data / Parameter | FFODc |
| --- | --- |
| Data unit | fraction |
| Description | Fraction of the crop residue that is removed for crop type c |
| Method 1 data source | See Appendix Table A.2.1.2. |
| Method 1 value | Select appropriate value based on crop type and state. |
| Method 2 data source | Calculated from equation 6.1.1.2 (1). Where residues are stubble-grazed, the default value for FFODc should be used. |
| Quality assurance / quality control considerations | Where a Method 2 approach is adopted, justification of FFODc shall be provided and evidence documented, which may include: - records or invoices for number of bales sold B, - justification of approximate mass of bale MB. |

| Data / Parameter | B |
| --- | --- |
| Data unit | bales |
| Description | Number of bales generated |
| Method 1 data source | N/A – this parameter is not used in Method 1 |
| Method 1 value | N/A – this parameter is not used in Method 1 |
| Method 2 data source | Invoices for the number of bales sold off-farm, or other farm records documenting the number of bales made. |
| Quality assurance / quality control considerations | Check for alignment with rice cropping area and expected bale yield based on area. |

| Data / Parameter | MB |
| --- | --- |
| Data unit | kg |
| Description | Average mass of each bale generated |
| Method 1 data source | N/A – this parameter is not used in Method 1 |
| Method 1 value | N/A – this parameter is not used in Method 1 |
| Method 2 data source | Invoices from bale sales |
| Quality assurance / quality control considerations | Check for alignment with national average records for standard and large bale sizes. |

#### Constant Parameters

| Data / Parameter | EFNi |
| --- | --- |
| Data unit | kg N2O-N/kg N |
| Description | Emission factor for crop residues returned to the soil for climate zone i |
| Data source | See Appendix Table A.2.1.4 |
| Value | Select appropriate value based on climate zone. Refer to Chapter 1 Section 1.8.2 on guidance to determine which climate zones the enterprise is located in. Wet and dry climates used in this classification are a subset of the temperate and tropical climate zones – this is not to be confused with rainfall zones. |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

| Data / Parameter | CN2O |
| --- | --- |
| Data unit | fraction |
| Description | Factor to convert elemental mass of nitrous oxide to molecular mass |
| Data source | IPCC Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories. Chapter 5: Agriculture [6] |
| Value | 1.57 |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

| Data / Parameter | RAGc |
| --- | --- |
| Data unit | fraction |
| Description | Residue to crop ratio for crop type c |
| Data source | See Appendix Table A.2.1.1 |
| Value | Select the appropriate value based on crop type. Where the crop type is not listed, select the crop that has the most similar attributes. If the crop has no residues in the soil, N2O emissions associated with residues returned to zero are assumed 0. |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

| Data / Parameter | RBGc |
| --- | --- |
| Data unit | fraction |
| Description | Below ground-residue to above ground residue ratio for crop type c |
| Data source | See Appendix Table A.2.1.1 |
| Value | Select the appropriate value based on crop type. Where the crop type is not listed, select the crop that has the most similar attributes. If the crop has no residues in the soil, N2O emissions associated with residues returned to zero are assumed 0. |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

| Data / Parameter | DMc |
| --- | --- |
| Data unit | kg dry weight/kg crop residue |
| Description | Dry matter content of crop type c |
| Data source | See Appendix Table A.2.1.1 |
| Value | Select the appropriate value based on crop type. Where the crop type is not listed, select the crop that has the most similar attributes. If the crop has no residues in the soil, N2O emissions associated with residues returned to zero are assumed 0. |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

| Data / Parameter | NCAGc |
| --- | --- |
| Data unit | kg N/kg DM |
| Description | Nitrogen content of above-ground crop residue of crop type c |
| Data source | See Appendix Table A.2.1.1 |
| Value | Select the appropriate value based on crop type. Where the crop type is not listed, select the crop that has the most similar attributes. If the crop has no residues in the soil, N2O emissions associated with residues returned to zero are assumed 0. |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

| Data / Parameter | NCBGc |
| --- | --- |
| Data unit | kg N/kg DM |
| Description | Nitrogen content of below-ground crop residue of crop type c |
| Data source | See Appendix Table A.2.1.1 |
| Value | Select the appropriate value based on crop type. Where the crop type is not listed, select the crop that has the most similar attributes. If the crop has no residues in the soil, N2O emissions associated with residues returned to zero are assumed 0. |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

## Return of pasture residues to soils

### Estimation methodology

Nitrous oxide emissions are generated when pasture residues are returned to soils.

#### Method 1 — Pasture Residue N2O Emissions

(1) 	Nitrous oxide emissions from the return of pasture residues from all pasture types p to the soil, EN2O (t N2O), are calculated as:

EN2O=p(Mp×EFNi)×CN2O×10-3

Where	Mp = the mass of nitrogen in pasture p residues returned to the soil (kg N)

EFNi = emission factor for crop residues returned to the soil in climate zone i (kg N2O-N/ kg N)

CN2O  = factor to convert elemental mass of nitrous oxide to molecular mass

(2)	In equation (1) mass of nitrogen in pasture residues returned to the soil, Mp (kg N), is calculated as:
Mp=Ap×Yp×10-3×(1-FFODp)×NCAGp+Ap×Yp×10-3×RBGp×NCBGp

Where 		Ap = area of pasture renewed or removed (ha)

Question Reference 6.3.

What types of data do farmers typically have regarding pasture renewal activities?

Would it be useful to include a list of activities that qualify as pasture renewal? For example, returning residues to soils during seedbed preparation (e.g. tilling), or when pastures are removed as part of rotations.

Yp = average yield of pasture crop (t DM/ha)

NCAGp = N content of above-ground pasture residue (kg N/kg DM)

RBGp = below-ground residue to above-ground residue ratio for pasture crop

NCBGp = N content of below-ground pasture crop residue (kg N/kg DM)

FFODp = fraction of pasture yield that is removed

#### Method 2 — Pasture Residue N2O Emissions

Method 2 is the same as Method 1 except that under equation 6.2.1.1 (2) an entity-specific value for average pasture yield Yp is required.

### Data/Parameters

#### Input Data (Required)

| Data / Parameter | Ap |
| --- | --- |
| Data unit | ha |
| Description | Area of pasture crop p renewed or removed |
| Data source | Geospatial files, farm records |
| Quality assurance / quality control considerations | Area estimates for each pasture crop type p should not exceed the total area of pasture available on the farm. When using geospatial records to estimate area, conservative approaches to map the edges of pastures should be adopted. Area of pasture renewed or removed should align with typical renewal periods based on system intensity. |

Queston Reference 6.4.

Would it be valuable to have more specific mapping guidance on how to quantify areas of pasture crop renewed or removed using spatial mapping software, for example such as the guidance provided for ACCU methodologies?

#### Input Data (Method 1 and 2 Options)

| Data / Parameter | Yp |
| --- | --- |
| Data unit | t dry matter/ha |
| Description | Average yield of pasture crop p |
| Method 1 data source | See Appendix Table A.2.1.3 |
| Method 1 value | Select the pasture attribute data specific to each pasture crop. Where the pasture crop type is not listed, select the pasture type that has the most similar attributes. |
| Method 2 data source | Where yield, in dry matter, of pasture crops is known, this may be used to replace the default (Method 1) values. Yield records shall be used to justify the use of the Method 2 approach, rather than estimates. |
| Quality assurance / quality control considerations | If Method 1 is used ensure the most recently available published data is used in alignment with the Australian National Inventory Report. If Method 2 is used Method 1 default values may be used to cross check Method 2 inputs. |

#### Constant Parameters

Refer to Section 6.1 of this module for the following parameter tables: EFNi and CN2O.

| Data / Parameter | NCAGp |
| --- | --- |
| Data unit | kg N/kg dry matter |
| Description | N content of above-ground pasture crop p residue |
| Data source | See Appendix Table A.2.1.3 |
| Value | Select the pasture attribute data specific to each pasture crop. Where the pasture crop type is not listed, select the pasture type that has the most similar attributes. |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

| Data / Parameter | RBGp |
| --- | --- |
| Data unit | fraction |
| Description | Below-ground residue to above-ground residue ratio for pasture crop p |
| Data source | See Appendix Table A.2.1.3 |
| Value | Select the pasture attribute data specific to each pasture crop. Where the pasture crop type is not listed, select the pasture type that has the most similar attributes. |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

| Data / Parameter | NCBGp |
| --- | --- |
| Data unit | kg N/kg dry matter |
| Description | N content of below-ground pasture crop p residue |
| Data source | See Appendix Table A.2.1.3 |
| Value | Select the pasture attribute data specific to each pasture crop. Where the pasture crop type is not listed, select the pasture type that has the most similar attributes. |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

| Data / Parameter | FFODp |
| --- | --- |
| Data unit | fraction |
| Description | Fraction of pasture yield that is removed for pasture crop p |
| Data source | See Appendix Table A.2.1.3 |
| Value | Select the pasture attribute data specific to each pasture crop. Where the pasture crop type is not listed, select the pasture type that has the most similar attributes. |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

## Leaching and run-off from crop and pasture residues

### Estimation methodology

#### Method 1 — Residue Leaching and Runoff N2O

(1)	Nitrous oxide leaching and runoff emissions from crop and pasture residues returned to the soil, across all crop residues c and pasture residues p, EN2O (t N2O), is calculated as:

EN2O=cp(Mleach,cp×EFleach)×CN2O×10-3

Where	Mleach,cp = mass of nitrogen lost through leaching and runoff from the crop residue c or pasture residue p (kg N)

EFleach= emission factor for leaching and runoff (kg N2O-N / kg N)

CN2O  = factor to convert elemental mass of nitrous oxide to molecular mass

(2)	In equation (1) mass of nitrogen lost through leaching and runoff, Mleach,cp (kg N), is calculated as:

Mleach,cp=Mcp×FracWETj×FracLEACH

Where	Mcp = mass of nitrogen in crop residues c or pasture residues p applied to soils (kg N) (refer to Sections 6.1 and 6.2 for the calculation of this value for crops and pastures respectively)

FracWETj = fraction of N that is available for leaching and runoff from production system j

FracLEACH = fraction of N that is lost through leaching and runoff

#### Method 2 — Residue Leaching and Runoff N2O

There is no Method 2 option for this emission source.

### Data/Parameters

#### Input Data (Required)

The input data required for the estimation of field burning emissions is the same as that used in Sections 6.1 and 6.2 of this module, for the input data required for the calculation of Mc and Mp for crops and pastures respectively.

#### Data (Method 1 and 2 Options)

Not applicable.

#### Constant Parameters

Refer to Section 6.1.2.3 for the parameter tables for CN2O.

| Data / Parameter | EFleach |
| --- | --- |
| Data unit | kg N2O-N / kg N |
| Description | Emission factor for nitrogen leaching and runoff |
| Data source | National Inventory Report Volume 1 [9, p. 1] |
| Value | 0.011 |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

| Data / Parameter | FracWETj |
| --- | --- |
| Data unit | fraction |
| Description | Fraction of N that is available for leaching and runoff |
| Data source | National Inventory Report Volume 1 [10] |
| Value | Select appropriate value based on production system and state. To determine whether the entity is located in an area where leaching occurs refer to Chapter 1 Section 1.8.2 on how to determine whether the enterprise is located in a leaching zone. Where it is determined that leaching does occur, FracWET = 1 Where it is determined that leaching does not occur, FracWET = 0. Noting that for aquaculture, it has been assumed that the emissions associated with leaching and runoff from fertiliser application is has been captured in direct application emission factors and FracWET is equal to 0. |
| Quality assurance / quality control considerations | Entity title data may be checked to ensure spatial boundaries used to define leaching zone are correct. Other inputs such as electricity or fuel for irrigation purposes may be used to highlight if pastures are irrigated and therefore leaching will be occurring. |

| Data / Parameter | FracLEACH |
| --- | --- |
| Data unit | fraction |
| Description | Fraction of nitrogen that is lost through leaching and runoff |
| Data source | National Inventory Report Volume 1 [7] |
| Value | 0.24 |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

## Field burning of crop residues

### Estimation methodology

Emissions from the burning of crop residues are estimated only for methane and nitrous oxide. Carbon dioxide (CO2) emissions are not estimated, as it is assumed that any CO2 released during burning is equivalent to the CO2 that was absorbed from the atmosphere while the crop was growing.

#### Method 1 — Burning of Crop Residues CH4 and N2O

(1)	Methane emissions from the burning of crop residues ECH4 (t CH4), are calculated as:

ECH4=c(Mburn,c×CCc)×EFCH4×CCH4×10-3

Where		Mburn,c = mass of residue burnt for crop type c (kg)

CCc = carbon mass fraction in the residues of crop type c

EFCH4= emission factor for methane from burning of crop residues (kg CH4/kg CH4 burnt)

CCH4 = factor to convert elemental mass of methane to molecular mass

- Nitrous oxide emissions from the burning of crop residue EN2O (t N2O), are calculated as:
EN2O=c(Mburn,c×NCAGc)×EFN2O×CN2O×10-3

Where	NCAGc = nitrogen content in the above-ground residue of crop type c (as defined in Section 6.1)

EFN2O= emission factor for nitrous oxide from burning of crop residues (kg N2O/kg N2O burnt)

CN2O  = factor to convert elemental mass of nitrous oxide to molecular mass

(3) 	In equation (1) and (2), mass of residue of crop c burnt, Mburn,c (kg), is calculated as:

Mburn,c=Pc×RAGc×Sc×DMc×Zc×Fc

Where		Pc = annual production of crop type c (kg) (as defined in section 6.1)

RAGc= residue to crop ratio of crop type c (kg crop residue/kg crop) (as defined in Section 6.1)

Sc = fraction of residues of crop type c remaining at burning

DMc= dry matter content of crop type c (kg dry weight/kg crop residue) (as defined in Section 6.1)

Zc = burning efficiency (ratio of fuel burnt to fuel load) of crop type c

Fc = fraction of the annual production of crop type c that is burnt (as defined in Section 6.1)

#### Method 2 — Burning of Crop Residues CH4 and N2O

There is no Method 2 option for this emission source.

### Data/Parameters

#### Input Data (Required)

No additional input data is required for the estimation of field burning emissions. The same data as that used in Section 6.1 are required for this module.

#### Data (Method 1 and 2 Options)

Not applicable

#### Constants

Refer to Section 6.1 of this module for the following parameter tables: NCAGc, RAGc, DMc, CN2O and Fc.

| Data / Parameter | CCH4 |
| --- | --- |
| Data unit | fraction |
| Description | Factor to convert elemental mass of methane to molecular mass |
| Data source | National Inventory Report Volume 1 [7] |
| Value | 1.33 |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

| Data / Parameter | CCc |
| --- | --- |
| Data unit | fraction |
| Description | Carbon mass fraction in the residues of crop type c |
| Data source | See Appendix Table A.2.1.1 |
| Value | Select the appropriate value based on crop type. Where the crop type is not listed, select the crop that has the most similar attributes. |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

| Data / Parameter | EFCH4 |
| --- | --- |
| Data unit | kg CH4/kg CH4 burnt |
| Description | Emission factor for methane from burning of crop residues |
| Data source | National Inventory Report Volume 1 Table 5.31 [7] |
| Value | 0.0035 |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

| Data / Parameter | EFN2O |
| --- | --- |
| Data unit | kg N2O/kg N2O burnt |
| Description | Emission factor for nitrous oxide from burning of crop residues |
| Data source | National Inventory Report Volume 1 Table 5.31 [7] |
| Value | 0.0076 |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

| Data / Parameter | Sc |
| --- | --- |
| Data unit | fraction |
| Description | Fraction of residues of crop type c remaining at burning |
| Data source | See Appendix Table A.2.1.1 |
| Value | Select the appropriate value based on crop type. Where the crop type is not listed, select the crop that has the most similar attributes. |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

| Data / Parameter | Zc |
| --- | --- |
| Data unit | fraction |
| Description | Burning efficiency (ratio of fuel burnt to fuel load) of crop type c |
| Data source | See Appendix Table A.2.1.1 |
| Value | Select the appropriate value based on crop type. Where the crop type is not listed, select the crop that has the most similar attributes. |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

# Rice Cultivation

Methane emissions from rice cultivation are estimated using the IPCC methodology, which can be adjusted to reflect Australian circumstances through appropriate selection of the scaling factors. The scaling factors relate to the agricultural practices employed pre-season (prior to the growing season) and those employed during the growing season (also called the cultivation period). The following sub-scripts are used in this estimation methodology to differentiate between these:

| Subscript | Meaning |
| --- | --- |
| w | Rice cultivation method employed during the growing season |
| p | Water regime employed pre-season prior to the growing season |
| o | Organic amendment used during rice cultivation |

The tables below describe the different water regimes and cultivation methods that could be employed during the growing season and pre-season respectively.

| Water regime employed during the cultivation period, w |  | Description |
| --- | --- | --- |
| Type | Sub-type |  |
| Upland | Paddy rotation, or fallow without flooding in previous year | Fields are never flooded for a significant period of time. |
| Irrigated |  | Fields are flooded for a significant period of time and the water regime is fully controlled |
|  | Continuously flooded | Fields have standing water throughout the rice growing season and may only dry out for harvest (end-season drainage) |
|  | Single drainage period | Fields have a single drainage event and period during the cropping season at any growth stage, in addition to the end of season drainage |
|  | Multiple drainage periods | Fields have more than one drainage event and period of time without flooded conditions during the cropping season, in addition to an end of season drainage, including alternate wetting and drying (AWD) |
| Rainfed and deep water |  | Fields are flooded for a significant period of time with water regimes that depend solely on precipitation |
|  | Regular rainfed | The water level may rise up to 50 cm during the cropping season |
|  | Drought prone | Drought periods occur during every cropping season |
|  | Deep-water rice | Water level rises to more than 50 cm above the soil for a significant period of time during the cropping season |

| Water regime employed pre-season, p | Description |
| --- | --- |
| Non-flooded pre-season < 180 days | Rice is planted more than once a year, but there is more than one month of fallow time between the two seasons. Usually implies pre-season draining. |
| Non-flooded pre-season > 180 days | Single rice crop following a dry fallow period, where rice is planted once a year and the field is not flooded in the non-rice growing season. |
| Flooded pre-season >30 days | Short pre-season flooding periods of less than 30 days are not considered in selection of SFp. Shorter flooding periods, usually done to prepare the soil for ploughing, are assumed de minimis. |
| Non-flooded preseason >365 | Rice fields that are preceded by two upland crops or an upland crop and a drained fallow season (i.e., "upland crop - paddy rotation" or fallow without flooding in previous year). |

It is recommended to select the methods for pre-season and cultivation period practices that most closely resemble the practices adopted on-farm. For example:

- Aerial sowing practices: most commonly reflected by the “continuously flooded” water regime employed during the cultivation period

- Drill sowing practices: most commonly reflected by the “single drainage period” or “multiple drainage period” irrigated regimes, or one of the rainfed and deep water growing regimes.

It has been noted that research is underway to develop EFs specific to the aerial sowing and drill sowing practices of Australia. Until these have been published and reviewed, the IPCC-based approach outlined here (consistent with that used in the National Inventory Report) will be adopted.

## Estimation methodology

Direct emissions from rice cultivation arise through methane released from the decomposition of organic matter in the soil, which occurs through microbial action under anaerobic conditions following flooding of the rice crop. The methane emissions from rice cultivation can fluctuate both daily and throughout seasons, impacted by temperature, crop development, and changing soil conditions. An average emission factor EFrice,wpo, is employed, which is dependent on cultivation method employed both during the growing season as well as in the pre-season prior to the cultivation period

### Method 1 — Rice Cultivation CH4 Emissions

(1)	Methane emissions from rice cultivation, Erice (t CH4), are calculated as:

Erice=wpo(Arice,wpo×EFrice,wpo×trice,wpo)×10-3

Where	Arice,wpo = annual area of rice cultivation under water regime during the growing season w, pre-season water regime p, and organic amendment condition o (ha)

EFrice,wpo = emission factor from rice cultivation growing under specific water regime and amendment conditions (w, p and o) (kg CH4/ha/day)

trice,wpo = cultivation period for rice growing under specific water regime and amendment conditions (w, p and o) (days)

(2)	In equation (1) emission factor EFrice,wpo (kg CH4/ha), for rice grown under a specific water regime and organic amendment conditions is calculated as:

EFrice,wpo=EFc×SFw×SFp×SFo

Where	EFc = baseline EF for continuously flooded fields without organic amendments (kg CH4/ha/day)

SFw = scaling factor for water regime employed during the growing season cultivation period

SFp = scaling factor for water regime employed in the pre-season before the cultivation period

SFo= scaling factor for the type and amount of organic amendments applied. Where inorganic fertiliser is applied to the rice fields rather than organic amendments, SFo is set to 1.

(3)	In equation (2) the organic amendments scaling factor, SFo is calculated based on the type and amount of organic amendment o applied as follows:

SFo=1+oROAo×CFOAo0.59

Where	ROAo = application rate of organic amendment o (tonnes/ha)

CFOAo = conversion factor of organic amendment o

### Method 2 — Rice Cultivation CH4 Emissions

There is no Method 2 approach for this emission source.

## Data/Parameters

### Input Data (Required)

| Data / Parameter | Arice,wpo |
| --- | --- |
| Data unit | ha |
| Description | Annual area of rice cultivation under water regime during the growing season w, pre-season water regime p, and organic amendment condition o |
| Data source | Spatial analysis, farm records |
| Quality assurance / quality control considerations | Arice,wpo is the annual area under rice cultivation. Where there is more than one cropping season per year, Arice,wpo shall be scaled accordingly. For example, if a 150,000 ha rice area is planted for two growing seasons annually, Arice,wpo = 150,000 x 2 = 300,000 ha. |

| Data / Parameter | trice,wpo |
| --- | --- |
| Data unit | days |
| Description | Number of days in rice growing under water regime during the growing season w, pre-season water regime p, and organic amendment condition o |
| Data source | Monitoring records |
| Quality assurance / quality control considerations | trice,wpo should be based on the number of days of the rice growing season that fall in the reporting year. |

| Data / Parameter | ROAo |
| --- | --- |
| Data unit | For straw: tonnes (dry weight) / ha Other: tonnes (fresh weight) / ha |
| Description | Rate of application of organic amendment |
| Data source | Farm records |
| Quality assurance / quality control considerations | This input data is only applicable in systems where organic amendments are applied during rice cultivation, in order to calculate the scaling factor for organic amendments SFo. In systems where only inorganic fertilisers are applied, this input data is not applicable and SFo =1. |

### Data (Method 1 and 2 Options)

Not applicable

### Constant Parameters

| Data / Parameter | EFc |
| --- | --- |
| Data unit | kg CH4/ha/day |
| Description | Baseline emission factor for continuously flooded fields without organic amendments |
| Data source | IPCC 2019 Refinement Volume 4 Table 5.11 [8] |
| Value | 1.19 |
| Quality assurance / quality control considerations | N/A |

| Data / Parameter | SFw |
| --- | --- |
| Data unit | fraction |
| Description | Scaling factor based on the water regime employed during the growing season cultivation period |
| Data source | See Appendix Table A.2.3.1 |
| Value | Select appropriate value based on cultivation method employed during the growing season and water regime type and sub-type |
| Quality assurance / quality control considerations | N/A |

| Data / Parameter | SFp |
| --- | --- |
| Data unit | fraction |
| Description | Scaling factor based on the water regime employed in the pre-season before the growing period |
| Data source | See Appendix Table A.2.3.2 |
| Value | Select appropriate value based on water regime prior to cultivation period |
| Quality assurance / quality control considerations | Short pre-season flooding periods of less than 30 days (usually done to prepare the soil for ploughing) are assumed de minimis and not included in the emission quantification. |

| Data / Parameter | CFOAi |
| --- | --- |
| Data unit | fraction |
| Description | Conversion factor for organic amendment i |
| Data source | See Appendix Table A.2.3.3 |
| Value | Select appropriate value based on organic amendment |
| Quality assurance / quality control considerations | Straw incorporation means that straws are incorporated into the soil. It does not include cases where straws are just placed on soil surface, and straws that were burnt on the field. |

# References

[1]	K. Hergoualc’h et al., “Chapter 11: N₂O Emissions from Managed Soils, and CO₂ Emissions from Lime and Urea Application,” in 2019 Refinement to the 2006 IPCC Guidelines for National Greenhouse Gas Inventories. Volume 4: Agriculture, Forestry and Other Land Use (AFOLU), IPCC – Task Force on National Greenhouse Gas Inventories, Ed., IPCC, 2019. [Online]. Available: https://www.ipcc-nggip.iges.or.jp/public/2019rf/pdf/4_Volume4/19R_V4_Ch11_Soils_N2O_CO2.pdf

[2]	P. Grace et al., “Revised emission factors for estimating direct nitrous oxide emissions from nitrogen inputs in Australia’s agricultural production systems: a meta-analysis,” Soil Res., vol. 62, no. 1, Jan. 2024, doi: 10.1071/SR23070.

[3]	P. Grace, I. Shcherbak, B. Macdonald, C. Scheer, and D. Rowlings, “Emission factors for estimating fertiliser-induced nitrous oxide emissions from clay soils in Australia’s irrigated cotton industry,” Soil Res., vol. 54, pp. 598–603, Jul. 2016, doi: 10.1071/SR16091.

[4]	S. Wood and A. Cowie, A Review of Greenhouse Gas Emission Factors for Fertiliser Production, vol. 38. 2004.

[5]	“Urea Ammonium Nitrate - UAN Determination,” Mettler Toledo. Accessed: Aug. 11, 2025. [Online]. Available: https://www.mt.com/au/en/home/library/applications/lab-analytical-instruments/Urea-ammonium-nitrate-determination.html

[6]	IPCC Expert Groups, “IPCC Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories. Chapter 4: Agriculture.” IPCC. [Online]. Available: https://www.ipcc-nggip.iges.or.jp/public/gp/english/4_Agriculture.pdf

[7]	Department of Climate Change, Energy, the Environment and Water, “National Inventory Report 2023, Volume I.” Australian Government, 2023. [Online]. Available: https://www.dcceew.gov.au/sites/default/files/documents/national-inventory-report-2023-volume-1.pdf

[8]	E. Calvo Buendia et al., “Chapter 5: Cropland,” in 2019 Refinement to the 2006 IPCC Guidelines for National Greenhouse Gas Inventories. Volume 4: Agriculture, Forestry and Other Land Use (AFOLU), Intergovernmental Panel on Climate Change (IPCC), Ed., IPCC, 2019.

[9]	Department of Climate Change, Energy, the Environment and Water, “National Inventory Report 2022: Volume 1,” Australian Government, 2024. [Online]. Available: https://www.dcceew.gov.au/sites/default/files/documents/national-inventory-report-2022-volume-1.pdf

[10]	Department of Climate Change, Energy, the Environment and Water, “National Inventory Report 2023, Volume I.” Australian Government, 2023. [Online]. Available: https://www.dcceew.gov.au/sites/default/files/documents/national-inventory-report-2023-volume-1.pdf

The term “inorganic fertilisers” in this context refers to both synthetic N-containing fertilisers as well as urea-containing synthetic fertilisers, noting that urea is an organic compound.