# DRAFT Greenhouse Gas Emissions Estimation and Reporting Guidelines for Agriculture, Fisheries and Forestry

Methodological Guidance

Chapters 8-9: Fuel and Refrigerants

© Commonwealth of Australia 2026

Ownership of intellectual property rights

Unless otherwise noted, copyright (and any other intellectual property rights) in this publication is owned by the Commonwealth of Australia (referred to as the Commonwealth).

Creative Commons licence

All material in this publication is licensed under a Creative Commons Attribution 4.0 International Licence except content supplied by third parties, logos and the Commonwealth Coat of Arms.

Inquiries about the licence and any use of this document should be submitted via our online contact form.

Cataloguing data

This publication (and any material sourced from it) should be attributed as: DCCEEW 2026, DRAFT Greenhouse Gas Emissions Estimation and Reporting Guidelines for Agriculture, Fisheries and Forestry - Methodological Guidance, Department of Climate Change, Energy, the Environment and Water, Canberra, March. CC BY 4.0.

This publication is available at dcceew.gov.au.

Department of Climate Change, Energy, the Environment and Water

GPO Box 3090 Canberra ACT 2601

Telephone 1800 920 528

Web dcceew.gov.au

Disclaimer

The Australian Government acting through the Department of Climate Change, Energy, the Environment and Water has exercised due care and skill in preparing and compiling the information and data in this publication. Notwithstanding, the Department of Climate Change, Energy, the Environment and Water, its employees and advisers disclaim all liability, including liability for negligence and for any loss, damage, injury, expense or cost incurred by any person as a result of accessing, using or relying on any of the information or data in this publication to the maximum extent permitted by law.

Acknowledgements

We thank Environmental Accounting Services for drafting this document, and the Voluntary Greenhouse Gas Estimation and Reporting Standards Reference Group for their valued contributions.

Acknowledgement of Country

We acknowledge the Traditional Owners of Country throughout Australia and recognise their continuing connection to land, waters and culture. We pay our respects to their Elders past and present.

Contents

8	Fuel Use	4

8.1	Transport fuel	4

8.1.1	Estimation methodology	5

8.1.1.1	Method 1 – Fuel use transport	5

8.1.1.2	Method 2 – Fuel use transport	5

8.1.2	Data/Parameters	5

8.1.2.1	Input Data (Required)	5

8.1.2.2	Data (Method 1 and 2 Options)	6

8.1.2.3	Constant Parameters	6

8.2	Stationary combustion fuel	7

8.2.1	Estimation methodology	7

8.2.1.1	Method 1 – Fuel use stationary combustion	7

8.2.1.2	Method 2 – Fuel use stationary combustion	8

8.2.2	Data/Parameters	8

8.2.2.1	Input Data (Required)	8

8.2.2.2	Data (Method 1 and 2 Options)	8

8.2.2.3	Constant Parameters	9

9	Refrigerant Use	10

9.1	Estimation methodology	10

9.1.1	Method 1 – Refrigerant use	10

9.1.2	Method 2 – Refrigerant use	10

9.2	Data/Parameters	11

9.2.1	Input Data (Required)	11

9.2.2	Data (Method 1 and 2 Options)	11

9.2.3	Constant Parameters	11

# Fuel Use

## Transport fuel

This module covers the estimation of Scope 1 emissions from fuel consumption for transportation purposes within the entity boundary. Such activities may include:

- fuel used in on-road vehicles owned or leased by the entity, including cars, utes, vans, forklifts or heavy-duty vehicles

- fuel used in off-road vehicles and equipment such as tractors, combines and logging machinery

- fuel used in aircrafts owned or leased by the entity, such as helicopters or aeroplanes, used for operational purposes

- fuel used in vessels owned or leased by the entity, such as fishing vessels and motorboats, used for operational purposes.

Scope 1 fuel use emissions shall only include fuel used in vehicles owned or leased by the entity. Fuel consumed in privately-owned vehicles or contractor-owned vehicles shall be categorised as Scope 3 emissions.

The Scope 1 estimate only covers the emissions generated from the combustion of the fuel itself. Upstream Scope 3 emissions associated with the extraction, transportation and distribution of the fuel are estimated as described in Section 15.9. Emissions of fuel consumption used for stationary combustion purposes (such as operating diesel generators) is covered in Section 8.2.

The following subscripts are used in this module:

| Subscript | Meaning |
| --- | --- |
| q | Fuel type |
| t | Transport type |
| g | Greenhouse gas |

Emissions are estimated based on the fuel type and the type of engine technology used (the transport type). The emissions are summed across each fuel and transport type.

The approach for estimating emissions resulting from the combustion of fuels is based on the approach of the National Greenhouse Accounts Factors (NGAF).

Combustion of fuels results in emissions of three types of greenhouse gases: carbon dioxide CO2, methane CH4, and nitrous oxides N2O. Note that the emissions factors for each greenhouse gas are provided as carbon dioxide equivalents (CO2e).

### Estimation methodology

#### Method 1 – Fuel use transport

- Emissions for each GHG from the combustion of fuels for transport purposes, Eg (t CO2e), is estimated by:

Eg=tq(QTrans,tq×ECTrans,q×EFTrans,1,tqg)×10-3

Where	QTrans,tq = amount of fuel type q combusted in each transport type t (kL or m3)

ECTrans,q = energy content factor of fuel (GJ/kL or GJ/m3)

EFTrans,1,tqg = Scope 1 emission factor for each fuel type and transport type for each greenhouse gas (kg CO2e/GJ)

#### Method 2 – Fuel use transport

There is no Method 2 option for this emission source.

### Data/Parameters

#### Input Data (Required)

| Data / Parameter | Q Trans,q |
| --- | --- |
| Data unit | Varies by fuel type q (kL or m3) Refer to the NGAF for the required data unit. |
| Description | The amount of fuel combusted of fuel type q for transportation purposes. |
| Data source | Invoices, fuel cards, receipts |
| Quality assurance / quality control considerations | Data on fuel consumption should be collected across the entire reporting period. The fuel data should only be for vehicles owned or leased by the agricultural entity and not for any privately-owned vehicles. Where on-site fuel tanks may be used to fill privately-owned vehicles and differentiating fuel use for non-Scope 1 emission activities may be challenging, it may be appropriate and conservative to assume these are de minimis and include them in the Scope 1 emission estimation. An assumption may be made that all fuel purchased in the reporting period was consumed in that reporting period. For example, where a fuel tank was filled up but not fully consumed by the end of the reporting period, QTrans,q should be based on the invoice of filling the entire tank. Where data cannot be disaggregated between fuel used for different transport types or between transport and stationary combustion purposes (e.g., a single diesel fuel tank exists on site that is used to fuel both diesel generators as well as tractors), an estimation may be used to allocate total fuel consumption between the two emission activities. |

#### Data (Method 1 and 2 Options)

Not applicable

#### Constant Parameters

| Data / Parameter | EC Trans,q |
| --- | --- |
| Data unit | Varies by fuel type q (GJ/kL or GJ/m3) |
| Description | Energy content factor of fuel type q |
| Data source | On-road vehicles and aviation – See the National Greenhouse Accounts Factors Off-road vehicles and vessels – See Appendix Table A.4.2.1 |
| Value | Select appropriate value based on transport type |
| Quality assurance / quality control considerations | If Q Trans,q is specified in gigajoule (GJ) then EC Trans,q equals 1. |

| Data / Parameter | EFTrans,1,tqg |
| --- | --- |
| Data unit | kg CO2e /GJ |
| Description | Emission factor for Scope 1 emissions of fuel type q for transport type t each GHG |
| Data source | On-road vehicles and aviation - See the National Greenhouse Accounts Factors Off-road vehicles and vessels – See Appendix Table A.4.2.1 |
| Value | Select appropriate value based on transport type |
| Quality assurance / quality control considerations | N/A |

## Stationary combustion fuel

This module covers the estimation of Scope 1 emissions from fuel consumption for stationary operation purposes within the entity boundary. Such activities may include:

- fuel use in generators to produce electricity

- fuel use in boilers or furnaces for heating

- fuel use in generators driving water pumps or grain augers.

Scope 1 fuel use emissions shall only include fuel used in machinery owned or leased by the entity. Fuel consumed in privately-owned or contractor-owned machinery shall be categorised as Scope 3 emissions.

The Scope 1 estimate only covers the emissions generated from the combustion of the fuel itself. Upstream Scope 3 emissions associated with the extraction, transportation and distribution of the fuel are estimated as described in Section 15.9. Emissions of fuel consumption used for transport purposes (such as tractors or other mobile machinery) is covered in Section 8.1.

The following subscripts are used in this module:

| Subscript | Meaning |
| --- | --- |
| q | Fuel type |
| g | Greenhouse gas |

Emissions are estimated based on the fuel type. The approach for estimating emissions resulting from the combustion of fuels on-farm or at the entity is based on the approach of the National Greenhouse Accounts Factors (NGAF).

Stationary combustion of fuels results in emissions of three types of GHG: carbon dioxide CO2, methane CH4, and nitrous oxides N2O. Note that the emissions factors for each greenhouse gas are provided as carbon dioxide equivalents (CO2e).

### Estimation methodology

#### Method 1 – Fuel use stationary combustion

- Emissions for each GHG from the stationary combustion of fuels, Eg (t CO2e), is estimated as:

Eg=q(QSC,q×ECSC,q×EFSC,1,qg) ×10-3

Where	QSC,q = amount of fuel type q used for stationary combustion operations (kL, m3 or t)

ECSC,q = energy content factor for stationary combustion operations (GJ/kL, m3 or t)

EFSC,1,qg = Scope 1 emission factor for stationary combustion for each greenhouse gas g (kg CO2e /GJ)

#### Method 2 – Fuel use stationary combustion

There is no Method 2 option for this emission source.

### Data/Parameters

#### Input Data (Required)

| Data / Parameter | QSC,q |
| --- | --- |
| Data unit | Varies by fuel type q (kL, m3 or tonnes) Refer to the National Greenhouse Accounts Factors to determine the necessary data unit specific to each fuel type. |
| Description | The amount of fuel combusted of fuel type q in stationary combustion in operations. |
| Data source | Invoices, fuel cards, receipts |
| Quality assurance / quality control considerations | Data on fuel consumption should be collected across the entire reporting period. An assumption may be made that all fuel purchased in the reporting period was consumed in that reporting period. For example, where a fuel tank was filled up but not fully consumed by the end of the reporting period, QSC,q should be based on the invoice of filling the entire tank. Where data cannot be disaggregated between fuel used for stationary combustion and transport purposes (e.g., a single diesel fuel tank exists on site that is used to fuel both diesel generators as well as tractors), an estimation may be used to allocate total fuel consumption between the two emission sources. |

#### Data (Method 1 and 2 Options)

Not applicable

#### Constant Parameters

| Data / Parameter | ECSC,q |
| --- | --- |
| Data unit | Varies by fuel type q (GJ/kL, GJ/m3 or GJ/t) |
| Description | Energy content factor of fuel type q for stationary combustion operations |
| Data source | Refer to Australian National Greenhouse Accounts Factors Table 4, 5 and 8 |
| Value | Select appropriate value based on fuel type |
| Quality assurance / quality control considerations | If QSC,q is specified in gigajoule (GJ) then ECSC,q equals 1. |

| Data / Parameter | EFSC,1,qg |
| --- | --- |
| Data unit | kg CO2e /GJ |
| Description | Emission factor for Scope 1 emissions of fuel type q for stationary combustion operations for each greenhouse gas g |
| Data source | Refer to Australian National Greenhouse Accounts Factors Table 4, 5 and 8 |
| Value | Select appropriate value based on fuel type |
| Quality assurance / quality control considerations | N/A |

# Refrigerant Use

This module covers the estimation of emissions from refrigerant use within the entity boundary. Scope 1 emissions result from refrigerants leaking out of refrigeration and air conditioning equipment from operation. Leakages during installation or disposal of equipment are not covered.

The following subscripts are used in this module:

| Subscript | Meaning |
| --- | --- |
| R | Refrigerant type |
| a | Appliance |

Emissions are estimated based on the appliance type reflecting different refrigerant charges and leakage rates and the different refrigerant type. The emissions are summed across each appliance type.

The approach for estimating Scope 1 emissions from refrigeration leaking is based on the approach of the National Greenhouse Accounts Factors (NGAF).

## Estimation methodology

### Method 1 – Refrigerant use

- The emissions of refrigerant leaking, ER (tonnes of gas R), is estimated for each refrigerant gas R as:

ER=a(chargeRa ×leakage ratea100)×10-3

Where		chargeRa = amount of refrigerant R contained in appliance a (kg)

leakage ratea = percentage of total charge leaked each year (%)

### Method 2 – Refrigerant use

There is no Method 2 option for this emission source.

## Data/Parameters

### Input Data (Required)

| Data / Parameter | chargeRa |
| --- | --- |
| Data unit | kg |
| Description | The amount of refrigerant gas R contained in appliance a |
| Data source | Manufacturer’s product specification should contain information of the charge of the refrigerant contained in each appliance. |
| Quality assurance / quality control considerations | N/A |

### Data (Method 1 and 2 Options)

Not applicable

### Constant Parameters

| Data / Parameter | Leakage ratea |
| --- | --- |
| Data unit | Per cent |
| Description | The percentage of total charge leaked from appliance each year. |
| Data source | Refer to Australian National Greenhouse Accounts Factors Table 10 |
| Value | Select appropriate value based on product type |
| Quality assurance / quality control considerations | N/A |