DRAFT Greenhouse Gas Emissions Estimation and Reporting Guidelines for Agriculture, Fisheries and Forestry

Methodological Guidance

Chapters 10-13: Management of Waste,

Waste Water Treatment,

Purchased CO2 and N2O Emissions

In Aquaculture

© Commonwealth of Australia 2026

Ownership of intellectual property rights

Unless otherwise noted, copyright (and any other intellectual property rights) in this publication is owned by the Commonwealth of Australia (referred to as the Commonwealth).

Creative Commons licence

All material in this publication is licensed under a Creative Commons Attribution 4.0 International Licence except content supplied by third parties, logos and the Commonwealth Coat of Arms.

Inquiries about the licence and any use of this document should be submitted via our online contact form.

Cataloguing data

This publication (and any material sourced from it) should be attributed as: DCCEEW 2026, DRAFT Greenhouse Gas Emissions Estimation and Reporting Guidelines for Agriculture, Fisheries and Forestry - Methodological Guidance, Department of Climate Change, Energy, the Environment and Water, Canberra, March. CC BY 4.0.

This publication is available at dcceew.gov.au.

Department of Climate Change, Energy, the Environment and Water

GPO Box 3090 Canberra ACT 2601

Telephone 1800 920 528

Web dcceew.gov.au

Disclaimer

The Australian Government acting through the Department of Climate Change, Energy, the Environment and Water has exercised due care and skill in preparing and compiling the information and data in this publication. Notwithstanding, the Department of Climate Change, Energy, the Environment and Water, its employees and advisers disclaim all liability, including liability for negligence and for any loss, damage, injury, expense or cost incurred by any person as a result of accessing, using or relying on any of the information or data in this publication to the maximum extent permitted by law.

Acknowledgements

We thank Environmental Accounting Services for drafting this document, and the Voluntary Greenhouse Gas Estimation and Reporting Standards Reference Group for their valued contributions.

Acknowledgement of Country

We acknowledge the Traditional Owners of Country throughout Australia and recognise their continuing connection to land, waters and culture. We pay our respects to their Elders past and present.

Contents

10	Scope 1 – Solid Waste Treatment	4

10.1	Estimation methodology	5

10.1.1	Method 1 – Solid Waste Treatment	5

10.1.2	Method 2 – Solid Waste Treatment	6

10.2	Data/Parameters	7

10.2.1	Input Data (Required)	7

10.2.2	Data (Method 1 and 2 Options)	7

10.2.3	Constant Parameters	7

11	Scope 1 – Wastewater Treatment	10

11.1	Estimation methodology	10

11.1.1	Method 1 – Methane emissions from onsite Wastewater Management	10

11.1.2	Method 2 – Methane emissions from onsite Wastewater Management	11

11.1.3	Method 1 – Nitrous oxide emissions from onsite Wastewater Management	11

11.1.4	Method 2 – Nitrous oxide emissions from onsite Wastewater Management	12

11.2	Data/Parameters	12

11.2.1	Input Data (Required)	12

11.2.2	Data (Method 1 and 2 Options)	14

11.2.3	Constants	14

12	Scope 1 – Supplementary CO2	17

12.1	Estimation methodology	17

12.1.1	Method 1 – Supplementary CO2	17

12.1.2	Method 2 – Supplementary CO2	17

12.2	Data/Parameters	17

12.2.1	Input Data (Required)	17

12.2.2	Data (Method 1 and 2 Options)	17

12.2.3	Constants	17

13	N2O Emissions from Aquaculture	18

13.1	Estimation Methodology	18

13.1.1	Method 1 – Aquatic N2O	18

13.1.2	Method 2 – Aquatic N2O	18

13.2	Data/Parameters	18

13.2.1	Input Data (Required)	18

13.2.2	Input Data (Method 1 and 2 Options)	18

13.2.3	Constant Parameters	19

References	20

# Scope 1 – Solid Waste Treatment

This module covers the estimation of greenhouse gas emissions that result from the treatment of solid waste generated by the entity and disposed of within its organisational boundary.

Where waste is disposed of outside the organisational boundary (e.g., at a facility not owned or operated by the reporting entity), the associated emissions shall be classified as Scope 3 emissions. The same emission estimation methodologies as outlined here may be used to quantify Scope 3 emissions associated with solid waste treatment.

This module does not account for emissions associated with the transport of the solid waste to its treatment/disposal location.

Emissions are estimated based on the composition of the waste and the treatment method.

The following subscripts are used in this module:

| Subscript | Meaning |
| --- | --- |
| w | Waste type |
| t | Waste treatment type |
| g | Greenhouse gas |

The following solid waste treatments are included in this module:

- landfill

- incineration

- open burning

- composting

- anaerobic digestion.

Incineration refers to the combustion of solid waste in controlled incineration facilities that can ensure complete combustion of waste materials by maintaining high combustion temperatures and long residence times [1].

Emissions from field burning of crop residues should be estimated following the approach in Chapter 6.4. Emissions from biomass burning in forestry and land use (e.g., slash burning) should be estimated following the approach in Chapter 16, Section 16.1 in the LULUCF guidance. Emissions from the use of wood waste for stationary combustion purposes (e.g., at wood processing mills) should be estimated following the approach in Chapter 8.2.

The GHG emissions from solid waste disposal depend on the type of treatment. In alignment with the methodologies of the National Greenhouse and Energy Reporting (Measurement) Determination 2008, the emissions of the following gases are estimated from each treatment type:

- landfill: methane only

- incineration: carbon dioxide only

- composting: methane and nitrous oxides

- anaerobic digestion: methane and nitrous oxides.

Note that the emissions factors for each greenhouse gas are provided as carbon dioxide equivalents (CO2-e).

Methodologies for estimating emissions from open burning of waste are based on IPCC and include emissions from carbon dioxide, methane and nitrous oxides [1].

In alignment with the estimation methodologies of the National Greenhouse Accounts Factors, the methods outlined below do not account for methane gas capture.

Questions Reference 10.1

Do these modes of waste treatment appropriately capture how dead animals are dealt with on-farm? Are there are management methods or factors that may impact the emissions from dead animals that need to be incorporated into the emission estimation? For estimation of the management of poultry deaths through manure management systems, see consultation questions in Chapter 4

## Estimation methodology

### Method 1 – Solid Waste Treatment

(1) 	Methane emissions from waste management Eg (t CO2e) are calculated as the sum of emissions from all types of waste w treated across different facilities t for each greenhouse gas g as:
Eg=twQwt×EFwtg

Where	Qwt = quantity of solid waste of type w being sent to treatment facility t (t), on a wet weight basis

EFwtg = emission factor for greenhouse gas g for the management of waste at the treatment facility (t CO2e/t)

(2) 	In equation (1) if the mass of the waste Qwt is unknown, it may be calculated based on the volume of waste as:

Qwt=Vwt×VTMw

Where		Vwt = volume of solid waste type w being sent to treatment facility t (m3)

VTMw = conversion factor of volume to mass for waste type w (t/m3)

### Method 2 – Solid Waste Treatment

There is no Method 2 option for this emission source.

Questions Reference 10.2

Would it be preferential to provide a Method 1 and Method 2 option for solid waste treatment, where Method 1 only includes a single default EF, to encourage consistency across enterprises? Which waste category would most accurately reflect the typical waste that is sent off-farm from agricultural enterprises? Are there any studies on typical waste composition from farms, that could be used to create a more appropriate EF?

## Data/Parameters

### Input Data (Required)

| Data / Parameter | Qwt |
| --- | --- |
| Data unit | tonnes |
| Description | Quantity of waste of type w sent for disposal to waste treatment t |
| Data source | Invoices for skip disposal, weighbridge receipts, farm records (e.g., number and weight of deceased livestock) Data should be in tonnes of wet weight, including the natural moisture content of the waste. |
| Quality assurance / quality control considerations | It is preferential to use waste data by mass rather than by volume to attain a more accurate emission estimation. Where Qwt is not known, it may be estimated based on volume Vwt. |

| Data / Parameter | Vwt |
| --- | --- |
| Data unit | m3 |
| Description | Volume of waste type w being sent to treatment facility t |
| Data source | Invoices from skip disposal, or other farm records (only required if Qwt is not known) |
| Quality assurance / quality control considerations | It is preferential to use waste data by mass (Qwt) rather than by volume to attain a more accurate emission estimation. |

### Data (Method 1 and 2 Options)

N/A

### Constant Parameters

Refer to the data and parameter tables in Chapters 5.2.2.3, 5.4.2.3, and 5.5.2.3 for parameters used in the estimation of EN2O, Ead, and Eleach respectively, for the estimation of emissions from manure sent off-site.

| Data / Parameter | EFwtg |
| --- | --- |
| Data unit | t CO2e/t |
| Description | Emission factor for each greenhouse gas g for waste type w disposed to treatment type t |
| Data source | See Appendix Table A.4.1.1 |
| Value | Select the EF for the specific waste type. If the composition of waste is not known or a waste specific EF is not available, select the EF from the following average waste mix types that best reflects the composition of the waste stream: - Municipal solid waste - ~26% food - 26% paper and cardboards - ~26% inert (e.g concrete, metal, plastic, glass) - ~10% green waste - ~12% other (e.g wood, textiles, nappies) - commercial and industrial waste - ~55% paper and cardboards - ~16% inert - ~14 wood and woodwaste - ~15% other (e.g. sludge, food, green waste, textiles) - construction and demolition waste - ~89% inert - ~11% other (e.g. wood, green waste, paper and cardboard) |
| Quality assurance / quality control considerations | The method does not take into account any landfill gas capture occurring at the landfill. Methane emissions from incineration have not been included in the emission estimation, following the approach of the National Greenhouse and Energy Reporting (Measurement) Determination 2008 (EFwt=incineration,g=CH4 = 0). The emission factors reflect the amount of fossil carbon in each waste stream, to ensure that only anthropogenic carbon dioxide emissions from incineration are accounted for. For landfill and biological treatments of organic waste, the carbon contained in the organic waste is of biogenic origin and therefore emissions of carbon dioxide do not need to be included in the estimation (EFwt=landfill, composting, anaerobic digestion,g=CO2 = 0). Nitrous oxide emissions from incineration have not been included in the emission estimation, following the approach of the National Greenhouse and Energy Reporting (Measurement) Determination 2008 (EFwt=incineration,g=N2O = 0). Nitrous oxide emissions from landfill are de minimis (EFwt=landfill,g=N2O = 0) [3]. |

| Data / Parameter | VTMw |
| --- | --- |
| Data unit | t/m3 |
| Description | Volume to mass conversion factor for waste type w |
| Data source | See Appendix Table A.4.1.2 |
| Value | Select appropriate value based on waste type, |
| Quality assurance / quality control considerations | It is preferential to use waste data by mass (Qwt) rather than by volume to attain a more accurate emission estimation. |

# Scope 1 – Wastewater Treatment

This module covers the estimation of greenhouse gas emissions that result from the treatment of wastewater within the organisational boundary. The approach assumes nitrous oxides emissions from the wastewater and sludge treatment are negligible.

Treatment of wastewater refers to systems in which wastewater is collected and treated, either aerobically or anaerobically, prior to discharge. Emissions from untreated or uncollected systems are not included in this module (for example, wastewater from flushing animal sheds or housing).

Where entities conduct wastewater treatment on-site prior to discharge to sewers or the environment, the methodologies described here shall be implemented to quantify Scope 1 emissions from wastewater treatment.

Where wastewater is treated outside the organisational boundary (e.g., at a municipal wastewater facility, not owned or operated by the reporting entity), the associated emissions shall be classified as Scope 3 emissions. Entities may choose to exclude this Scope 3 emission source from the inventory boundary due to challenges associated with acquiring the necessary activity data. It is recommended that immateriality of this Scope 3 emission source is justified.

Emissions associated with fuel or electricity consumption at the treatment facility, e.g., for the running of pumps, is not included in module and should be estimated using the methods provided under Scope 1 stationary combustion fuel (Chapter 8.2) or Scope 2 purchased electricity (Chapter 14).

The following subscripts are used in this module:

| Subscript | Meaning |
| --- | --- |
| t | Wastewater treatment facility type |

## Estimation methodology

Question Reference 11.1.

The current emission estimation method is based on the approach of the National Greenhouse and Energy Reporting Measurement Determination 2008. Which parameters should be made default values in the Guidelines (where it would be otherwise unrealistic to expect entities with on-site wastewater treatment to know these values (e.g. Fsludge))?

### Method 1 – Methane emissions from onsite Wastewater Management

- Methane emissions from wastewater treatment, ECH4 (t CH4), are calculated following the approach of the National Greenhouse and Energy Reporting Measurement Determination 2008 as the sum of methane released from the wastewater, methane released from the sludge, and methane released from any flared biogas as:

ECH4=11000×Wt×CODin×1-Fsludge-CODout×CFww,t×EFww+Wt×CODin×(Fsludge-Fremoved)×CFsludge,t×EFsludge-6.784×10-4×Qcap+Qflared+Qout+Qflared×ECflared×EFflared,CH4

Where	Wt = quantity of wastewater treated at facility type t (m3)

CODin = average inlet Chemical Oxygen Demand (COD) concentration of wastewater (kg COD/m3)

Fsludge = fraction of COD removed from wastewater as sludge ()

CODout = average outlet COD concentration of wastewater (kg COD/m3)

CFww,t = methane correction factor for facility type t used to treat wastewater ()

EFww = methane emission factor for wastewater (kg CO2e /kg COD)

Fremoved = fraction of COD removed from the wastewater treatment facility and transferred to landfill, biochar production, or other site ()

CFsludge,t = methane correction factor for facility type t used to treat sludge ()

EFsludge = methane emission factor for sludge (kg CO2e/kg COD)

Qcap = quantity of methane in sludge biogas captured for combustion for the operation of the treatment facility (m3 CH4)

Qflared = quantity of methane in sludge biogas flared by the treatment facility (m3 CH4)

Qout= quantity of methane in sludge biogas transferred out of the treatment facility (m3 CH4)

ECflared = energy content factor of methane in sludge biogas flared (GJ/m3)

EFflared,CH4 = methane emission factor for sludge biogas flared (kg CO2e/GJ)

### Method 2 – Methane emissions from onsite Wastewater Management

There is no Method 2 option for this emission source.

### Method 1 – Nitrous oxide emissions from onsite Wastewater Management

In alignment with the National Greenhouse and Energy Reporting Measurement Determination 2008, nitrous oxide emissions from the treatment of the wastewater and sludge are assumed de minimis. Only nitrous oxide emissions from the flared biogas are included in this emission estimation.

- Nitrous oxide emissions released from any flared biogas, EN2O (t N2O), are estimated as:

EN2O=Qflared1000×ECflared×EFflared,N2O

Where	EFflared,N2O = nitrous oxide emission factor for sludge biogas flared (kg CO2e/GJ)

### Method 2 – Nitrous oxide emissions from onsite Wastewater Management

There is no Method 2 option for this emission source.

## Data/Parameters

### Input Data (Required)

| Data / Parameter | Wt |
| --- | --- |
| Data unit | m3 |
| Description | Quantity of wastewater treated at facility type t |
| Data source | Direct measurement of daily or hourly flows scaled to give an annual average |
| Quality assurance / quality control considerations | Comparison to typical wastewater generation rates for the corresponding industry as listed in the National Greenhouse and Energy Reporting Measurement Determination 2008, Part 5.42(8) [4] may be made, although these are commodity-specific and may not be accurate. |

| Data / Parameter | CODin |
| --- | --- |
| Data unit | kg COD/m3 |
| Description | Average inlet chemical oxygen demand (COD) concentration of waste water |
| Data source | Direct measurement |
| Quality assurance / quality control considerations | Comparison to typical COD concentration values for the corresponding industry as listed in the National Greenhouse and Energy Reporting Measurement Determination 2008, Part 5.42(8) [4] should be made. |

| Data / Parameter | CODout |
| --- | --- |
| Data unit | kg COD/m3 |
| Description | average outlet COD concentration of wastewater |
| Data source | Direct measurement |
| Quality assurance / quality control considerations | Value may not exceed CODin. Comparison to typical COD concentration values for the corresponding industry as listed in the National Greenhouse and Energy Reporting Measurement Determination 2008, Part 5.42(8) [4] should be made. |

| Data / Parameter | Fsludge |
| --- | --- |
| Data unit | fraction |
| Description | Fraction of COD removed from the wastewater treatment facility as sludge |
| Data source | Measurement records, averaged over the reporting period |
| Quality assurance / quality control considerations | Fsludge shall be averaged across the entire reporting period. A mass balance may be conducted to ensure that all COD in, out, and treated within the system is accounted for appropriately. |

| Data / Parameter | Fremoved |
| --- | --- |
| Data unit | fraction |
| Description | Fraction of COD removed from the wastewater treatment facility and transferred to landfill, biochar production, or other site |
| Data source | Measurement records, averaged over the reporting period |
| Quality assurance / quality control considerations | Fremoved shall be averaged across the entire reporting period. A mass balance may be conducted to ensure that all COD in, out, and treated within the system is accounted for appropriately. |

| Data / Parameter | Qcap |
| --- | --- |
| Data unit | m3 CH4 |
| Description | Quantity of methane in sludge biogas captured for combustion for the operation of the treatment facility |
| Data source | Direct measurement of methane content of sludge biogas, multiplied by total volume of biogas combusted by the facility. Measurement of methane should be conducted in alignment with Division 2.3.6 of the National Greenhouse and Energy Reporting Measurement Determination 2008. |
| Quality assurance / quality control considerations | If methane content of sludge biogas is unknown, it shall be assumed to be 70%. |

| Data / Parameter | Qflared |
| --- | --- |
| Data unit | m3 CH4 |
| Description | Quantity of methane in sludge biogas flared by the treatment facility |
| Data source | Direct measurement of methane content of sludge biogas, multiplied by total volume of biogas flared. Measurement of methane should be conducted in alignment with Division 2.3.6 of the National Greenhouse and Energy Reporting Measurement Determination 2008. |
| Quality assurance / quality control considerations | If methane content of sludge biogas is unknown, it shall be assumed to be 70%. |

| Data / Parameter | Qout |
| --- | --- |
| Data unit | m3 CH4 |
| Description | Quantity of methane in sludge biogas transferred out of the treatment facility |
| Data source | Direct measurement of methane content of sludge biogas, multiplied by total volume of biogas leaving the treatment facility. Measurement of methane should be conducted in alignment with Division 2.3.6 of the National Greenhouse and Energy Reporting Measurement Determination 2008. |
| Quality assurance / quality control considerations | If methane content of sludge biogas is unknown, it shall be assumed to be 70%. |

### Data (Method 1 and 2 Options)

N/A

### Constants

| Data / Parameter | CFww,t |
| --- | --- |
| Data unit | fraction |
| Description | Methane correction factor for treatment facility type t used to treat wastewater |
| Data source | See Appendix Table A.4.1.3 |
| Value | Select appropriate value based on facility type. |
| Quality assurance / quality control considerations |  |

| Data / Parameter | CFsludge,t |
| --- | --- |
| Data unit | fraction |
| Description | Methane correction factor for treatment facility type t used to treat sludge |
| Data source | See Appendix Table A.4.1.4 |
| Value | Select appropriate value based on facility type. |
| Quality assurance / quality control considerations |  |

| Data / Parameter | EFww |
| --- | --- |
| Data unit | kg CH4 /kg COD |
| Description | Methane emission factor for wastewater |
| Data source | National Greenhouse and Energy Reporting Measurement Determination 2008, Part 5.42(6) [4] |
| Value | 0.25 |
| Quality assurance / quality control considerations |  |

| Data / Parameter | EFsludge |
| --- | --- |
| Data unit | kg CH4/kg COD |
| Description | Methane emission factor for sludge |
| Data source | National Greenhouse and Energy Reporting Measurement Determination 2008, Part 5.42(7) [4] |
| Value | 0.25 |
| Quality assurance / quality control considerations |  |

| Data / Parameter | ECflared |
| --- | --- |
| Data unit | GJ/m3 |
| Description | Energy content factor of sludge biogas flared |
| Data source | National Greenhouse and Energy Reporting Measurement Determination 2008, Schedule 1 [4] |
| Value | 0.0377 |
| Quality assurance / quality control considerations | N/A |

| Data / Parameter | EFflared,CH4 |
| --- | --- |
| Data unit | kg CH4/GJ |
| Description | Methane emission factor for sludge biogas flared |
| Data source | National Greenhouse and Energy Reporting Measurement Determination 2008, Schedule 1 [4] |
| Value | 0.2289 |
| Quality assurance / quality control considerations | N/A |

| Data / Parameter | EFflared,N2O |
| --- | --- |
| Data unit | kg N2O/GJ |
| Description | Nitrous oxide emission factor for sludge biogas flared |
| Data source | National Greenhouse and Energy Reporting Measurement Determination 2008, Schedule 1 [4] |
| Value | 1.132 x 10-4 |
| Quality assurance / quality control considerations | N/A |

# Scope 1 – Supplementary CO2

This module covers the estimation of greenhouse gas emissions that result from the use of supplementary CO2 within the operational boundary, for example the use of dry ice or CO2 canisters in winemaking.

This emission methodology assumes that CO2 used in the operations or processing are released into the atmosphere. It does not discount emissions for supplementary CO2 that may be stored within the final product, such as in sparkling wine, that is released in product end-use.

Embedded emissions from purchasing the supplementary CO2 should be categorised as Scope 3.

## Estimation methodology

### Method 1 – Supplementary CO2

- Emissions from the use of supplementary CO2, E (t CO2e), are calculated as:

E=Q×10-3

Where	Q  = quantity of supplementary CO2 used within the operations (kg CO2)

### Method 2 – Supplementary CO2

There is no Method 2 option for this emission source.

## Data/Parameters

### Input Data (Required)

| Data / Parameter | Q |
| --- | --- |
| Data unit | kg CO2 |
| Description | Quantity of supplementary CO2 used |
| Data source | Purchase records |
| Quality assurance / quality control considerations | Where it is reasonable to assume that all supplementary CO2 purchased within the reporting year is applied in that year, Q should be based on concrete records or invoices of CO2 purchased within the reporting period. Where supplementary CO2 is not pure carbon dioxide, Q may need to be calculated based on the CO2 concentration. |

### Data (Method 1 and 2 Options)

N/A

### Constants

N/A

# N2O Emissions from Aquaculture

This module covers the estimation of greenhouse gas emissions that result from aquaculture ponds. The N2O emissions (called “aquatic N2O”) occur as a result of increased nutrient loads in the culture ponds, where N2O is emitted as a by-product of the conversion of ammonia contained in fish urea from the high-protein feeds in aquaculture systems, to nitrate through nitrification and nitrate to N2 gas through denitrification [5].

These emissions only need to be accounted for aquaculture systems both on and offshore, but not for wildcatch fisheries.

## Estimation Methodology

### Method 1 – Aquatic N2O

- Nitrous oxide emissions from aquaculture, E (t N2O) are estimated as:

E=Q×EF

Where	Q = quantity of fish / crustaceans produced (t)

EF = emission factor for N2O emissions from fish / crustacean production (t N2O-N / t)

### Method 2 – Aquatic N2O

There is no Method 2 option for this emission source.

## Data/Parameters

### Input Data (Required)

| Data / Parameter | Q |
| --- | --- |
| Data unit | tonnes |
| Description | Quantity of fish / crustaceans produced in aquaculture ponds in the reporting period |
| Data source | Production records, sales records |
| Quality assurance / quality control considerations | Q shall be summed across all species of fish and crustaceans produced in culture ponds in coastal wetlands. |

### Input Data (Method 1 and 2 Options)

N/A

### Constant Parameters

Question Reference 13.1

Current guidance matches the approach taken in the National Greenhouse Accounts. However, the possibility of emission factors for various aquatic systems is still being investigated. Are there any other industry guidance that could be referenced to support estimation of aquatic N2O emissions from different aquatic production systems, e.g., offshore caged farming, offshore oyster/mussel farming, onshore fish ponds, onshore prawn ponds, etc.?

| Data / Parameter | EF |
| --- | --- |
| Data unit | t N2O-N / t |
| Description | Emission factor for aquatic N2O emissions from fish production |
| Data source | Hu et. al. [6] |
| Value | 0.00169 |
| Quality assurance / quality control considerations | Ensure the most recently available published data is used in alignment with the Australian National Inventory Report. |

# References

[1]	G.H. Sabin Guendehou, Matthias Koch, Leif Hockstad, Riitta Pipatti, and Masato Yamada, “Chapter 5 - Incineration and Open Burning of Waste,” in 2006 IPCC Guidelines for National Greenhouse Gas Inventories Volume 5: Waste, IPCC, 2006. Accessed: Dec. 17, 2025. [Online]. Available: https://www.ipcc-nggip.iges.or.jp/public/2006gl/pdf/5_Volume5/V5_5_Ch5_IOB.pdf

[2]	Department of Climate Change, Energy, the Environment and Water, “Australian National Greenhouse Accounts Factors,” 2024.

[3]	Riitta Pipatti et al., “Chapter 3: Solid Waste Disposal,” in 2006 IPCC Guidelines for National Greenhouse Gas Inventories Volume 5: Waste, IPCC, 2006. Accessed: Dec. 17, 2025. [Online]. Available: https://www.ipcc-nggip.iges.or.jp/public/2006gl/pdf/5_Volume5/V5_3_Ch3_SWDS.pdf

[4]	National Greenhouse and Energy Reporting (Measurement) Determination 2008. 2025. [Online]. Available: https://www.legislation.gov.au/F2008L02309/latest/text

[5]	Hiraishi, T., Krug, T., Tanabe, K., Srivastava, N., Baasansuren, J., Fukuda, M. and Troxler, T.G., “2013 Supplement to the 2006 IPCC Guidelines for National Greenhouse Gas Inventories: Wetlands.” IPCC, 2014. [Online]. Available: https://www.ipcc.ch/site/assets/uploads/2018/03/Wetlands_Supplement_Entire_Report.pdf

[6]	Z. Hu, J. Lee, K. Chandran, S. Kim, and S. Khanal, “Nitrous Oxide (N2O) Emission from Aquaculture: A Review,” Environmental science & technology, vol. 46, pp. 6470–80, May 2012, doi: 10.1021/es300110x.